3GPP TSG-RAN WG2 #102	R2-1807195

# Busan, Republic of Korea, 21st – 25th May 2018			  Revision of R2-1805433

Agenda Item:	10.3.1.11

Source:	Ericsson

Title:	PDCP duplication impact to MAC

Document for:	Discussion, Decision

# Introduction

In RAN2#101bis, the impact of PDCP duplication to MAC was discussed. In this contribution we discuss the remaining issues regarding LCP and BSR.

# Discussion

## Impact to LCP

In RAN2#101 and #101bis, the impact of PDCP duplication to MAC, and in particular the deactivation of the duplication i.e. the secondary logical channel to LCP had been discussed. One in #101 an FFS was noted on the Bj calculation of the secondary logical channel. Bj is initially set to zero and updated by the product of prioritized bitrate and time since the last update. It was discussed that upon reactivation of the secondary logical channel (i.e. reactivation of duplication), the Bj associated with this logical channel would undergo an update/increase by the deactivation time, since it had not been updated during that time. It was motivated in [1] that Bj should be set to zero upon duplication reactivation, in order to mitigate an imbalance between Bj of the primary and secondary logical channel for the next duplicate transmission. Given, however, that the two logical channels are not contending for UL resources, since in duplication cell/carrier transmission restrictions are applied, we do not see that this imbalance causes any issue. Furthermore, we assume that for a duplication bearer, also no issues are caused by this effect among logical channels restricted to the same cell/carrier, since a likely configuration in case of multiple bearers would be absolute priorities in order to guarantee deterministic behaviour (latency) for the duplication bearers. Therefore, we propose the following:

No optimization of LCP Bj calculation is applied. Bj is not reset when duplication is reactivated.

## BSR triggering

It had been agreed in RAN2#101 that as a baseline no new BSR triggers are introduced to support packet duplication. An FFS was noted if this applies also to duplication in CA. We did not see a difference in the BSR triggering between DC and CA, i.e. also in CA, BSR calculation will be done per LCG, and associating primary and secondary logical channel with separate LCGs will result in independent BSR triggering among those channels in duplication. Furthermore, we do not see an issue in potential redundant BSR reporting for the secondary logical channel, in case it is deactivated. Deactivation will not happen often enough to justify an optimization of not reporting BSR in this case. To keep the impact of PDCP duplication to MAC as simple as possible, we propose:

No optimization for BSR triggering is applied. Confirm that no new BSR triggers are introduced to support duplication neither for DC nor for CA.

# Conclusion

Based on the discussion in section 2 we propose the following:

Proposal 1	No optimization of LCP Bj calculation is applied. Bj is not reset when duplication is reactivated.

Proposal 2	No optimization for BSR triggering is applied. Confirm that no new BSR triggers are introduced to support duplication neither for DC nor for CA.

# References

- R2-1803117, Impact of PDCP duplication on LCP,	Qualcomm Incorporated, 3GPP TSG-RAN WG2 Meeting #101, Athens, Greece, 26th February – 2nd March 2018