3GPP TSG-RAN WG2 #102	R2-1807198

# Busan, Republic of Korea, 21st – 25th May 2018

Agenda Item:	10.3.4.3

Source:	Ericsson

Title:	Remapping of multiple QoS Flows and end marker

Document for:	Discussion, Decision

# Introduction

In discussing solutions for a UE based end-marker solution to support QoS flow relocation, RAN2 have made the following agreements:

RAN2#101:

Agreements:

=>	For DL it is left up to gNB implementation.  FFS if RAN3 signalling is required.

=>	FFS - We define an end/start marker on UE side and how it is used it is up to gNB implementation.   At least for RLC AM the start/end marker solution is used as a baseline.

RAN2#101bis:

An uplink end marker is introduced in the SDAP layer, for QoS flow relocation.

FFS whether we go for A or B.

In the above, option A and B refers to the options: A) end marker in the header and not transmitted in this case, assume the case is handled by a timeout in the receiver, and: B) the end marker can be sent stand-alone (a control PDU, header-only field etc)

Note that in  time of writing, email discussion ‘ [101bis#76][NR UP]  SDAP end marker solutions ‘, was not completed, however details to solution options w.r.t handling multiple simultaneous QoS Flow relocations was not discussed.

Based on the above, this contribution discusses stage 3 details of UL end marker solution for QoS Flow remapping.

# General

When a QoS flow is remapped, the remapping is done from a source DRB to a target DRB. In this case the SDAP entity is responsible of making the path switch, which can be made based on RDI indication or RRC signalling. After the path switch, packets which have already been transmitted to source PDCP entity will be transmitted via source path and new SDAP SDUs will be transmitted via new path, i.e. target DRB. Figure 1 illustrates this scenario.

Figure 1: Flow Relocation with PDCP receiver buffering and UE end marker

Upon receiving indication either via RRC signalling or via RDI bit on SDAP header, the SDAP entity shall re-route all new traffic to target DRB.

## Relocation of multiple QoS Flows

As already described above, the new QoS framework allows mapping of multiple QoS flows to DRBs simultaneously. Additionally, the new QoS Framework supports mapping of large amount of QoS Flow to DRBs (up to 64 with 6-bit QFI). This results in a situation where multiple parallel QoS Flow relocations are on-going can be expected. As the PDCP is not aware of QoS Flows, in order to block traffic from one QoS Flow, PDCP needs to block (buffer) traffic of all QoS Flows associated with the DRB.

As the PDCP is not aware of QoS Flows, it needs to block all traffic on DRB meanwhile QFI is being relocated.

The figure below illustrates a problem on relocation of multiple QoS flows simultaneously. For example, when considering a case where two QoS Flows, Flow 1 and Flow 2, are relocated to a new, DRB 1. Flow 1 is associated with an End marker EM1 and QoS Flow 2 is associated with the end marker EM2. Because PDCP/RLC cannot be aware of QoS Flows, there is a need to block all outgoing traffic from DRB 1 until the relocation is completed. In the case where only one flow is relocated at the time, the PDCP could release the traffic from DRB 1 upon receiving any end marker. When multiple QFIs are activated simultaneously, the delay the flow relocation procedures incur depend on the completion of all relocated flows. This would be problematic especially when multiple or all QFIs are mapped to a default DRB when traffic is established. Delaying each activation of RAN QFI remapping procedure by picking one QFI at the time procedure may result in an undefined behaviour as the 5GC and end application assumes that the QFI has already been established. Additionally, long delays on critical QFIs may result that PDB is exceeded. Note that it does not matter for the end user whether packets reach the destination too late or are dropped.

Figure 2: Two QoS Flows are relocated simultaneously, both are sending end markers to target PDCP entity

With the new QoS framework and Default DRB concept, it does not seem practical to relocate one QFI at the time.

Confirm that multiple QoS Flows relocated simultaneously is supported

In order to identify which QoS Flow the end marker is associated to, the SDAP entities needs to identify for the QoS Flow. It is is suggested to use the QFI for this purpose using either header field, or a SDAP Control PDU ([101bis#76][NR UP]  SDAP end marker solutions). A header only can in this respect be seen as equivalent to a SDAP Control PDU.

The buffering is based of remapping through RRC reconfiguration or AS reflective QoS. I.e. QFI information from the the SDAP entity is used in the receiver for start of buffering as well as releasing the buffer when the all end marker associated with a relocated DRB is received. As the RRC re-configuration allows the relocation of multiple flows in a single procedure, one could optimize the end-marker solution to include more than one QFI in the SDAP control PDU. This is not possible for reflective QoS as a single QFI detected on a corresponding DL DRB would initiate the remapping per QoS flow. As a result, it would allow for a simpler design if a single end-marker format can be used, i.e. the SADP Control PDU (or header only) is always 1-octet.

The SDAP end-marker is a 1-byte indication per relocated QoS Flow

As the end marker (for example as a SDAP control PDU) can be lost over the air, a timer can be used to allow completion of a remapping procedure. I.e. the time to release the PDUs buffered for data on the target DRB. The length of this timer can be set in the NW based on QoS characteristics or other criteria associated with the flows that are being buffered. Assuming a receiver side buffering solution, i.e. at the gNB, no particular specification or behaviour is required as this is entirely up to gNB implementation.

# Conclusion

In section 2 we made the following observations:

- As the PDCP is not aware of QoS Flows, it needs to block all traffic on DRB meanwhile QFI is being relocated.

- With the new QoS framework and Default DRB concept, it does not seem practical to relocate one QFI at the time.

Based on the discussion in section 2 we propose the following:

- Confirm that multiple QoS Flows relocated simultaneously is supported

The SDAP end-marker is a 1-byte indication per relocated QoS Flow