3GPP TSG-RAN WG2 #102	R2-1807322

# Busan, Republic of Korea, 21st – 25th May 2018

Agenda Item:	9.18.5

Source:	Ericsson

Title:	Connection control for UEs without subscription

Document for:	Discussion, Decision

# 1	Introduction

In this paper, we consider the scenario where eNB has identified a UE that may be air-borne but the UE does not have a subscription.  We then further discuss a relevant feature for eNB to perform necessary control for such UE without subscription.

This paper has been updated based on offline feedbacks and online discussions in RAN2#101bis.

# 2	Discussion

In [1], SA2 has specified that the subscription information can authorize the user to operate aerial UE function over E-UTRAN is supported. The subscription information is signaled from MME to eNB during connection setup procedure and from source eNB to target eNB during the mobility procedures. The eNB may use this information together with UE radio capability to perform the necessary control using the relevant feature(s).

It has been clear throughout the WI that we will specify new features to support aerial UE function for an UE with subscription. However, it is not clear which of these relevant feature(s) in the specification can be used for eNB to perform necessary control for an UE without subscription.

One straightforward solution is that eNB releases the RRC connection so that this UE does not have connection. The problem is that eNB cannot prevent the UE to immediately re-connect again. The earliest point in time when eNB can know UE’s identity is in the RRCConnectionRequest message where S-TMSI is sent as UE identity. After that eNB can reject the RRC connection, but UE can still retry to connect again and again which consumes radio resources (e.g. RACH-resources). There are a couple of drawbacks:

- Resources are wasted during the repetitive initial access, i.e., resources are unnecessarily used for transmitting Message 1 to Message 4 in vain.

- This does not prevent the UE to connect to the same cell or another cell associated with another eNB (most likely after reaching maximum preambleTransMax). Since flying UE can see more cells [1], this will certainly continue for a very long time. It further increases utilization on random-access resources.

- eNB needs to keep track of the UE identity (S-TMSI) which has been identified as one without subscription.

Directly releasing RRC connection for an UE without subscription cannot prevent the UE to reconnect immediately.

To prevent the UE to reconnect again, eNB can add extendedWaitTime in RRCConnectionRelease and RRCConnectionReject messages. This mechanism was introduced for MTC with Delay Tolerant access in which UE shall not reconnect if the timer has not expired. In more detail, if the UE supports the delay-tolerant-access feature, then RRC layer forwards the received extendedWaitTime to the NAS layer which then backs off from NAS request.

The similar approach can be used here to prevent an UE without subscription to reconnect immediately.  The usage of extendedWaitTime is only possible if the UE supports “Delay Tolerant access”, so to solve the above problem RAN2 can decide that Aerial UEs shall support the extended wait time. One example can be shown as below

extendedWaitTime Value in seconds for the wait time for Delay Tolerant access requests and for Aerial UE access request.

Alternatively, it can be mandated that Aerial UEs shall support Delay tolerant access by describing in 36.306 that it is mandatory to support Delay Tolerant access as specified in 36.331 for a UE supporting Rel-15 Aerial UE features. Exact wording would depend on how Aerial UE capabilities are going to be captured.

Upon receiving “extendedWaitTime” in RRC message, UE is back-offed from NAS request for the configured time.

“extendedWaitTime” is supported only for delay-tolerant access and aerial UE should also support this.

It has been mentioned that “WaitTime” is supported in RRC Reject message. The longest waiting time that can be configured is 16 seconds, which is too short. More importantly, this mechanism only prevents the UE to access one particular cell for a configurable time and UE can still re-select the other cell during this time. It is written in RRC that if UE finds another cell for cell reselection (Section 5.3.3.5), then the UE stops the timer T302 associated with “waitTime”. This becomes problematic because air-borne UEs can see multiple cells and one can expect a ping-pong effect in which just after the waitTime timer is started after a rejection from one cell, the timer is stopped due to the re-selection of another cell.

“WaitTime” in RRC message can only bar UE’s access to one cell.

Another approach is that in the RRC release cause, eNB adds that the reason for releasing the connection is that UE flies above the altitude threshold in which it does not have any subscription. One example is to add a new field in the information element “ReleaseCause” as follows:

ReleaseCause ::=				ENUMERATED {loadBalancingTAUrequired,

other, cs-FallbackHighPriority-v1020, rrc-Suspend-v1320, flyingWithoutSubscription}

However, we must define a UE action when such messages are received. Some candidate examples can be that: it waits a configured time before re-connecting or it re-connects when the altitude is lower than a threshold. All of these seem appealing but require a significant amount of work not feasible in the last meeting of this release. In the first example, we aim to achieve something exactly as extendWaitTime but with more specification work. In the second example, it is not clear how to mandate this and how to test this.

It would be possible to add the field “flyingWithoutSubscription” in the IE “ReleaseCause”, but it requires some specification work to define a UE action when this message is received.

Based on the above discussions and observations, adding the support of “extendedWaitTime” can fulfil the purpose and is simple and straightforward.

Aerial UE support the use of extendedWaitTime in RRC message.

# Conclusion

In the previous sections we made the following observations:

Observation 1	Directly releasing RRC connection for an UE without subscription cannot prevent the UE to reconnect immediately.

Observation 2	Upon receiving “extendedWaitTime” in RRC message, UE is back-offed from NAS request for the configured time.

Observation 3	“extendedWaitTime” is supported only for delay-tolerant access and aerial UE should also support this.

Observation 4	“WaitTime” in RRC message can only bar UE’s access to one cell.

Observation 5	It would be possible to add the field “flyingWithoutSubscription” in the IE “ReleaseCause”, but it requires some specification work to define a UE action when this message is received.

Based on the discussion in the previous sections we propose the following:

Proposal 1	Aerial UE support the use of extendedWaitTime in RRC message.

# References

S2-184058, CR for 23.401:Subscription for Aerial UE in 3GPP system, SA WG#127, Sanya, 2018