3GPP TSG-RAN WG2 #102	Tdoc R2-1807666

Busan, Korea, 21-25 May, 2018

Title:	Draft LS to SA5 on L2 measurements

Release:	Rel-15

Work Item:	NR_newRAT-Core

Source:	RAN2

To:	SA5, RAN3

Contact Person:

#### Name:	Riikka Susitaival

E-mail Address:	riikka.susitaival@ericsson.com

1. Overall Description:

Question 1: 3GPP SA5 would like to ask 3GPP RAN2 group to agree to the temporary way of working or propose a new way so that L2 radio measurements can be used by the management system in Rel-15.

Answer 1: RAN2 is fine with approach.

Question 2: 3GPP SA5 ask whether the measurements in the attached contributions can be approved by RAN2.

Answer 2:

pCR S5-182557

This pCR proposes packet drop measurements in the DL. The proposed measurements are following:

  - DL packet drop in gNB-CU-UP or eNB (option 3) on PDCP SDU level

    - Per 5QI or QCI

  - DL packet drop in gNB-DU on RLC SDU level

It is notified that DL packet drop in gNB-CU-UP is similar to existing measurement in 36.314. Main new thing is that instead of QCI, measurement is done per 5QI. In principle, 5QI is not known in the PDCP protocol, so it is not possible to know the (original) 5QI for each PDCP packet for a DRB if several QoS flows are mapped to the same DRB. However, there are alternative ways to do the measurement:

- RAN2 understanding is that all QoS flows mapped to one DRB get the same QoS treatment. Thus also resulting packet discard rate is same for all flows/5QIs mapped to the same DRB (and PDCP entity). QoS profile of the DRB can be called as “mapped 5QI”. Measurements can be performed per mapped 5QI (and thus per DRB). The mapped 5QI used for the DRB is used to determine which counter to step, instead of the 5QI of the individual flow.

- Perform measurements by 5QI by associating each PDCP SDU to the SDAP SDU and derive 5QI based on that.  This is more complex than having measurements per DRB.

From RAN2 point of view, packet drop in gNB-DU level per RLC SDU is similar to the previous measurement in the PDCP SDU level. However, 5QI is not known in the gNB-DU and thus this measurement cannot currently be performed per each QoS flow separately. However, the measurements can be performed per each RLC entity which would then correspond to the QoS treatment of all flows.

In Option 3 case, QCI is known in the eNB and thus measurements can be done per QCI. This is similar to LTE measurement in 36.314.

In general, RAN3 is responsible for CU-DU split and thus feasibility of measurements need to be confirmed in RAN3 as well.

pCR S5-182556

This pCR proposes UL packet loss on PDCP SDU level in gNB-CU-UP. This measurement provides the fraction of PDCP SDU packets which are lost (not successfully received) on the air interface or on the F1-U for the 3-split scenario in the uplink.  It is suggested that the measurement is per 5QI or QCI.

From RAN2 point of view, there is no much difference between LTE and NR measurements except that measurement is also per 5QI. It is proposed to monitor lost packets based on PDCP SNs. However, 5QI of each lost PDCP PDU cannot be known. Thus it is understood that measurement should be done per PDCP entity (corresponding to one DRB) and all QoS flows mapped to that entity share same result. By this way, this is considered to be feasible. However, also in this case, RAN3 confirmation is needed.

pCR S5-182422

Finally, S5-182422 introduces UL/DL F1-U packet loss rate in F1-U interface. This measurement provides the fraction of PDCP SDU packets which are lost (not successfully received) on the F1-U interface for 3-split in the uplink.  The measurement is optionally split into subcounters per QoS level and is done based GTP-U headers.

RAN3 is responsible for F1-U packet loss rate and thus it is meaningful to discuss this in RAN3.

2. Actions:

To SA5:

Take the reply in this LS into account.

To RAN3:

RAN3 is suggested to analyse measurements as presented in R2-1806358 and discussed in R2-1807665

3. Date of Next RAN2 Meetings:

RAN2-AH-1807	2nd – 6th July 2018, Montreal, Canada

RAN2-103	20-24 August 2018, Gothenburg, Sweden