3GPP TSG RAN WG1 Meeting #102	                       R2-1807669

Busan, Korea, May 21st – 25th, 2018

Source:	ITRI

Title:	Remaining issues on Resource Pool Sharing for Mode 3 and Mode 4 V2X

Agenda Item:	9.10.3

Document for:	Discussion and Decision

Introduction

According to chairman’s note for RAN2#101 [1], the related agreements for resource pool sharing are shown as follows,

Agreements

1: Support resource pool sharing between Rel-15 mode-3 and Rel-15 mode-4 UE.

2: Support resource pool sharing between Rel-15 mode-3 and Rel-14 mode-4 UE.

3: Not support resource pool sharing between Rel-14 mode-3 and Rel-15 mode-4 UE.

4: Full resource pool sharing is supported. Partial resource pool sharing scenario is deprioritized in Rel-15.

5: Reuse Rel-14 single pool configuration for mode-3, no enhancement is needed.

6: It is up to RAN1 to decide on the tool of non-zero reservation bits for resource pool sharing.

7: It is up to RAN1 to decide on the tool of mode indicator in SCI for resource pool sharing.

8: FFS on the need of support of new mode-3 sensing report for resource pool sharing.

For last RAN2 meeting, there is no consensus on the need of sensing and reporting from the mode-3 UE for resource pool sharing. In this contribution, we present our views on these subjects.

The design assisting to the resource pool sharing for Mode 3 and Mode 4 in V2X

According to the conclusion of [1], the sharing pool is supported in designated cases. The discussions now should be focused on the practical manner to improve the efficient of resource allocation in the sharing pool. In order to avoid the potential collisions and interference, Rel-15 Mode 4 UEs should not use the resource blocks which have been occupied by Rel-15 Mode 3 UEs. Hence, we agree that Rel-15 Mode 3 UEs shall utilize the resource reservation field in SCI-1 to indicate the information about the resource utilization. By acquiring that, Rel-15 Mode 4 UEs may avoid the access of the resource blocks which may have been occupied. According to the conclusion of [2], the resource reservation field in SCI-1 has been agreed to be able to use for assisting the resource allocation of pool sharing between mode 3 and mode 4.

- Observation 1:  The resource reservation field in SCI-1 has been agreed in [2] to be able to use for assisting the resource allocation of pool sharing between mode 3 and mode 4.

V2X UE-assisted resource allocation

To prevent collisions, Mode 4 TxUE respects the channel usage and conduct the sensing before transmitting brutally. We suggest this act should apply to Mode 3 UE. Since in Mode 3, the resource is eNB-controlled, the sensing result reporting to eNB seems to be necessary. We suggest that Mode 3 TxUE perform the sensing and report to assist eNB to conduct an efficiency resource allocation not only by taking the potential interference from Mode 4 TxUE into account but also respect the transmission of Mode 4 TxUE. On the other hand, how to provide the measurement report to assist eNB in resource allocation needs to be further discussed. Also, it needs to be discussed that the measurement report is obligated to provide by Mode 3 TxUE or provided based on request. It may also be highly connected to the detail arrangement of the resource pool allowing accessed by Mode 3 and Mode 4 UE coexisted nearby.

- eNB may require the assistance from Mode 3 UE near the edge to provide its measurement report in order to have an efficient resource allocation. It is not just for respecting the transmission of Mode 4 but also to take the potential interference into account.

- The measurement reports required and the corresponding configurations have to be decided.

Conclusion

In this paper, we highlight the remaining issues should be discussed for supporting coexistence of mode 3 and mode 4 in V2X communications.

- Observation 1:  The resource reservation field in SCI-1 has been agreed in [2] to be able to use for assisting the resource allocation of pool sharing between mode 3 and mode 4.

- eNB may require the assistance from Mode 3 UE near the edge to provide its measurement report in order to have an efficient resource allocation. It is not just for respecting the transmission of Mode 4 but also to take the potential interference into account.

- The measurement reports required and the corresponding configurations have to be decided.

[1] 3GPP RAN2#101 Chairman’s note, Athens, Mar. 2018

[2] 3GPP RAN1#92 Chairman’s note, Athens, Mar. 2018