3GPP TSG-RAN WG2 Meeting #102	R2-1807737

# Busan, Korea, 21st - 25th May 2018

| CR-Form-v11.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 36.321 | CR | 1206 | rev | 4 | Current version: | 13.8.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | X | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Clarification for DPR Update |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | MediaTek Inc. |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R2 |  |  |  |  |  |  |  |  |  |
| Work item code: | NB_IoT-Core |  |  |  |  |  |  | Date: |  | 2018-05-10 |
| Category: | F |  |  |  |  |  |  | Release: |  | Rel-13 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) Rel-12 (Release 12) Rel-13 (Release 13) Rel-14 (Release 14) Rel-15 (Release 15) Rel-16 (Release 16) |  |
| Reason for change: |  | According to TS 36.321, when first successful Random Access Response (RAR) reception occurs during RA, UE constructs Msg3. In this Msg3, there is also Data Volume and Power Headroom Report (DPR) MAC CE. (See Chapter 5.1.4, TS 36.321) If contention resolution fails, UE performs new preamble Tx and upon new successful RAR reception (during the same RA), UE retransmits Msg3 from Msg3 buffer. That is, the exactly same TB is retransmitted. (See Chapter 5.4.2, TS 36.321). In LTE, this is fine as Msg3 is pure CCCH SDU, i.e. RRC PDU. But in NB-IoT, the Msg3 contains also DPR MAC CE. With current specifications, the Data Volume and Power Headroom values indicated in the Msg3 is determined at the moment when the first successful RAR reception during the RA procedure occurred, and can be outdated upon RAR reception for RACH reattempt, especially in case of CE level ramping. |  |  |  |  |  |  |  |  |
| Summary of change: |  | In HARQ operation (5.4.2.1), add procedural text stating that if there is a MAC PDU containing DPR MAC CE in the Msg3 buffer and the uplink grant was received in a RAR, the MAC entity should update the DPR MAC CE in the MAC PDU in the Msg3 buffer . Impact analysis Impacted functionality: Data Volume and Power Headroom reporting in NB-IoT. Inter-operability: If the network is implemented according to this CR while the UE is not, the reported Data Volume and Power Headroom values may be outdated. If the UE is implemented according to this CR while the network is not, there is no inter-operability issue. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | The UE may be sending outdated Data Volume and Power Headroom values in Msg3 upon RAR reception after RACH reattempt. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 5.4.2.1 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  |  | Other core specifications |  |  | TS/TR ... CR ... |  |  |  |
| affected: |  |  |  | Test specifications |  |  | TS/TR ... CR ... |  |  |  |
| (show related CRs) |  |  |  | O&M Specifications |  |  | TS/TR ... CR ... |  |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |

[Start of Change]

5.4.2.1	HARQ entity

There is one HARQ entity at the MAC entity for each Serving Cell with configured uplink, which maintains a number of parallel HARQ processes allowing transmissions to take place continuously while waiting for the HARQ feedback on the successful or unsuccessful reception of previous transmissions.

The number of parallel HARQ processes per HARQ entity is specified in [2], clause 8. NB-IoT has one UL HARQ process.

When the physical layer is configured for uplink spatial multiplexing [2], there are two HARQ processes associated with a given TTI. Otherwise there is one HARQ process associated with a given TTI.

At a given TTI, if an uplink grant is indicated for the TTI, the HARQ entity identifies the HARQ process(es) for which a transmission should take place. It also routes the received HARQ feedback (ACK/NACK information), MCS and resource, relayed by the physical layer, to the appropriate HARQ process(es).

In asynchronous HARQ operation, a HARQ process is associated with a TTI based on the received UL grant except for UL grant in RAR. Except for NB-IoT, each asynchronous HARQ process is associated with a HARQ process identifier. For UL transmission with UL grant in RAR, HARQ process identifier 0 is used. HARQ feedback is not applicable for asynchronous UL HARQ.

When TTI bundling is configured, the parameter TTI_BUNDLE_SIZE provides the number of TTIs of a TTI bundle. TTI bundling operation relies on the HARQ entity for invoking the same HARQ process for each transmission that is part of the same bundle. Within a bundle HARQ retransmissions are non-adaptive and triggered without waiting for feedback from previous transmissions according to TTI_BUNDLE_SIZE. The HARQ feedback of a bundle is only received for the last TTI of the bundle (i.e the TTI corresponding to TTI_BUNDLE_SIZE), regardless of whether a transmission in that TTI takes place or not (e.g. when a measurement gap occurs). A retransmission of a TTI bundle is also a TTI bundle. TTI bundling is not supported when the MAC entity is configured with one or more SCells with configured uplink.

Uplink HARQ operation is asynchronous for NB-IoT UEs, BL UEs or UEs in enhanced coverage except for the repetitions within a bundle.

For NB-IoT UEs, BL UEs or UEs in enhanced coverage, the parameter UL_REPETITION_NUMBER provides the number of transmission repetitions within a bundle. For each bundle, UL_REPETITION_NUMBER is set to a value provided by lower layers. Bundling operation relies on the HARQ entity for invoking the same HARQ process for each transmission that is part of the same bundle. Within a bundle HARQ retransmissions are non-adaptive and are triggered without waiting for feedback from previous transmissions according to UL_REPETITION_NUMBER. An uplink grant corresponding to a new transmission or a retransmission of the bundle is only received after the last repetition of the bundle. A retransmission of a bundle is also a bundle.

TTI bundling is not supported for RN communication with the E-UTRAN in combination with an RN subframe configuration.

For transmission of Msg3 during Random Access (see subclause 5.1.5) TTI bundling does not apply. For NB-IoT UEs, BL UEs or UEs in enhanced coverage, uplink repetition bundling is used for transmission of Msg3.

For each TTI, the HARQ entity shall:

-	identify the HARQ process(es) associated with this TTI, and for each identified HARQ process:

-	if an uplink grant has been indicated for this process and this TTI:

-	if the received grant was not addressed to a Temporary C-RNTI on PDCCH and if the NDI provided in the associated HARQ information has been toggled compared to the value in the previous transmission of this HARQ process; or

-	if the uplink grant was received on PDCCH for the C-RNTI and the HARQ buffer of the identified process is empty; or

-	if the uplink grant was received in a Random Access Response:

-	if there is a MAC PDU in the Msg3 buffer and the uplink grant was received in a Random Access Response:

-	if the MAC PDU in the Msg3 buffer contains the Data Volume and Power Headroom Report MAC control element:

-	the MAC entity should update the Data Volume and Power Headroom Report MAC control element in the MAC PDU in the Msg3 buffer;

-	obtain the MAC PDU to transmit from the Msg3 buffer.

-	else:

-	obtain the MAC PDU to transmit from the "Multiplexing and assembly" entity;

-	deliver the MAC PDU and the uplink grant and the HARQ information to the identified HARQ process;

-	instruct the identified HARQ process to trigger a new transmission.

-	else:

-	deliver the uplink grant and the HARQ information (redundancy version) to the identified HARQ process;

-	instruct the identified HARQ process to generate an adaptive retransmission.

-	else, if the HARQ buffer of this HARQ process is not empty:

-	instruct the identified HARQ process to generate a non-adaptive retransmission.

When determining if NDI has been toggled compared to the value in the previous transmission the MAC entity shall ignore NDI received in all uplink grants on PDCCH for its Temporary C-RNTI.

[End of Change]