3GPP TSG-RAN WG2 #102	R2-1808783

# Busan, Republic of Korea, 21st – 25th May 2018

Agenda Item:	10.3.1.11

Source:	Ericsson

Title:	MAC CE for duplication

Document for:	Discussion, Decision

# Introduction

In the email discussion on Control for Duplication [1] regarding the MAC CE design for activation and deactivation of PDCP duplication, the following was proposed:

Proposal: RAN2 to discuss and decide among the following options:

Option 1: per MAC entity mapping for both DC and CA duplication, no further standardization effort other than clarify it in MAC that the bitmap only concerns DRBs of the CG.

Option 2: per UE mapping for both DC and CA duplication, without additional inter-node message/X2AP elements for updating CA configuration/activation status nor additional requirement of MeNB comprehension of configurations from SgNB, hence CA duplication with DC is not supported for Rel-15.

Option 3: per UE mapping for both DC and CA duplication, with additional inter-node message/X2AP elements to be defined to support CA duplication with DC, which requires revisit of some previous agreements as well as further control plane and RAN3 involvement.

In this contribution we provide a way-forward for this remaining open issue.

# Discussion

The options summarized in the email discussion [1] all have shortcomings in terms of

- Simplicity in design and independence of cell group

- Common applicability to DC and CA duplication

- Flexibility for network to utilize any node to control duplication without need for specification

The main reason for this is that a bitmap referring to DRBs configured with duplication is to be used. The node sending the MAC CE needs to know the duplication activation/deactivation status of all DRBs to now interfere with a MAC CE sent previously from the other node. Trying to solve this by option 1, i.e. per MAC entity, defeats on the other hand the flexibility in the network to use any node to control the duplication.

A simple way out of this dilemma is to use an explicit DRB ID in the MAC CE, instead of the bitmap. This way, to handle simultaneous activation/deactivation of duplications for multiple bearers, multiple MAC CEs would be required, coming with a somewhat higher overhead (in the scenario of simultaneous activation/deactivation. Given however that the overhead of these non-continuous control messages is minimal compared to overhead of duplicate transmission generally, in the unlikely scenario where several duplication bearers are activated/deactivated simultaneously, we believe that utilizing the bitmap is not justified. Using the explicit DRB ID is sufficient, and appropriate to solve the above dilemma situation.

A remaining detail would be whether a MAC CE with a DRB would toggle the duplication status or a bit should indicate activation/deactivation. We believe that the most flexible way is to include a bit, so that subsequent MAC CEs cannot override each other.

In conclusion, we propose:

RAN2 to consider MAC CE for duplication control containing single DRB ID instead of bitmap, and bit to indicate activation/deactivation.

No further optimizations are necessary and no inter-node signalling is required.

# Conclusion

Based on the discussion in section 2 we propose the following:

Proposal 1	RAN2 to consider MAC CE for duplication control containing single DRB ID instead of bitmap to toggle duplication activation status.

# References

- R2-1807597, Email report [101bis#74][NR UP] Control of Duplication (Nokia), 3GPP TSG-RAN WG2 Meeting #102, Busan, South Korea, 21st - 25th May 2018