3GPP TSG-RAN WG2 Meeting #102	R2-1808825

# Busan, Republic of Korea, 21st – 25th May 2018

| CR-Form-v11.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.321 | CR | 0103 | rev | - | Current version: | 15.1.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | X | Radio Access Network | X | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | CR for alternative (a) |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: |  |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R2 |  |  |  |  |  |  |  |  |  |
| Work item code: | NR_newRAT-Core |  |  |  |  |  |  | Date: |  | 2018-05-10 |
| Category: | F |  |  |  |  |  |  | Release: |  | Rel-15 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) Rel-12 (Release 12) Rel-13 (Release 13) Rel-14 (Release 14) Rel-15 (Release 15) Rel-16 (Release 16) |  |
| Reason for change: |  | RAN1 confirmed the need for beamFailureRecoveryTimer in the context of CFRA and CBRA. The behaviour according to current RAN1 agreement should be as follows: “Upon expiry of beamFailureRecoveryTimer means that UE shall not use CFRA for BFR after beamFailureRecoveryTimer expired and no indication to higher layer is required.” |  |  |  |  |  |  |  |  |
| Summary of change: |  | The following changes are made: - In subclause 5.1.2: The selection of contnetion free PRACH resources for a random access procedure triggered by beam failure recovery is subject to having the beamFailureRecoveryTimer running or not configured. - In subclause 5.17: The beamFailureRecoveryTimer is started upon detecting a BFR, and stopped upon the successful completion of the associated randome access procedure. The timer is optionally configured by RRC, part of BFR config IE Impact analysis Impacted functionality: Beam Failure Recovery Inter-operability: No inter-operability issues are foreseen. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | SSBs with only CBRA resources cannot be selected by the UE if SSBs/CSI-RSs with CFRA resources above threshold level are available. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 5.1.2 5.17 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  | TS/TR 38.331 CR R2-1808176 |  |  |  |
| affected: |  |  | X | Test specifications |  |  | TS/TR ... CR ... |  |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  | TS/TR ... CR ... |  |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |

<Start of modification 1>

### 5.1.2	Random Access Resource selection

The MAC entity shall:

1>	if the Random Access procedure was initiated for beam failure recovery (as specified in subclause 5.17); and

1>	if the beamFailureRecoveryTimer (in subclause 5.17) is either running or not configured; and

1>	if the contention-free Random Access Resources for beam failure recovery request associated with any of the SSBs and/or CSI-RSs have been explicitly provided by RRC; and

1>	if at least one of the SSBs with SS-RSRP above rsrp-ThresholdSSB amongst the associated SSBs or the CSI-RSs with CSI-RSRP above csirs-Threshold amongst the associated CSI-RSs is available:

2>	select an SSB with SS-RSRP above rsrp-ThresholdSSB amongst the associated SSBs or a CSI-RS with CSI-RSRP above csirs-Threshold amongst the associated CSI-RSs;

2>	set the PREAMBLE_INDEX to a ra-PreambleIndex corresponding to the selected SSB or CSI-RS from the set of Random Access Preambles for beam failure recovery request.

1>	else if the ra-PreambleIndex has been explicitly provided by either PDCCH or RRC; and

1>	if the ra-PreambleIndex is not 0b000000; and

1>	if contention-free Random Access Resource associated with SSBs or CSI-RS have not been explicitly provided by RRC:

2>	set the PREAMBLE_INDEX to the signalled ra-PreambleIndex.

1>	else if the contention-free Random Access Resources associated with SSBs have been explicitly provided by RRC and at least one SSB with SS-RSRP above rsrp-ThresholdSSB amongst the associated SSBs is available:

2>	select an SSB with SS-RSRP above rsrp-ThresholdSSB amongst the associated SSBs;

2>	set the PREAMBLE_INDEX to a ra-PreambleIndex corresponding to the selected SSB.

1>	else if the contention-free Random Access Resources associated with CSI-RSs have been explicitly provided by RRC and at least one CSI-RS with CSI-RSRP above csirs-Threshold amongst the associated CSI-RSs is available:

2>	select a CSI-RS with CSI-RSRP above csirs-Threshold amongst the associated CSI-RSs;

2>	set the PREAMBLE_INDEX to a ra-PreambleIndex corresponding to the selected CSI-RS.

1>	else:

2>	if at least one of the SSBs with SS-RSRP above rsrp-ThresholdSSB is available:

3>	select an SSB with SS-RSRP above rsrp-ThresholdSSB.

2>	else:

3>	select any SSB.

2>	if Msg3 has not yet been transmitted:

3>	if Random Access Preambles group B exists; and

3>	if the potential Msg3 size (UL data available for transmission plus MAC header and, where required, MAC CEs) is greater than ra-Msg3SizeGroupA and the pathloss is less than PCMAX (of the Serving Cell performing the Random Access Procedure) – preambleReceivedTargetPower – deltaPreambleMsg3 – messagePowerOffsetGroupB:

4>	select the Random Access Preambles group B.

3>	else:

4>	select the Random Access Preambles group A.

2>	else (i.e. Msg3 is being retransmitted):

3>	select the same group of Random Access Preambles as was used for the Random Access Preamble transmission attempt corresponding to the first transmission of Msg3.

2>	if the association between Random Access Preambles and SSBs is configured:

3>	select a ra-PreambleIndex randomly with equal probability from the Random Access Preambles associated with the selected SSB and the selected Random Access Preambles group.

2>	else:

3>	select a ra-PreambleIndex randomly with equal probability from the Random Access Preambles within the selected Random Access Preambles group.

2>	set the PREAMBLE_INDEX to the selected ra-PreambleIndex.

1>	if an SSB is selected above and an association between PRACH occasions and SSBs is configured:

2>	determine the next available PRACH occasion from the PRACH occasions corresponding to the selected SSB permitted by the restrictions given by the ra-ssb-OccasionMaskIndex if configured (the MAC entity may take into account the possible occurrence of measurement gaps when determining the next available PRACH occasion corresponding to the selected SSB).

1>	else if a CSI-RS is selected above and an association between PRACH occasions and CSI-RSs is configured:

2>	determine the next available PRACH occasion from the PRACH occasions in ra-OccasionList corresponding to the selected CSI-RS (the MAC entity may take into account the possible occurrence of measurement gaps when determining the next available PRACH occasion corresponding to the selected CSI-RS).

1>	else:

2>	determine the next available PRACH occasion (the MAC entity may take into account the possible occurrence of measurement gaps when determining the next available PRACH occasion).

1>	perform the Random Access Preamble transmission procedure (see subclause 5.1.3).

<End of modification 1>

<Start of modification 2>

## 5.17	Beam Failure Detection and Recovery procedure

The MAC entity may be configured by RRC with a beam failure recovery procedure which is used for indicating to the serving gNB of a new SSB or CSI-RS when beam failure is detected on the serving SSB(s)/CSI-RS(s). Beam failure is detected by counting beam failure instance indication from the lower layers to the MAC entity.

RRC configures the following parameters in the BeamFailureRecoveryConfig for the Beam Failure Detection and Recovery procedure:

-	beamFailureInstanceMaxCount for the beam failure detection;

-	beamFailureDetectionTimer for the beam failure detection;

-	beamFailureRecoveryTimer for the beam failure recovery procedure;

-	beamFailureCandidateBeamThreshold: an RSRP threshold for the beam failure recovery;

-	preamblePowerRampingStep: preamblePowerRampingStep for the beam failure recovery;

-	preambleReceivedTargetPower: preambleReceivedTargetPower for the beam failure recovery;

-	preambleTxMax: preambleTxMax for the beam failure recovery;

-	ra-ResponseWindow: the time window to monitor response(s) for the beam failure recovery using contention-free Random Access Preamble.

The following UE variables are used for the beam failure detection procedure:

-	BFI_COUNTER: counter for beam failure instance indication which is initially set to 0.

The MAC entity shall:

1>	if beam failure instance indication has been received from lower layers:

2>	start or restart the beamFailureDetectionTimer;

2>	increment BFI_COUNTER by 1;

2>	if BFI_COUNTER = beamFailureInstanceMaxCount + 1:

3>	start the beamFailureRecoveryTimer, if configured;

3>	initiate a Random Access procedure (see subclause 5.1) on the SpCell by applying the parameters configured in BeamFailureRecoveryConfig.

1>	if the beamFailureDetectionTimer expires:

2>	set BFI_COUNTER to 0.

1>	if the Random Access procedure is successfully completed (see subclause 5.1):

2>	stop the beamFailureRecoveryTimer, if configured;

2>	consider the Beam Failure Recovery procedure successfully completed.

<End of modification 2>