3GPP TSG-RAN WG2 Meeting #102	R2-1808898

Busan, Korea, 21-25 May 2018

Title:	LS on Addition of access/load control for CE

Response to:

# Release:	Rel-15

# Work Item:	LTE_eMTC4-Core, NB_IOTenh2-Core

Attachment: 	None

Source:	RAN2

To:	SA1

Cc:

# Contact Person:

# Name:	Steve Bennett

E-mail Address:	sbennett@sierrawireless.com

# 1. Overall Description:

RAN2 has agreed to introduce enhanced access/load control for EAB and coverage extension capable UEs, and for NB-IoT UEs, using specific enhanced coverage level(s) by temporarily barring UEs from using specific (N)PRACH resources. This barring only applies to UEs that belong to normal access classes (0 to 9).

Agreements - Add new bits in system information for Rel-15 UEs that will bar them from network access in enhanced coverage. - Add SIB14 bits for indicating 4 levels of CE barring for eMTC UEs applied as PRACH resource barring. - Add SIB14-NB to indicate 3 levels of PRACH resource barring for NB-IoT UE. - All eMTC that support EAB and NB-IoT UEs that belong to only access classes 0 to 9, i.e. if the network does not assign a special class to the UE, is barred with this mechanism regardless of PLMN. - No CE/RSRP level barring with respect to access classes 0 to 9. - Existing mechanism, i.e. eab-ParamModification/ab-enabled, is used to indicate the related system information update in SIB14 and SIB14-NB. - Emergency (eMTC) and MO exception data (NB-IoT) are exempted from this barring mechanism.

RAN2 has noted that these changes may impact the service requirements in TS22.011.

# 2. Actions:

To SA1:

RAN2 respectfully asks SA1 to take note of the agreements above, and if necessary provide feedback.

# 3. Date of Next TSG RAN WG2 Meetings:

TSG RAN WG2 Meeting #103	20 – 24 August 2018   Gothenburg, Sweden

TSG RAN WG2 Meeting #103bis	8 - 12 October 2018    Chengdu, China