3GPP TSG-RAN WG2#102				Tdoc R2-1809118

Busan, Korea, 21st - 25th May 2018

Agenda Item:	10.4.1.6.7

Source:	Ericsson

Title:	TP for the ASN.1 to provide SIBs via dedicated signalling

Document for:	Discussion, Decision

# 1	Introduction

During RAN2#102, it was agreed to introduce support for providing SIBs to the UE using dedicated signalling:

Agreements

1	On SI modification indication, MIB contents are not re-acquired by UEs' in RRC_CONNECTED

2	MIB does not need to be provided via dedicated signalling, as all the required information can be provided to the UE via servingCellConfigCommon.

3: 	If the UE’s active DL BWP is not configured with common search space then UE is not required to acquire SI updates from broadcast

4: 	NW may provide updated SI by dedicated signalling (Can be used at least for the case if the UE’s active DL BWP is not configured with common search space.)

5	RRCReconfiguration message is used to provide SIBs through dedicated

6	RRCReconfiguration can perform SIB updates (other than for SIB1 parameters that are provided via servingCellConfigCommon) without reconfiguration with sync

7	No need for additional requests from the UE to the NW to determine which SIBs to be provided in dedicated signalling.

8	Any SIB (including PWS SIBs) can be provided to the UE through dedicated signaling.

9:	How long updated SI is broadcast in UE’s active DL BWP, when SI-RNTI and P-RNTI common search space is configured, is left to network implementation.

=>	TP for the ASN.1  to be provided in R2-1809118 (Offline discussion #69, Ericsson)

Section 2 of this document contains the corresponding text proposal. Note that the text proposal is based on R2-1808000.

# 2	Text Proposal

#### –	RRCReconfiguration

The RRCReconfiguration message is the command to modify an RRC connection. It may convey information for measurement configuration, mobility control, radio resource configuration (including RBs, MAC main configuration and physical channel configuration) including and security configuration.

Signalling radio bearer: SRB1 or SRB3

RLC-SAP: AM

Logical channel: DCCH

Direction: Network to UE

RRCReconfiguration message

-- ASN1START

-- TAG-RRCRECONFIGURATION-START

RRCReconfiguration ::= 				SEQUENCE {

rrc-TransactionIdentifier			RRC-TransactionIdentifier,

criticalExtensions					CHOICE {

rrcReconfiguration					RRCReconfiguration-IEs,

criticalExtensionsFuture			SEQUENCE {}

}

}

RRCReconfiguration-IEs ::= 			SEQUENCE {

radioBearerConfig						RadioBearerConfig 														OPTIONAL, -- Need M

secondaryCellGroup						OCTET STRING (CONTAINING CellGroupConfig)								OPTIONAL, -- Need M

measConfig								MeasConfig																OPTIONAL, -- Need M

lateNonCriticalExtension				OCTET STRING															OPTIONAL,

nonCriticalExtension					RRCReconfiguration-vxx-IEs																	OPTIONAL

}

RRCReconfiguration-vxx-IEs ::= 			SEQUENCE {

masterCellGroup							OCTET STRING (CONTAINING CellGroupConfig)								OPTIONAL, -- Need M

fullConfig								ENUMERATED {true} 														OPTIONAL,

dedicatedSIB1-Delivery				OCTET STRING (CONTAINING SIB1)										OPTIONAL,

dedicatedSystemInformationDelivery	OCTET STRING (CONTAINING SystemInformation)							OPTIONAL,

nonCriticalExtension					SEQUENCE {}																OPTIONAL

}

-- TAG-RRCRECONFIGURATION-STOP

-- ASN1STOP

RRCReconfiguration-IEs field descriptions

fullConfig Indicates that the full configuration option is applicable for the RRCReconfiguration message.

masterCellGroup Configuration of master cell group.

dedicatedSIB1-Delivery Used to transmit SIB1 to the UE using dedicated signaling. The UE shall ignore the contents of the ServingCellConfigCommonSIB.

dedicatedSystemInformationDelivery Used to transmit SIBs other than SIB1 to the UE using dedicated signaling.

radioBearerConfig Configuration of Radio Bearers (DRBs, SRBs) including SDAP/PDCP. In EN-DC this field may only be present if the RRCReconfiguration is transmitted over SRB3.

secondaryCellGroup Configuration of secondary cell group (EN-DC).

Editor’s Note: FFS: the details of the conditional presence of fullConfig.