3GPP TSG-RAN WG2 Meeting #102	R2-1806988

Busan, South Korea, 21st - 25th May 2018

Agenda item:	10.4.5.6

Source:	Nokia, Nokia Shanghai Bell

Title:	Stage 3 aspects for encoding of paging message with truncated UE-ID

WID/SID:	NR_newRAT-Core - Release 15

Document for:	Discussion and Decision

# 1	Introduction

Response-driven paging has been proposed to reduce the high downlink overhead resulting from transmitting the paging message in multi-beam system [1][2]. In this contribution, we discuss different ASN. 1 encoding options of paging message for supporting both direct paging using full UE ID such as S-TMSI (LTE-like paging method) and truncated-UE ID (response-driven paging).

# 2	Discussion

The different ASN.1 encoding options for the paging message are provided in the Appendix and they are described briefly in the following:

- Option 1: The network can select to page UEs using either a paging record list corresponding to full UE-ID (pagingRecordList) or truncated UE-ID (pagingRecordList2).

- Option 2: The network can page UEs using paging record list(s) corresponding to full UE-ID (pagingRecordList) and/or truncated UE-ID (pagingRecordList2).

- Option 3: The network pages UEs using a single paging record list (pagingRecordList) with UE ID (PagingUE-Identity) corresponding either to full UE-ID (fullIdentity) or truncated UE-ID (shortIdentity). The shortIdentity structure, defining truncated UE-IDs (short S-TMSI or short I-RNTI), is defined separately from fullIdentity providing full UE-IDs.

- Option 4: The network pages UEs using a single paging record list (pagingRecordList) with UE ID (PagingUE-Identity) corresponding either to full/truncated S-TMSI or I-RNTI. In contrast to option 3, the PagingUE-Identity structure contains explicitly the full/truncated S-TMSI or I-RNTI.

For all options 1-4, the length of the truncated S-TMSI or I-RNTI is configurable, see Short-NG-5G-S-TMSI and Short-I-RNTI-Value in the appendix.

Proposal 1: ASN.1 encoding of the paging message supports configurable length for the truncated S-TMSI and I-RNTI.

The pros and cons of each option are summarized in Table 1.

|  | Option 1 | Option 2 | Option 3 | Option 4 |
| --- | --- | --- | --- | --- |
| Possibility to page UEs of same PO using full and truncated UE-IDs | No | Yes | Yes | Yes |
| ASN. 1 encoding overhead when using full UE-ID (direct paging) | Negligible as the pagingRecodList2 can be entirely omitted (1 bit for encoding “CHOICE” in Option 1 and “OPTIONAL” in Option 2) |  | Higher than Option 1 and 2 as the ASN. 1 encoding overhead scales with the number of paged UEs |  |

According to the table, Option 2 provides both 1) the flexibility to page UEs of same PO with full or truncated UE-IDs and 2) low ASN.1 encoding overhead (1 bit) if the network decides to page UE using full UE-IDs.

Observation 1: Paging UEs using paging record list(s) corresponding to full UE-ID (pagingRecordList) and/or truncated UE-ID (pagingRecordList2) provides both flexibility and low ASN.1 encoding overhead when full UE-IDs are used.

Proposal 1: RAN2 to discuss the different ASN.1 encoding options of the paging message for supporting both direct and response-driven paging.

# 3	Conclusion

## In this contribution, we have discussed the different ASN.1 encoding options of the paging message to support both direct and response-driven paging. The observations and proposals are summarized in the following:

Observation 1: Paging UEs using paging record list(s) corresponding to full UE-ID (pagingRecordList) and/or truncated UE-ID (pagingRecordList2) provides both flexibility and low ASN.1 encoding overhead when full UE-IDs are used.

Proposal 1: ASN.1 encoding of the paging message supports configurable length for the truncated S-TMSI and I-RNTI.

Proposal 1: RAN2 to discuss the different ASN.1 encoding options of the paging message for supporting both direct and response-driven paging.

# 4	Appendix

The different ASN. 1 encoding options of the paging message are provided in the following,

- Option 1: The network can select to page UEs using either a paging record list corresponding to full UE-ID (pagingRecordList) or truncated UE-ID (pagingRecordList2).

#### –	Paging

The Paging message is used for the notification of one or more UEs.

Signalling radio bearer: N/A

RLC-SAP: TM

Logical channel: PCCH

Direction: Network to UE

Paging message

-- ASN1START

-- TAG-PAGING-START

Paging ::= 							SEQUENCE {

pagingRecordList						CHOICE {

fullsizeList							PagingRecordList													OPTIONAL, -- Need M

truncatedList							PagingRecordList2													OPTIONAL -- Need N

}

systemInfoModification					ENUMERATED {true}														OPTIONAL, -- Need N

etwsAndCmasIndication					ENUMERATED {true}														OPTIONAL, -- Need N

lateNonCriticalExtension				OCTET STRING															OPTIONAL,

nonCriticalExtension					SEQUENCE{}																OPTIONAL

}

PagingRecordList ::=				SEQUENCE(SIZE(1..maxNrofPageRec)) OF PagingRecord

PagingRecordList2 ::=				SEQUENCE(SIZE(1..maxNrofPageRec)) OF PagingRecord2

PagingRecord ::=					SEQUENCE{

ue-Identity							PagingUE-Identity,

...

}

PagingRecord2 ::=					SEQUENCE{

ue-Identity							TruncatedUE-Identity,

...

}

PagingUE-Identity ::=				CHOICE{

ng-5g-s-tmsi						NG-5G-S-TMSI,

i-rnti								I-RNTI-Value,

...

}

TruncatedUE-Identity ::=			CHOICE{

short-ng-5g-s-tmsi					Short-NG-5G-S-TMSI,

short-i-rnti						Short-I-RNTI-Value,

...

}

-- TAG-PAGING-STOP

-- ASN1STOP

#### –	I-RNTI-Value

The I-RNTI-Value IE is used to identify the suspended UE context of a UE in RRC_INACTIVE.

I-RNTI-Valueinformation element

-- ASN1START

-- TAG-I-RNTI-VALUE-START

I-RNTI-Value ::=						BIT STRING (SIZE(52))

Short-I-RNTI-Value ::=					BIT STRING (SIZE(minI-RNTI..maxI-RNTI))

-- TAG-I-RNTI-VALUE-STOP

-- ASN1STOP

#### –	NG-5G-S-TMSI

The IE NG-5G-TMSI contains a 5G S-Temporary Mobile Subscription Identifier (5G-S-TMSI), a temporary UE identity provided by the 5GC which uniquely identifies the UE within the tracking area, see TS 23.003 [20].

NG-5G-S-TMSI information element

-- ASN1START

-- TAG-NG-5G-S-TMSI-START

NG-5G-S-TMSI::=							SEQUENCE {

amf-Set-Id								AMF-Set-ID,

amf-Pointer								AMF-Pointer,

ng-5g-TMSI								BIT STRING (SIZE (32))

}

Short-NG-5G-S-TMSI ::=						BIT STRING(SIZE(min5G-S-TMSI-size..max5G-S-TMSI-size))

-- TAG-NG-5G-S-TMSI-STOP

-- ASN1STOP

- Option 2: The network can page UEs using paging record list(s) corresponding to full UE-ID (pagingRecordList) and/or truncated UE-ID (pagingRecordList2).

#### –	Paging

The Paging message is used for the notification of one or more UEs.

Signalling radio bearer: N/A

RLC-SAP: TM

Logical channel: PCCH

Direction: Network to UE

Paging message

-- ASN1START

-- TAG-PAGING-START

Paging ::= 							SEQUENCE {

pagingRecordList						PagingRecordList														OPTIONAL, -- Need N

pagingRecordList						PagingRecordList2														OPTIONAL, -- Need N

systemInfoModification					ENUMERATED {true}														OPTIONAL, -- Need N

etwsAndCmasIndication					ENUMERATED {true}														OPTIONAL, -- Need N

lateNonCriticalExtension				OCTET STRING															OPTIONAL,

nonCriticalExtension					SEQUENCE{}																OPTIONAL

}

PagingRecordList ::=				SEQUENCE(SIZE(1..maxNrofPageRec)) OF PagingRecord

PagingRecordList2 ::=				SEQUENCE(SIZE(1..maxNrofPageRec)) OF PagingRecord2

PagingRecord ::=					SEQUENCE{

ue-Identity							PagingUE-Identity,

...

}

PagingRecord2 ::=					SEQUENCE{

ue-Identity							TruncatedUE-Identity,

...

}

PagingUE-Identity ::=				CHOICE{

ng-5g-s-tmsi						NG-5G-S-TMSI,

i-rnti								I-RNTI-Value,

...

}

TruncatedUE-Identity ::=			CHOICE{

short-ng-5g-s-tmsi					Short-NG-5G-S-TMSI,

short-i-rnti						Short-I-RNTI-Value,

...

}

-- TAG-PAGING-STOP

-- ASN1STOP

- Option 3: The network pages UEs using a single paging record list (pagingRecordList) with UE ID (PagingUE-Identity) corresponding either to full UE-ID (fullIdentity) or truncated UE-ID (shortIdentity).

#### –	Paging

The Paging message is used for the notification of one or more UEs.

Signalling radio bearer: N/A

RLC-SAP: TM

Logical channel: PCCH

Direction: Network to UE

Paging message

-- ASN1START

-- TAG-PAGING-START

Paging ::= 							SEQUENCE {

pagingRecordList						PagingRecordList														OPTIONAL, -- Need M

systemInfoModification					ENUMERATED {true}														OPTIONAL, -- Need N

etwsAndCmasIndication					ENUMERATED {true}														OPTIONAL, -- Need N

lateNonCriticalExtension				OCTET STRING															OPTIONAL,

nonCriticalExtension					SEQUENCE{}																OPTIONAL

}

PagingRecordList ::=				SEQUENCE(SIZE(1..maxNrofPageRec)) OF PagingRecord

PagingRecord ::=					SEQUENCE{

ue-Identity							CHOICE {

fullIdentity						PagingUE-Identity,

shortIdentity						TruncatedUE-Identity

}

...

}

PagingUE-Identity ::=				CHOICE{

ng-5g-s-tmsi						NG-5G-S-TMSI,

i-rnti								I-RNTI-Value,

...

}

TruncatedUE-Identity ::=			CHOICE{

short-ng-5g-s-tmsi					Short-NG-5G-S-TMSI,

short-i-RNTI						Short-I-RNTI-Value,

...

}

-- TAG-PAGING-STOP

-- ASN1STOP

- Option 4: The network pages UEs using a single paging record list (pagingRecordList) with UE ID (PagingUE-Identity) corresponding either to full/truncated S-TMSI or I-RNTI.

#### –	Paging

The Paging message is used for the notification of one or more UEs.

Signalling radio bearer: N/A

RLC-SAP: TM

Logical channel: PCCH

Direction: Network to UE

Paging message

-- ASN1START

-- TAG-PAGING-START

Paging ::= 							SEQUENCE {

pagingRecordList						PagingRecordList														OPTIONAL, -- Need M

systemInfoModification					ENUMERATED {true}														OPTIONAL, -- Need N

etwsAndCmasIndication					ENUMERATED {true}														OPTIONAL, -- Need N

lateNonCriticalExtension				OCTET STRING															OPTIONAL,

nonCriticalExtension					SEQUENCE{}																OPTIONAL

}

PagingRecordList ::=				SEQUENCE(SIZE(1..maxNrofPageRec)) OF PagingRecord

PagingRecord ::=					SEQUENCE{

ue-Identity							PagingUE-Identity,

...

}

PagingUE-Identity ::=				CHOICE{

ng-5g-s-tmsi						NG-5G-S-TMSI,

i-rnti								I-RNTI-Value,

short-ng-5g-s-tmsi					Short-NG-5G-S-TMSI,

short-i-RNTI						Short-I-RNTI-Value,

...

}

-- TAG-PAGING-STOP

-- ASN1STOP

# References

- R2-1805244, Definition of paging indication for response-driven paging, Nokia, Nokia Shanghai Bell, Interdigital, Meeting #101bis, Sanya, China, 16th – 20th April 2018.

- Rx-1805245, Truncated UE-ID with configurable length for response-driven paging, Nokia, Nokia Shanghai Bell, Qualcomm Inc., Interdigital, Meeting #101bis, Sanya, China, 16th – 20th April 2018.