3GPP TSG-RAN WG2 Meeting #102								R2-1807483

Busan, KOERA, 21 - 25 May 2018

Agenda item:	9.9.2

Source: 	Huawei, HiSilicon

Title: 	Discussion on new MAC CE for state transition

Document for:	Discussion and decision

1	Introduction

In RAN2#100 meeting, some agreements related to new SCell fast activation state were reached as follows [1].

Agreements:

Introduce a New SCell fast activation state as follows:

1) Not introduce L1 signalling

2) Only period CQI report based on CRS

3) without PDCCH monitoring

Agreements:

1	Transition between legacy SCell deactivated state and fast activation state is via MAC-CE (i.e., similar to legacy).

2	Legacy state transition mechanisms are applicable for transition between legacy SCell activated and SCell deactivated states.

And in RAN2#101 meeting, further discussion continued and more details were made clear [2].

Agreements:

1	Use “Dormant SCell state” as the name of the new Scell state and use ”Hibernate” as the action for moving from Activated SCell state to Dormant Scell state if necessary.

2	Dormant Scell follows PCell DRX for CQI/RRM measurement report triggering.

3	Additional new MAC CE is needed for state transition in/out of the dormant state. Legacy MAC CE is still used as in Rel-10 CA.

4	Define a MAC CE for state transition from activated SCell state to the dormant SCell state.

5    It is supported that UE transits from dormant state to deactivate state.

In RAN2#101bis meeting, some even further agreement has been made [3]:

Agreements:

1: Introduce 2 new MAC CEs to be used with dormant state.

=> Proponents of 1-bit MAC CE and 2-bit MAC CE shall bring contributions with full TPs highlighting how the proposals work to RAN2#102, at which time one approach is selected.4	Define a MAC CE for state transition from activated SCell state to the dormant SCell state.

In this document, we continue to discuss the detail of payload form for new MAC CE.

2	Discussion

When the dormant state is introduced, there are three states for a SCell. How to set the SCell state accurately is a problem. Two new MAC CE has been agreed for the three states, one for seven serving cells and the other one for 31 serving cells. But the agreement about payload design has not been achieved. There are three options to contain three SCell states in one MAC CE.

Option 1: 1 bit for either dormant or activated state [4]

The new MAC CEs only affects the state of SCells in either dormant or activated state. The state of deactivated SCells are not affected by the new MAC CEs. The existing MAC CEs for Scell deactivation can also deactivate Scells in dormant state. They cannot activate SCells in dormant state.

Option 2: two bits are used to indicate one SCell state [5]

There is only one bit for SCell state in current activation/deactivation MAC CE, bit 1 represents actiation state and bit 0 represents deactivation state. When the number of states increases, it is natrual to increase bits used too. So two bits can represent three states, e.g. bits ”00” represent activation state, bits”01” represent deactivation states, bits “10” represent dormant state. The payload structure is illustrated in Figure 1.

Figure 1 MAC control element of two octets

Option 3: flexible payload structure

Based on the payload form of current activation/deactivation MAC CE, the Reserved bit can be changed to Extension bit to indicate if the second byte is available to accommodate three SCell state, and a state transition diagram of the  state transition process as shown in Figure 2.

Figure 2 state transitions based on new MAC CE with flexible payload size

As shown in the figure 3 and figure 4, if bit E equals to ‘0’, there will be only one byte for payload, and this Ci field indicates the activation/deactivation status of the SCell with SCellIndex i as usual. Optionally this Ci field indicates activation state and the dormant state of the SCell with SCellIndex I, which can enable a simple transition between activation state and the dormant state.

If bit E equals to ‘1’, there will be two bytes for payload, this (Ci,0, Ci,1) field indicates the activation/ dormant /deactivation status of the SCell with SCellIndex i. The (Ci,0, Ci,1) field is set to (1, 1) to indicate that the SCell with SCellIndex i shall be activated. The (Ci,0, Ci,1) field is set to (1, 0) to indicate that the SCell with SCellIndex i shall be in dormant state. The (Ci,0, Ci,1) field is set to (0, 0) to indicate that the SCell with SCellIndex i shall be deactivated.  And UE can ignore the (Ci,0, Ci,1) field with value of (0, 1). The same extension mechanism can also be applied to Activation/Deactivation MAC control element of four octets.

Figure 3 Activation/Deactivation MAC control element of one octet

Figure 4 Activation/Deactivation MAC control element of two octets

Based on option 1, the new MAC CE has to cooperate with existing MAC CE to provide a clear indication, it seems unnecessarily complicated to use two MAC CEs for state change. And for option 2 although a larger payload overhead can be foreseen with more consumption of system resource, it is clear and simple to indicate a SCell state. Since a flexible payload design has not been agreed to be discussed further, we have the following proposal:

Proposal: 2-bit MAC CE should be introduced to accommodate three SCell state, as shown in the TP [6] .

3	Conclusions

In this document, we continue to discuss about the payload form for newly designed MAC CE, and we have the following proposal:

Proposal: 2-bit MAC CE should be introduced to accommodate three SCell state.

4	References

- Chairman notes, 3GPP TSG-RAN WG2 Meeting #100, Reno, Nevada, 27 November- 1 December 2017

- Chairman notes, 3GPP TSG-RAN WG2 Meeting #101, Athens, Greece, 26 Feb- 2 Mar 2018

- Chairman notes, 3GPP TSG-RAN WG2 Meeting #101b, Sanya, CHINA, 16 Apr- 20 Apr 2018

- R2-1804768, Details of Dormant Scell state, Nokia, Nokia Shanghai Bell

- R2-1804680, MAC CE for the new Scell state, vivo

- R2-1807484, TP of new state transition MAC CE for TS 36.321