3GPP TSG-RAN WG2 #102                                                                          R2-1807891

Busan, Korea, 21 – 25 May, 2018

| CR-Form-v11.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 36.321 | CR | 1284 | rev |  | Current version: | 14.6.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | X | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Flush HARQ buffer upon skipping a UL transmission |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Google Inc., Nokia, Nokia Shanghai Bell, LG Electronics Inc., Lenovo, HTC, Panasonic, Kyocera, CHTTL |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R2 |  |  |  |  |  |  |  |  |  |
| Work item code: | LTE_LATRED_L2-Core, TEI14 |  |  |  |  |  |  | Date: |  | 2018-05-04 |
| Category: | F |  |  |  |  |  |  | Release: |  | Rel-14 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) Rel-12 (Release 12) Rel-13 (Release 13) Rel-14 (Release 14) Rel-15 (Release 15) Rel-16 (Release 16) |  |
| Reason for change: |  | If a UE is assigned an uplink grant but the HARQ entity cannot obtain a MAC PDU to send, the UE skips the uplink transmission. Afterward, if the UE performs non-adaptive retransmission in a bundle or receives a second grant requesting a retransmission in unlicensed bands, if the corresponding HARQ buffer is not empty, the UE will transmit the data in HARQ buffer. However, the UE should not transmit the data in HARQ buffer. |  |  |  |  |  |  |  |  |
| Summary of change: |  | If a UE skips an uplink transmission, the UE flushes HARQ buffer for the corresponding HARQ process. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | The UE might mistakenly use a grant to retransmit a previous stored TB after UL skipping. Impact analysis: Impacted functionality: HARQ entity Interoperability: If the network is implemented according to this CR while the UE is not, the UE might mistakenly use a grant to retransmit a previous stored TB after UL skipping. If the UE is implemented according to this CR while the network is not, there is no inter-operability issue. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 5.4.2.1 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  | TS/TR ... CR ... |  |  |  |
| affected: |  |  | X | Test specifications |  |  | TS/TR ... CR ... |  |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  | TS/TR ... CR ... |  |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |

#### 5.4.2.1	HARQ entity

There is one HARQ entity at the MAC entity for each Serving Cell with configured uplink, which maintains a number of parallel HARQ processes allowing transmissions to take place continuously while waiting for the HARQ feedback on the successful or unsuccessful reception of previous transmissions.

The number of parallel HARQ processes per HARQ entity is specified in [2], clause 8. NB-IoT has one or two UL HARQ processes.

When the physical layer is configured for uplink spatial multiplexing [2], there are two HARQ processes associated with a given TTI. Otherwise there is one HARQ process associated with a given TTI.

At a given TTI, if an uplink grant is indicated for the TTI, the HARQ entity identifies the HARQ process(es) for which a transmission should take place. It also routes the received HARQ feedback (ACK/NACK information), MCS and resource, relayed by the physical layer, to the appropriate HARQ process(es).

In asynchronous HARQ operation, a HARQ process is associated with a TTI based on the received UL grant except for UL grant in RAR. Except for NB-IoT UE configured with a single HARQ process, each asynchronous HARQ process is associated with a HARQ process identifier. For UL transmission with UL grant in RAR, HARQ process identifier 0 is used. HARQ feedback is not applicable for asynchronous UL HARQ.

When TTI bundling is configured, the parameter TTI_BUNDLE_SIZE provides the number of TTIs of a TTI bundle. TTI bundling operation relies on the HARQ entity for invoking the same HARQ process for each transmission that is part of the same bundle. Within a bundle HARQ retransmissions are non-adaptive and triggered without waiting for feedback from previous transmissions according to TTI_BUNDLE_SIZE. The HARQ feedback of a bundle is only received for the last TTI of the bundle (i.e the TTI corresponding to TTI_BUNDLE_SIZE), regardless of whether a transmission in that TTI takes place or not (e.g. when a measurement gap occurs). A retransmission of a TTI bundle is also a TTI bundle. TTI bundling is not supported when the MAC entity is configured with one or more SCells with configured uplink.

Uplink HARQ operation is asynchronous for NB-IoT UEs, BL UEs or UEs in enhanced coverage except for the repetitions within a bundle, and in serving cells configured with pusch-EnhancementsConfig, and serving cells operating according to Frame Structure Type 3.

For serving cells configured with pusch-EnhancementsConfig, NB-IoT UEs, BL UEs or UEs in enhanced coverage, the parameter UL_REPETITION_NUMBER provides the number of transmission repetitions within a bundle. For each bundle, UL_REPETITION_NUMBER is set to a value provided by lower layers. Bundling operation relies on the HARQ entity for invoking the same HARQ process for each transmission that is part of the same bundle. Within a bundle HARQ retransmissions are non-adaptive and are triggered without waiting for feedback from previous transmissions according to UL_REPETITION_NUMBER. An uplink grant corresponding to a new transmission or a retransmission of the bundle is only received after the last repetition of the bundle. A retransmission of a bundle is also a bundle.

TTI bundling is not supported for RN communication with the E-UTRAN in combination with an RN subframe configuration.

For transmission of Msg3 during Random Access (see subclause 5.1.5) TTI bundling does not apply. For NB-IoT UEs, BL UEs or UEs in enhanced coverage, uplink repetition bundling is used for transmission of Msg3.

For each TTI, the HARQ entity shall:

-	identify the HARQ process(es) associated with this TTI, and for each identified HARQ process:

-	if an uplink grant has been indicated for this process and this TTI:

-	if the received grant was not addressed to a Temporary C-RNTI on PDCCH and if the NDI provided in the associated HARQ information has been toggled compared to the value in the previous transmission of this HARQ process; or

-	if the uplink grant was received on PDCCH for the C-RNTI and the HARQ buffer of the identified process is empty; or

-	if the uplink grant was received in a Random Access Response:

-	if there is a MAC PDU in the Msg3 buffer and the uplink grant was received in a Random Access Response:

-	obtain the MAC PDU to transmit from the Msg3 buffer.

-	else if the MAC entity is configured with semiPersistSchedIntervalUL shorter than 10 subframes and if the uplink grant is a configured grant, and if the HARQ buffer of the identified HARQ process is not empty, and if HARQ_FEEDBACK of the identified HARQ process is NACK; or if the MAC entity is configured with ul-SchedInterval shorter than 10 subframes and if the uplink grant is a preallocated uplink grant, and if the HARQ buffer of the identified HARQ process is not empty, and if HARQ_FEEDBACK of the identified HARQ process is NACK:

-	instruct the identified HARQ process to generate a non-adaptive retransmission.

-	else:

-	obtain the MAC PDU to transmit from the "Multiplexing and assembly" entity, if any;

-	if a MAC PDU to transmit has been obtained:

-	deliver the MAC PDU and the uplink grant and the HARQ information to the identified HARQ process;

-	instruct the identified HARQ process to trigger a new transmission.

-	else:

-	flush the HARQ buffer of the identified HARQ process.

-	else:

-	if the MAC entity is configured with skipUplinkTxSPS and if the uplink grant received on PDCCH was addressed to the Semi-Persistent Scheduling C-RNTI and if the HARQ buffer of the identified process is empty:

-	ignore the uplink grant;

-	else:

-	deliver the uplink grant and the HARQ information (redundancy version) to the identified HARQ process;

-	instruct the identified HARQ process to generate an adaptive retransmission.

-	else, if the HARQ buffer of this HARQ process is not empty:

-	instruct the identified HARQ process to generate a non-adaptive retransmission.

When determining if NDI has been toggled compared to the value in the previous transmission the MAC entity shall ignore NDI received in all uplink grants on PDCCH for its Temporary C-RNTI.