3GPP TSG-RAN WG2 Meeting #103	R2-1811176

# Gothenburg, Sweden, 20th - 24th August 2018

| CR-Form-v11.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.304 | CR | 0026 | rev | - | Current version: | 15.0.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | X | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Corrections about Any Cell Selection and Camped on Any Cell for UE in RRC Inactive |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | MediaTek Inc. |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R2 |  |  |  |  |  |  |  |  |  |
| Work item code: | NR_newRAT-Core |  |  |  |  |  |  | Date: |  | 2018-08-09 |
| Category: | F |  |  |  |  |  |  | Release: |  | Rel-15 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) Rel-12 (Release 12) Rel-13 (Release 13) Rel-14 (Release 14) Rel-15 (Release 15) Rel-16 (Release 16) |  |
| Reason for change: |  | In RAN2#AH1807 meeting, we agreed that ‘Any Cell Selection’ and ‘Camped on Any Cell’ are to be supported for UE in RRC_INACTIVE. |  |  |  |  |  |  |  |  |
| Summary of change: |  | <1> Modify the state transition diagram in 5.2.2 States and state transitions in RRC_IDLE state and RRC_INACTIVE state. <2> Add “RRC_INACTIVE” in 5.2.7 Any Cell Selection state and 5.2.8 Camped on Any Cell state. Impact analysis Impacted functionality: States and state transitions in RRC_IDLE state and RRC_INACTIVE state; Any Cell Selection state; Camped on Any Cell state Inter-operability: If the network is implemented according to this CR while the UE is not, there is no inter-operability issue. If the UE is implemented according to this CR while the network is not, there is no inter-operability issue. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Any Cell Selection and Camped on Any Cell are not supported for UE in RRC_INACTIVE. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 5.2.2, 5.2.7, 5.2.8 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  | TS/TR ... CR ... |  |  |  |
| affected: |  |  | X | Test specifications |  |  | TS/TR ... CR ... |  |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  | TS/TR ... CR ... |  |  |  |
| Other comments: |  | If the network is implemented according to this CR while the UE is not, network may think the UE is in RRC_INACTIVE while the UE enters RRC_IDLE (and camped on any cell). But this is not considered as inter-operability issue. |  |  |  |  |  |  |  |  |

[Start of Change]

5.2.2	States and state transitions in RRC_IDLE state and RRC_INACTIVE state

Figure 5.2.2-1 shows the states and state transitions and procedures in RRC_IDLE and RRC_INACTIVE. Whenever a new PLMN selection is performed, it causes an exit to number 1.

Figure 5.2.2-1 RRC_IDLE and RRC_INACTIVE Cell Selection and Reselection

[Next Change]

5.2.7	Any Cell Selection state

This state is only applicable for RRC_IDLE and RRC_INACTIVE mode. In this state, the UE shall attempt to find an acceptable cell of any PLMN to camp on, trying all RATs that are supported by the UE and searching first for a high-quality cell, as defined in subclause 5.1.1.2.

The UE, which is not camped on any cell, shall stay in this state until an acceptable cell is found.

5.2.8	Camped on Any Cell state

This state is only applicable for RRC_IDLE and RRC_INACTIVE mode. In this state, the UE shall perform the following tasks:

-	select and monitor the indicated paging channels of the cell as specified in clause 7;

-	monitor relevant System Information as specified in [3];

-	perform necessary measurements for the cell reselection evaluation procedure;

-	execute the cell reselection evaluation process on the following occasions/triggers:

1)	UE internal triggers, so as to meet performance as specified in [8];

2)	When information on the BCCH used for the cell reselection evaluation procedure has been modified.

-	regularly attempt to find a suitable cell trying all frequencies of all RATs that are supported by the UE. If a suitable cell is found, UE shall move to camped normally state.

[End of Change]