3GPP TSG-RAN WG2 Meeting #103                                                           	R2-1811228

Gothenburg, Sweden, 20th - 24th August 2018

Source:	CATT

Title:	Handling of UE for the camping cell with unsupported bandwidth

Agenda Item:	10.4.1.3.3

Document for:	Discussion and Decision

Introduction

In RAN2#AH 1807 meeting, agreements about BWP have been made as following:

Agreements

1:	Support configuring dedicated BWP (via either option 1 or option 2) in Msg4 for all cases (initial access, resume and re-establishment). This is applicable to PCells only.

2:	In all cases, UE does BWP switching immediately upon acting on the RRC (re)configuration. This is applicable to SpCells only.

3	Support switching SpCell BWP or reconfiguring SpCell BWP without RRC reconfiguration with sync. PSCell addition or change still requires reconfig with sync.

=>	CR to introduce these agreements can be submitted to the next meeting

=>	Discuss in RAN2#103 how camping in cells where the cell bandwidth is not supported by the UE should be handled.

This contribution will discuss how camping in cells where the cell bandwidth is not supported by the UE should be handled.

Discussion

Solution

For the case that UE can’t support the bandwidth of the CORESET indicated by MIB, the UE will consider the cell or the frequency of the cell is barred. If the UE can support the bandwidth of the CORESET indicated by MIB, but can’t support the bandwidth of the initial BWP, four options will be put forward:

Option 1: Consider the cell barred

As the UE can’t support the bandwidth of the camping cell, the UE can consider the cell is barred. It is easy to handle, but this may cause interference if the UE reselect to other intra-freq cell.

Option 2: Consider the frequency of the cell barred

As the reselected cell is the best rank cell, so if the UE reselect to other intra-cell, high interference will be introduced, so the UE can consider the frequency of the cell is barred. This option may restrict the choice of the UE, as the bandwidth is cell-specific.

Option 3: NW schedule the UE in the narrow bandwidth until the reception of the UE capability

Due to the UE can support the bandwidth of the SIB1 CORESET indicated by MIB and the NW doesn’t know the UE supported bandwidth, so during the access procedure, the NW schedule the UE in the bandwidth of the SIB1 CORESET. The UE monitor the physical channel according to its capability, but it can only receive schedule data in the bandwidth of the SIB1 CORESET. After the NW get the UE capability, the NW can schedule the UE in the UE supported bandwidth, and the UE will receive schedule data in it supported bandwidth.

This option will allow both UE that support the cell bandwidth and that can’t support the cell bandwidth to access the cell, and be served on the cell, but it will impact on the flexibility of the schedule of the NW.

Option 4: Define 64 bits RRCSetupRequest message containing UE supported bandwidth

For  this cell that the bandwidth of the initial BWP is larger than the bandwidth of the SIB1 CORESET, the cell should allow UE access the cell by 64 bits RRCSetupRequest message which containing the supported bandwidth of the UE in the access band.

This option can allow the NW to configure the UE with its supported bandwidth, but it will impact on the standard specified.

Comparison

| OPTIONS | Cons | Pros |
| --- | --- | --- |
| Consider the cell barred | Interference on intra-freq cell | Easy Less impact on standard |
| Consider the frequency of the cell barred | restrict the UE choice | Easy Less impact on standard |
| NW schedule the UE in the narrow bandwidth until the reception of the UE capability | impact the flexibility of the NW schedule | Less impact on standard |
| Define 64 bits RRCSetupRequest message containing UE supported bandwidth | impact on standard | Allow all kinds UE access |

Proposal 1: RAN2 should discuss the four solutions to specify the handling of UE camping in cells where the cell bandwidth is not supported:

- Option 1: Consider the cell barred

- Option 2: Consider the frequency of the cell barred

- Option 3: NW schedule the UE in the narrow bandwidth until the reception of the UE capability

- Option 4: Define 64 bits RRCSetupRequest message containing UE supported bandwidth

Proposals

In this contribution, we propose:

Proposal 1: RAN2 should discuss the four solutions to specify the handling of UE camping in cells where the cell bandwidth is not supported:

- Option 1: Consider the cell barred

- Option 2: Consider the frequency of the cell barred

- Option 3: NW schedule the UE in the narrow bandwidth until the reception of the UE capability

- Option 4: Define 64 bits RRCSetupRequest message containing UE supported bandwidth

Reference

[1] RAN2-NR-AH1807-Montreal-Chair-Notes-2018-07-06-eom