3GPP TSG-RAN WG2 Meeting #103	R2-1811438

# Gothenburg, Sweden, 20 - 24 August 2018

| CR-Form-v11.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.300 | CR | 0046 | rev | 1 | Current version: | 15.2.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | x | Radio Access Network | x | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | QoS Handling Corrections |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Rapporteur (Nokia), Intel Corporation, Nokia Shanghai Bell |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R2 |  |  |  |  |  |  |  |  |  |
| Work item code: | NR_newRAT-Core |  |  |  |  |  |  | Date: |  | 2018-08 |
| Category: | F |  |  |  |  |  |  | Release: |  | Rel-15 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) Rel-12 (Release 12) Rel-13 (Release 13) Rel-14 (Release 14) Rel-15 (Release 15) Rel-16 (Release 16) |  |
| Reason for change: |  | The Informative Annex on QoS Handling contains a number of errors: - The terminology used is not aligned with the Stage 3 terminology agreed in SA2 [23.502], RAN2 [38.331] and RAN3 [38.413] [38.423]; - PDU session establishment is wrongly depicted as being network initiated (it can only be UE initiated, possibly following paging). - Terminology across the different subclauses is not consistent. - Default DRB is wrongly described as mandatory. - From a RAN Stage 2 viewpoint, only AS RQoS is intersting, not NAS RQoS. Furthermore, NAS RQoS would need to be more detailed to be accurate. - RQoS is described as not requiring signalling although it does require some in the SDAP header. - It is not clear why a gNB would know about the QoS characteristics of a new QoS flow only when the new QoS flow appears. - In A.5 the description impies that removing a QoS flow and the corresponding mapping rule can be done implictely in AS. |  |  |  |  |  |  |  |  |
| Summary of change: |  | The following changes are incorporated: - Terminology is aligned with Stage 3 specifications. - PDU session establishment is depicted as being UE initiated. - Terminology is aligned. - Default DRB is made optional. - Details about NAS RQoS are removed. - RQoS terminology is used in subclause title. - It is clarified that QoS characteristics are known at PDU session establishment. - The incorrect sentence is removed. Impact analysis Impacted functionality: Radio Bearer Configuration and Reflective QoS. Inter-operability: - None foreseen as the corresponding requirements are already captured in Stage 3 specifications. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Annex A is not aligned with Stage 3 and contains a number of innacuracies and errors. Stage 2 file size is larger than necessary. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 2, Annex A |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | x | Other core specifications |  |  |  |  |  |  |
| affected: |  |  | x | Test specifications |  |  |  |  |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  |  |  |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |

First Modified Subclause

# 2	References

The following documents contain provisions which, through reference in this text, constitute provisions of the present document.

-	References are either specific (identified by date of publication, edition number, version number, etc.) or non-specific.

-	For a specific reference, subsequent revisions do not apply.

-	For a non-specific reference, the latest version applies. In the case of a reference to a 3GPP document (including a GSM document), a non-specific reference implicitly refers to the latest version of that document in the same Release as the present document.

[1]	3GPP TR 21.905: "Vocabulary for 3GPP Specifications".

[2]	3GPP TS 36.300: "Evolved Universal Terrestrial Radio Access (E-UTRA) and Evolved Universal Terrestrial Radio Access Network (E-UTRAN); Overall description; Stage 2".

[3]	3GPP TS 23.501: "System Architecture for the 5G System; Stage 2".

[4]	3GPP TS 38.401: "NG-RAN; Architecture description".

[5]	3GPP TS 33.501: "Security Architecture and Procedures for 5G System".

[6]	3GPP TS 38.321: "NR; Medium Access Control (MAC) protocol specification".

[7]	3GPP TS 38.322: "NR; Radio Link Control (RLC) protocol specification".

[8]	3GPP TS 38.323: "NR; Packet Data Convergence Protocol (PDCP) specification".

[9]	3GPP TS 37.324: "NR; Service Data Protocol (SDAP) specification".

[10]	3GPP TS 38.304: "NR; User Equipment (UE) procedures in idle mode".

[11]	3GPP TS 38.306: "NR; User Equipment (UE) radio access capabilities".

[12]	3GPP TS 38.331: "NR; Radio Resource Control (RRC); Protocol specification".

[13]	3GPP TS 38.133: "NR; Requirements for support of radio resource management".

[14]	3GPP TS 22.168: "Earthquake and Tsunami Warning System (ETWS) requirements; Stage 1".

[15]	3GPP TS 22.268: "Public Warning System (PWS) Requirements".

[16]	3GPP TS 38.410: "NG-RAN; NG general aspects and principles".

[17]	3GPP TS 38.420: "NG-RAN; Xn general aspects and principles".

[18]	3GPP TS 38.101: "NR; User Equipment (UE) radio transmission and reception".

[19]	3GPP TS 22.261: "Service requirements for next generation new services and markets".

[20]	3GPP TS 38.202: "NR; Physical layer services provided by the physical layer"

[21]	3GPP TS 37.340: "NR; Multi-connectivity; Overall description; Stage-2".

[22]	3GPP TS 23.502: "Procedures for the 5G System; Stage 2".

[23]	IETF RFC 4960 (2007-09): "Stream Control Transmission Protocol".

[24]	3GPP TS 26.114: "Technical Specification Group Services and System Aspects; IP Multimedia Subsystem (IMS); Multimedia Telephony; Media handling and interaction".

[yy]	3GPP TS 38.413: "NG-RAN; NG Application Protocol (NGAP)".

Next Modified Subclause

Annex A (informative):
QoS Handling in RAN

# A.1	PDU Session Establishment

The following figure shows an example message flow for a PDU session establishment. NAS procedures details between gNB and 5GC can be found in 3GPP TS 23.501 [3], TS 23.502 [22] and 3GPP TS 38.413 [yy].

Figure A.1-1: PDU session establishment

1.	UE requests a PDU session establishment to AMF.

2.	AMF sends a PDU SESSION RESOURCE SETUP REQUESTmessage to gNB, which includes the NAS message to be sent to the UE with NAS QoS related information.

3.	gNB sends an RRCReconfiguration message to UE including the configuration of at least one DRB and the NAS message received at Step 2.

4.	UE establishes the DRB(s) for the new PDU session and creates the QFI to DRB mapping rules.

5.	UE sends an RRCReconfigurationComplete message to gNB.

6.	gNB sends a PDU SESSION RESOURCE SETUP RESPONSE message to AMF.

7.	User Plane Data can then be exchanged between UE and gNB over DRB(s) according to the mapping rules and between UPF and gNB over the tunnel for the PDU session. QFI marking over Uu is optional (see subclause 12) while QFI marking over NG-U is always present.

# A.2	New QoS Flow with RQoS

The following figure shows an example message flow when RQoS is used for a new QoS flow. In this example, the gNB receives from UPF a first downlink packet associated with a QFI for which the QoS parameters are known from the PDU session establishment, but for which there is no association to any DRB yet in AS..

Figure A.2-1: DL data with new QFI sent over existing DRB

0.	PDU session and DRB(s) have been already established.

1.	gNB receives a downlink packet with a new QFI from UPF.

2.	gNB decides to send the new QoS flow over an existing DRB.

NOTE:	If gNB decides to send it over a new DRB, it needs to establish the DRB first.

3.	gNB sends the DL packet over the selected DRB with the new QFI and RDI set in the SDAP header.

4.	UE identifies the QFI and RDI in the received DL packet and the DRB on which the packet was received. The AS mapping rules are then updated accordingly.

5.	User Plane Data for the new QoS flow can then be exchanged between UE and gNB over the DRB according to the updated mapping rules and between UPF and gNB over the tunnel for the PDU session.

# A.3	New QoS Flow with Explicit RRC Signalling

The following figure shows an example message flow when explicit RRC signalling is used for a new QoS flow.  In this example, the gNB receives from UPF a first downlink packet associated with a QFI, for which the QoS parameters are already known from the PDU session establishment, but for which there is no association to any DRB yet in AS..

Figure A.3-1: DL data with new QFI sent over existing DRB

0.	PDU session and DRB(s) have been already established.

1.	gNB receives a downlink packet with a new QFI from UPF.

2.	gNB decides to send the new QoS flow over an existing DRB using explicit RRC signalling for updating the AS mapping rules.

3.	gNB sends an RRCReconfiguration message to UE with the new QFI to DRB mapping rule. gNB may also decide to update the DRB configuration if required to meet the QoS requirements for the new QoS Flow.

4.	UE updates the QFI to DRB mapping rules and DRB configuration (if received).

5.	UE sends an RRCReconfigurationComplete message to gNB.

6.	User Plane Data for the new QoS flow can then be exchanged between UE and gNB over the DRB according to the updated mapping rules and between UPF and gNB over the tunnel for the PDU session.

# A.4	New QoS Flow with Explicit NAS Signalling

The following figure shows an example message flow when the gNB receives a new QoS flow establishment request from CN that involves NAS explicit signalling. The QoS flow establishment request provides the gNB and UE with the QoS parameters for the QFI.  In this example, the gNB decides to establish a new DRB (rather than re-use an existing one) for this QoS flow and provides the mapping rule over RRC signalling. NAS procedures details between gNB and 5GC can be found in 3GPP TS 23.501 [3], TS 23.502 [22] and 3GPP TS 38.413 [yy].

Figure A.4-1: DL data with new QoS Flow ID sent over new DRB with explicit signalling

0.	PDU session DRB(s) have been already established.

1.	gNB receives a PDU SESSION RESOURCE MODIFY REQUEST messagefrom AMF for a new QoS flow.

2.	If gNB cannot find an existing DRB to map this new QoS flow, it decides to establish a new DRB.

3.	gNB sends an RRCReconfiguration message to UE including the DRB configuration with the new QFI to DRB mapping rule and the NAS message received at step 1.

4.	UE establishes the DRB for the new QoS flow associated with this PDU session and updates the mapping rules.

5.	UE sends an RRCReconfigurationComplete message to gNB.

6.	gNB sends a PDU SESSION RESOURCE MODIFY RESPONSE message to AMF.

7.	User Plane Data can then be exchanged between UE and gNB over DRB(s) according to the mapping rules and between UPF and gNB over the tunnel for the PDU session.

# A.5	Release of QoS Flow with Explicit Signalling

The following figure shows an example message flow when the gNB receives a request to release a QoS flow from CN that involves explicit NAS signalling.  NAS procedures details between gNB and 5GC can be found in 3GPP TS 23.501 [3], TS 23.502 [22] and 3GPP TS 38.413 [yy].

Figure A.5-1: Release of QoS Flow with Explicit Signalling

0.	PDU session and DRB(s) have been already established.

1.	gNB receives a PDU SESSION RESOURCE MODIFY REQUEST message from AMF to release a QoS flow.

2.	The gNB decides to release the corresponding QFI to DRB mapping rule.  Since the DRB also carries other QoS flows, the DRB is not released.

3.	gNB sends an RRCReconfiguration message to UE to release the QFI to DRB mapping rule.

4.	UE updates the AS QFI to DRB mapping rules to release this QFI to DRB mapping rule.

5.	UE sends an RRCReconfigurationComplete message to gNB.

6.	gNB sends a PDU SESSION RESOURCE MODIFY RESPONSE message to AMF.

# A.6	UE Initiated UL QoS Flow

The following figure shows an example message flow when the UE AS receives an UL packet for a new QoS flow for which a QFI for DRB mapping rule does not exist.

Figure A.6-1: UL packet with a new QoS flow for which a mapping does not exist in UE

0.	PDU session and DRBs (including a default DRB) have been already established.

1.	UE AS receives a packet with a new QFI from UE NAS.

2.	UE uses the QFI of the packet to map it to a DRB. If there is no mapping of the QFI to a DRB in the AS mapping rules for this PDU session, then the packet is assigned to the default DRB.

3.	UE sends the UL packet on the default DRB. The UE includes the QFI in the SDAP header.

4.	gNB sends UL packets to UPF and includes the corresponding QFI.

5.	If gNB wants to use a new DRB for this QoS flow, it sets up one. It can also choose to move the QoS flow to an existing DRB using RQoS or RRC signalling (see subclauses A.2 and A.3).

6.	User Plane Data for the new QoS flow can then be exchanged between UE and gNB over the DRB according to the updated mapping rules and between UPF and gNB over the tunnel for the PDU session.