3GPP TSG-RAN WG2 #103	Tdoc R2-1811872

Gothenburg, Sweden, 20th – 24th August 2018

Agenda Item:	10.3.3.4

Source:	Ericsson

Title:	PDCP selective retransmissions

Document for:	Discussion, Decision

# Introduction

In RAN2 AH June 2018, the option of selective PDCP retransmissions at PDCP re-establishment was discussed. In this contribution we analyse advantages and downsides of this option.

# Discussion

In current PDCP specifications, the following behaviour for PDCP retransmissions is outlined:

-	for AM DRBs, from the first PDCP SDU for which the successful delivery of the corresponding PDCP Data PDU has not been confirmed by lower layers, perform retransmission or transmission of all the PDCP SDUs already associated with PDCP SNs in ascending order of the COUNT values associated to the PDCP SDU prior to the PDCP entity re-establishment as specified below:

-	perform header compression of the PDCP SDU as specified in the subclause 5.7.4;

-	perform integrity protection and ciphering of the PDCP SDU using the COUNT value associated with this PDCP SDU as specified in the subclause 5.9 and 5.8;

-	submit the resulting PDCP Data PDU to lower layer, following the data submission procedure in subclause 5.2.1.

Therein, the retransmissions are not “selective”, i.e. transmissions or retransmissions are done for consecutive SDUs of PDUs from the first PDU that had not been confirmed by lower layers, to the last SDU of PDU already assigned an SN.

For single connectivity, gaps in the indications of successful delivery occur depending on the RLC status report format received. I.e. when the RLC ACK SN is moved up, some RLC SNs may still be indicated as NACKed, those would result in gaps of indications of successful delivery to PDCP. In multi-connectivity, gaps may additionally occur due to PDCP PDUs distributed via multiple RLC links.

Example: PDCP SNs 0,2,4 might be indicated as successfully delivered. With the current PDCP behaviour, the PDCP transmission would retransmit 1-4. While with the alternative selective retransmission scheme as outlined in [1], PDCP SN 1 and SN 3 would be retransmitted only.

It is further described in [1], that selective retransmission might be configured for the case where the network does provide UL data forwarding, or where the network does not need to provide it, i.e. where the PDCP entity termination point is not changed. Otherwise, the selectively not retransmitted PDUs by the UE would be lost.

We see only limited gain in such an optimization approach, since first of all, the potential performance gain would be limited to intra-site handover scenarios, i.e. is not continuous or applicable to all handovers, and secondly, given that by RLC status reporting (for which the frequency is configurable), the window of data in flight, i.e. not-acked, can be kept small, the potential of saving redundant retransmissions by selective retransmissions can be considered limited.

PDCP selective retransmissions at re-established are not further pursued in Rel-15.

In the following we further analyse the impact selective retransmission would have on ROHC.

When ROHC is continued after re-establishment, the receiver decompresses after reordering/duplicate discard. This means that independent of whether re-transmissions are selective or not, the receiver considers only those PDUs that were not already received, i.e. does selective reception.

Considering the transmitter side, the performance depends on the detailed ROHC implementation: a simple transmitter would compress retransmissions with latest ROHC context update, i.e. not look up what was the ROHC context for the PDU for which a retransmission is done. This does not break the ROHC decompression in receiver, but due to different (out of order) ROHC context used for the retransmissions, lowers the decompression performance, since decompression is done in order of PDCP SNs, while retransmissions are compressed in different order. The more out of order retransmissions with different ROHC context are received, the lower the ROHC decompression performance can be expected. This is similar with and without selective retransmissions.

When ROHC is reset at re-establishment, the receiver decompresses already received PDUs based on their context. Since there might be transmission gaps in-between, this leads already to a decompression performance loss. After the re-establishment, the receiver decompresses re-transmissions as well as new transmissions based on the new context. It is noted however that thereby, re-transmissions of already received PDUs are discarded. Interestingly in this case, given the simple ROHC transmitter implementation described above, the ROHC performance could expected higher with selective retransmissions where ROHC context is updated for the selective retransmission, since in this case of ROHC context reset in the receiver, the receiver would decompress these selective retransmissions in the same order. Nevertheless, resetting ROHC during re-establishment would certainly lead to an overall lower performance than the positive side-effect of decompressing the selectively retransmitted PDUs in the right order after the re-establishment.

# Conclusion

Based on the discussion in section 2 we propose the following:

Proposal 1	PDCP selective retransmissions at re-established are not further pursued in Rel-15.

# References

- R2-1809890	Selective retransmission on PDCP re-establishment	vivo