3GPP TSG-RAN WG2#103	Tdoc R2-1811875

Gothenburg, Sweden, 20th – 24th August 2018

Agenda Item:	10.3.1.2

Source:	Ericsson

Title:	Use of MCS-C-RNTI in C-RNTI MAC CE

Document for:	Discussion, Decision

# Introduction

In RAN2 AdHoc1807 [1], MCS-C-RNTI with respect to the new MCS table was discussed. There is still an open issue left on whether the MCS-C-RNTI can also be carried in C-RNTI MAC CE. This paper discusses the necessity to carry C-RNTI in the C-RNTI MAC CE.

# Discussion

In RAN1, a new MCS table was defined and a MCS-C-RNTI is used to indicate which MCS table is used for a scheduled data transmission. If the received PDCCH to schedule a transmission is addressed to the C-RNTI, traditional MCS table is used in the scheduled transmission; otherwise, if the received PDCCH to schedule a transmission is addressed to the MCS-C-RNTI, the new MCS table is used. The new MCS table, which has more robust channel coding, is expected to improve the reliability of URLLC data transmission or extend the service coverage. In RAN2 AdHoc1807, it was already agreed that “RAN2 assumes the “new RNTI” is equivalent to C-RNTI w.r.t. MAC functionality”. However, there is still an open issue on whether to carry the MCS-C-RNTI in C-RNTI MAC CE, which will be discussed in the following.

C-RNTI MAC CE is carried in message 3 for contention resolution for a UE during contention based random access procedure if the UE already has valid C-RNTI. The corresponding part from Section 5.1.4 of 3GPP TS 38.321-f20 is attached below for quick reference:

… …

4>	if the Random Access Preamble was not selected by the MAC entity among the contention-based Random Access Preamble(s):

5>	consider the Random Access procedure successfully completed.

4>	else:

5>	set the TEMPORARY_C-RNTI to the value received in the Random Access Response;

5>	if this is the first successfully received Random Access Response within this Random Access procedure:

6>	if the transmission is not being made for the CCCH logical channel:

7>	indicate to the Multiplexing and assembly entity to include a C-RNTI MAC CE in the subsequent uplink transmission.

6>	obtain the MAC PDU to transmit from the Multiplexing and assembly entity and store it in the Msg3 buffer.

C-RNTI MAC CE is used for contention resolution during contention based random access when the UE already has a valid C-RNTI.

For one instance, during the Random Access procedure for radio connection reestablishment upon radio link failure or contention based random access for beam failure recovery, a UE can transmit C-RNTI MAC CE in message 3, while upon the reception of the C-RNTI MAC CE, the gNB can identify which UE triggers the procedure of radio connection reestablishment or beam failure recovery. If the MCS-C-RNTI can also be carried in C-RNTI, it means that the MCS-C-RNTI instead of C-RNTI is used for contention resolution for contention based random access. However, there seems no any difference or additional benefit to use the MCS-C-RNTI rather than C-RNTI for contention resolution as the MCS-C-RNTI and C-RNTI are equal with respect to contention resolution.

There is no additional benefit to use MCS-C-RNTI in C-RNTI MAC CE.

If MCS-C-RNTI can also be carried in C-RNTI, the gNB may need to monitor both MCS-C-RNTI and the C-RNTI in C-RNTI MAC CE, which means increased complexity. In order to reduce the complexity of specification and the implementation in gNB without additional benefit, we propose:

- Only C-RNTI is carried in C-RNTI MAC CE (i.e. not MCS-C-RNTI).

# Conclusion

In section 2 we made the following observations:

Observation 1	C-RNTI MAC CE is used for contention resolution during contention based random access when the UE already has a valid C-RNTI.

Observation 2	There is no additional benefit to use MCS-C-RNTI in C-RNTI MAC CE.

Based on the discussion in section 2 we propose the following:

Proposal 1	Only C-RNTI is carried in C-RNTI MAC CE (i.e. not MCS-C-RNTI).

# References

R2-1810775, Report Breakout session UP, IAB, NR-U, RAN2-AdHoc 1807