3GPP TSG-RAN WG2#103	Tdoc R2-1811878

Gothenburg, Sweden, 20th – 24th August 2018

Agenda Item:	10.3.1.8

Source:	Ericsson

Title:	Configured Scheduling during Random Access Procedure

Document for:	Discussion, Decision

# Introduction

In RAN2-AdHoc1807, configuredGrantTimer operation during random access procedure was discussed but not settled. It was concluded that the HARQ procedure interaction between Msg3 transmission and uplink transmission using a configured grant should be further discussed. This paper discusses configured scheduling behavior with respect to Msg3 transmission.

# Discussion

During random access procedure, HARQ transmission is supported for Msg3. However, in NR, the random access procedure may be triggered for a UE when configured scheduling is activated for the UE. A HARQ process (e.g. HARQ process 0), which is configured for configured scheduling, may be also used for Msg3 transmission.  In the following, we briefly introduce the timing for Msg3 PUSCH transmission, HARQ process assignment scheme based on configuredGrantTimer according to the existing specification (3GPP TS 38.321-f20) and then discuss the options to protect the HARQ procedure of Msg3 transmission in such condition.

## Timing of Msg3 PUSCH

In Section 8.2 of in 3GPP TS 38.213-f20, the entry to identify the time resource allocation for Msg3 PUSCH is indicated using a 4-bit field “Msg3 PUSCH time resource allocation” in the UL grant (see Table 8.2-1 in 3GPP TS 38.213-f20). Further, the UE determines the time resource allocation parameters by looking up the time resource allocation table which is determined according to the following in the same section:

“If higher layer parameter pusch-AllocationList is provided by pusch-ConfigCommon, a value of the Msg3 PUSCH time resource allocation field indicates an entry to a table provided by pusch-AllocationList; otherwise a value of the Msg3 PUSCH time resource allocation field indicates an entry to default time resource allocation table [6, 38.214].”

The IE to configure the PUSCH time resource allocation can be found in 3GPP TS 38.331-f20. The corresponding part is copied below for quick reference:

PUSCH-Config information element

-- ASN1START

-- TAG-PUSCH-CONFIGCOMMON-START

PUSCH-ConfigCommon ::= 					SEQUENCE {

groupHoppingEnabledTransformPrecoding	ENUMERATED {enabled}			OPTIONAL,	-- Need R

pusch-TimeDomainAllocationList	PUSCH-TimeDomainResourceAllocationList	OPTIONAL,	-- Need R

msg3-DeltaPreamble		INTEGER (-1..6)		OPTIONAL,	-- Need R

p0-NominalWithGrant	 	INTEGER (-202..24)	OPTIONAL,	-- Need R

...

}

PUSCH-TimeDomainResourceAllocationList ::= 	SEQUENCE (SIZE(1..maxNrofUL-Allocations)) OF PUSCH-TimeDomainResourceAllocation

PUSCH-TimeDomainResourceAllocation ::= 	SEQUENCE {

k2										INTEGER(0..32)			OPTIONAL,	-- Need S

mappingType								ENUMERATED {typeA, typeB},

startSymbolAndLength					INTEGER (0..127)

}

K2 is interpreted as “Corresponds to L1 parameter 'K2' (see 38.214, section FFS_Section) When the field is absent the UE applies the value 1 when PUSCH SCS is 15/30KHz; 2 when PUSCH SCS is 60KHz and 3 when PUSCH SCS is 120KHz.”

When the PUSCH time allocation table is not explicitly configured via pusch-ConfigCommon. The UE shall rely on the default time allocation table to determine the Msg 3 PUSCH preparation time. The default time allocation in Table 6.1.2.1.1-2 of 3GPP TS 38.214-f20. K2 (i.e. PUSCH preparation time) depends on the j and an additional offset value, wherein j corresponds to the subcarrier spacing for Msg3 PUSCH. The correspondence between subcarrier spacing index µPUSCH and j value can be found in Table 6.1.2.1.1-4. According to the default time allocation table, the maximum K2 value can be up to 6 slots.

Table 6.1.2.1.1-2: Default PUSCH time domain resource allocation A for normal CP

| Row index | PUSCH mapping type | K2 | S | L |
| --- | --- | --- | --- | --- |
| 1 | Type A | j | 0 | 14 |
| 2 | Type A | j | 0 | 12 |
| 3 | Type A | j | 0 | 10 |
| 4 | Type B | j | 2 | 10 |
| 5 | Type B | j | 4 | 10 |
| 6 | Type B | j | 4 | 8 |
| 7 | Type B | j | 4 | 6 |
| 8 | Type A | j+1 | 0 | 14 |
| 9 | Type A | j+1 | 0 | 12 |
| 10 | Type A | j+1 | 0 | 10 |
| 11 | Type A | j+2 | 0 | 14 |
| 12 | Type A | j+2 | 0 | 12 |
| 13 | Type A | j+2 | 0 | 10 |
| 14 | Type B | j | 8 | 6 |
| 15 | Type A | j+3 | 0 | 14 |
| 16 | Type A | j+3 | 0 | 10 |

Table 6.1.2.1.1-4: Definition of value j

| µPUSCH | j |
| --- | --- |
| 0 | 1 |
| 1 | 1 |
| 2 | 2 |
| 3 | 3 |

After reception Msg3 PUSCH signal from a UE, the gNB takes some time to decode Msg3 PUSCH. If a decoding error is determined for Msg3 decoding, the gNB may generate and transmit a retransmission grant for Msg3 upon for the UE. Upon reception of the Msg 3 retransmission grant, the UE needs K2 slots, which is determined as above, to prepare the Msg3 PUSCH retransmission. In such condition, the HARQ round trip time for Msg3, which is the sum of these time budgets, can be quite several slots.

The HARQ round trip time for Msg3 PUSCH can be several slots according to RAN1 specification.

## HARQ operation based on configuredGrantTimer for configured scheduling

When a HARQ process, which is configured to configured scheduling, is used for a HARQ transmission using a configured grant or a dynamic grant, the associated configuredGrantTimer is started with a preconfigured value. After PUSCH transmission using a configured grant using a HARQ process, the UE starts the associated configuredGrantTimer for the HARQ process. When the associated configuredGrantTimer is running, the UE should not use the HARQ process for new data transmission using another configured grant. This means the HARQ process is locked when the associated configuredGrantTimer is running so that gNB can have enough time budget to schedule the HARQ retransmission upon PUSCH decoding error and the UE can have enough time budget to prepare the PUSCH for the HARQ retransmission upon the reception of the retransmission grant.

In order to support services of various QoS requirements, a wide range of candidate initial values for configuredGrantTimer have been defined in 3GPP TS 38.331-f10, which can be found in the IE ConfiguredGrantConfig:

periodicity								ENUMERATED {

sym2, sym7, sym1x14, sym2x14, sym4x14, sym5x14, sym8x14, sym10x14, sym16x14, sym20x14,sym32x14, sym40x14, sym64x14, sym80x14, sym128x14, sym160x14, sym256x14, sym320x14, sym512x14,sym640x14, sym1024x14, sym1280x14, sym2560x14, sym5120x14,

sym6, sym1x12, sym2x12, sym4x12, sym5x12, sym8x12, sym10x12, sym16x12, sym20x12, sym32x12,

sym40x12, sym64x12, sym80x12, sym128x12, sym160x12, sym256x12, sym320x12, sym512x12, sym640x12,

sym1280x12, sym2560x12},

configuredGrantTimer					INTEGER (1..64)

The configuredGrantTimer indicates the initial value of the configured grant timer in number of periodicities. According to the above analysis based on the existing specification, we can observe the following:

The configuredGrantTimer values ranges between 2 OFDM symbols and several thousand of slots.

## Msg 3 transmission and configured scheduling

In RAN2 AdHoc 1807, there were some discussions on whether/how to use the configuredGrantTimer to lock HARQ process 0 for message 3 transmission. Based on the observations in Section 2.1 and 2.2, there are quite a few small candidate initial values (e.g. the shortest initial value can be only 2 OFDM symbols) for configuredGrantTimer.  These small candidate initial values for configuredGrantTimer may be even shorter than the HARQ round trip time of Msg3 transmission.

Even if the configuredGrantTimer is started/restarted upon Msg 3 transmission, it may expire before the retransmission fo Msg3 when a small initial value is configured for configuredGrantTimer. When the associated configuredGrantTimer expires, the UE can perform uplink data transmission using a configured grant corresponding to the HARQ process (i.e. HARQ process 0). This means that the HARQ procedure protection mechanism based on configuredGrantTimer is not enough the protect the HARQ procedure for Msg3, i.e. the HARQ procedure for Msg3 transmission can be corrupted by a PUSCH transmission using a configured grant when a small initial value is configured for configuredGrantTimer, e.g. for URLLC service.  Figure 1 shows one such example.

Figure 1 HARQ procedure of Msg3 PUSCH is broken by PUSCH transmission using a configured grant

When a small initial value is configured for configuredGrantTimer, starting/restarting configuredGrantTimer upon Msg3 PUSCH TX is not enough to protect the Msg3 HARQ procedure.

## Solution discussion

In the following, two types of solution options are discussed. Option 1 requires a UE to skip the configured grants during CBRA procedure, while Option 2 still relies on the configuredGrantTimer.

Option 1: UE skips all configured grants during random access procedure.

A UE should skip all configured grants during CBRA procedure for BFR (e.g. from PRACH or Msg3 PUSCH transmission to message 4 reception) regardless if there is a need to use configured grants for uplink data transmissions or not. Considering that uplink transmission using a configured grant may not be so meaningful due to poor radio condition during beam failure, it is feasible for a UE to skip all uplink grants during CBRA procedure for BFR. If we want to reduce the potential impact, a UE may only skip the configured grants corresponding to HARQ process 0 instead of skipping all configured grants during CBRA based BFR procedure, i.e.

Option 1x: UE skips all configured grants corresponding to HARQ process 0 during random access procedure.

Option 2: A specific large enough configuredGrantTimer value is used during random access procedure

In order to protect the HARQ procedure for Msg3, the gNB can configure a specific configuredGrantTimer value, which can be different from the preconfigured configuredGrantTimer value for configured scheduling, to be used during the random access procedure. The specific configuredGrantTimer value is determined based on the HARQ round trip time with Msg3 retransmission taken into accounted. Upon Msg3 transmission/retransmission, the UE shall start/restart the configuredGrantTimer with the specific value. The specific value may be used only when the configuredGrantTimer value for configured scheduling is too short. Similar as Option 1, UE may only apply the specific value for the configuredGrantTimer associated with HARQ process 0 instead of any configured HARQ process for configured scheduling.

Option 2x: UE applies a preconfigured specific value for configuredGrantTimer associated with HARQ process 0 during CBRA procedure.

Option 1 only requires minor change in the MAC protocol, while Option 2 requires a new RRC parameter to configure the new initial configuredGrantTimer value for Msg3 HARQ transmission in addition to the MAC protocol change, i.e. Option 1 has lower complexity than Option 2. Further, considering that Option 1x requires less configured grant skipping than Option 1, we propose:

UE skips all configured grants corresponding to a HARQ process used for random access procedure.

Based P1 above, a correction has been made.

RAN2 agree on the correction in [1].

# Conclusion

In section 2 we made the following observations:

Observation 1	The HARQ round trip time for Msg3 PUSCH can be several slots according to RAN1 specification.

Observation 2	The configuredGrantTimer values ranges between 2 OFDM symbols and several thousand of slots.

Observation 3	When a small initial value is configured for configuredGrantTimer, starting/restarting configuredGrantTimer upon Msg3 PUSCH TX is not enough to protect the Msg3 HARQ procedure.

Based on the discussion in section 2 we propose the following:

Proposal 1	UE skips all configured grants corresponding to a HARQ process used for random access procedure.

Proposal 2	RAN2 agree on the correction in [1].

# References

R2 – 1811879 Correction on Configured Scheduling during Random Access Procedure