3GPP TSG-RAN WG2#103	Tdoc R2-1811882

Gothenburg, Sweden, 20th – 24th August 2018 	Revision of R2-1810178

Agenda Item:	10.3.2.4

Source:	Ericsson

Title:	RLC behavior after RLC failure in CA duplication

Document for:	Discussion, Decision

# 1	Introduction

With CA duplication, a new issue which did not exist in LTE has been introduced. This is the case when there is an RLC failure due to reaching the maximum number of RLC retransmissions in the Scell. This topic was discussed in [1]. This contribution further analyses the problem.

# 2	Discussion

When there is an RLC failure and in case of CA duplication and SCell-RLF is triggered, duplication should be deactivated [2]. When the UE experience an RLF due to maximum number of RLC retransmissions in a Scell, the UE sends a RRC message to the network and duplication is deactivated. When CA duplication is deactivated, PDCP and RLC will discard the duplicated PDCP PDUs and RLC SDU/PDU(s). However, those RLC PDUs which are undergoing retransmissions cannot be discarded. In addition, the logical channel restrictions are lifted. This results in that the RLC entity mapped to the Scell which experienced a failure will continue retransmissions in the Pcell.

Figure 1 – CA duplication activated

Figure 2 - CA duplication deactivated after RLC failure

When a UE experiences an RLF failure due to the maximum number of retransmissions, the expected behavior is that the UE stops all transmissions in RLC, including retransmissions, i.e. the RLC halts. But, this is not the specified behavior in the current NR RLC specification. According to current specifications the UE will continue retransmitting PDUs beyond the retransmission limit if the UE gets a grant.

In previous discussions, it was claimed that the NW should reconfigure the UE. First of all, this is not mandated. Secondly, even when the network eventually reconfigures the UE (i.e. at least after RRC i indication of failure from UE to network and then RRC reconfiguration from network to UE), the UE might already have done several redundant unwanted retransmissions that could even degrade performance on the PCell, since they are multiplexed with transmissions on the remaining RLC entity (see Figure 2). This can have impact on URLLC data transmission for which CA duplication is used due to time and reliability requirements. Instead, the UE behavior should be to stop all RLC transmissions as stated above. This is the general expected RLC behavior upon this type of RLC failure. Therefore, we propose:

The expected UE behavior when there is an RLC failure due to maximum number of retransmissions reached is that no more RLC SDUs, SDU segments, or PDUs shall be considered for (re-)transmission.

A CR with proposed changes can be found in [3].

# Conclusion

Based on the discussion in the previous sections we propose the following:

Proposal 1	The expected UE behavior when there is an RLC failure due to maximum number of retransmissions reached is that no more RLC SDUs, SDU segments, or PDUs shall be considered for (re-)transmission.

# References

R2-1810178, RLC behavior after RLC failure in CA duplication

R2-1811574, Draft CR to 38.331 on SCell-RLF

R2-1811883, Correction on RLC retransmissions at RLC failure in CA duplication