3GPP TSG-RAN WG2 Meeting #103	R2-1812035

Gothenburg, Sweden, 20 - 24 August 2018

Agenda item:	10.4.4.2

Source:	Nokia, Nokia Shanghai Bell

Title:	On requirements applicable to L2-buffer dimensioning

WID/SID:	NR_newRAT-Core - Release 15

Document for:	Discussion and Decision

# 1	Introduction

On the dimensioning of the UE L2 buffer size, the previous RAN2 meeting noted the following.

=> For EN-DC: X2 delay + Queuing in MN = FFS ms.

This contribution continues on that topic.

# 2	Discussion

As proposed in [2], LTE DC assumes X2 delay + Queuing in SeNB = 130ms. There have been varying views on whether to adopt the same assumption for the LTE eNB in EN-DC. While [3] proposes to do so, [4] claims that the user-plane latency requirements in TR 38.913 [1], of 4ms or lower, become an obstacle.

It seems worth discussing whether the LTE eNB involved in EN-DC is subject to those requirements. We do not think that it is, for the following reasons:

- The 38-series of 3GPP specifications and reports is titled “Radio technology beyond LTE” and therefore does not apply to an LTE eNB;

- The hardware of LTE eNBs already deployed in the field will be used for EN-DC.

For these reasons, we propose the following.

Proposal 1:	RAN2 agree that the latency requirements in TR 38.913 do not apply to LTE eNBs (not even those involved in EN-DC).

RAN2 have already agreed that X2/Xn delay + Queueing in SN = 25ms. We can assume that this reflects a split into a 20ms X2 delay and 5ms queuing delay (where the latter roughly fits within the requirements in [1]). Accordingly, we can use the simulation results copied into Figure 1 below from [2] to arrive at a 100ms total for X2 delay + Queuing in MN (80ms queueing @ 20ms X2 delay).

Proposal 2:	For EN-DC: X2 delay + Queuing in MN = 100ms.

Figure 1: Simulation results of SeNBTXQueueDelay [2]

# 3	Conclusion

Based on the above discussion, we propose the following.

Proposal 1:	RAN2 agree that the latency requirements in TR 38.913 do not apply to LTE eNBs (not even those involved in EN-DC).

Proposal 2:	For EN-DC: X2 delay + Queuing in MN = 100ms.

# References

- 3GPP TR 38.913 Study on Scenarios and Requirements for Next Generation Access Technologies

- R2-144103 Total layer-2 buffer size for dual-connectivity capable UEs - Nokia Networks, Nokia Corporation

- R2-1711618 Total L2 buffer size calculation - NTT DOCOMO

- R2-1712692 L2 buffer size capability - Intel Corporation