3GPP TSG-RAN WG2 #103	R2-1812121

Gothenburg, Sweden, 20th - 24th August 2018

Agenda Item:	9.15.3

Source:	Ericsson

Title:	UE capability related with SPS

Document for:	Discussion, Decision

# 1. Introduction

During LTE HRLLC WI in rel-15, several enhancements related with SPS have been agreed. In this paper, we discuss the UE capability related with these enhancements.

# 2. Discussion

In RAN2#100, it is agreed that

1	SPS configuration can be active simultaneously for Pcell, PSCell and SCell.

In Rel-14, eNB can configure SPS only on PScell and this agreement means that eNB can configure SPS on all serving cells.  A separate UE capability should be added for the support of this functionality.

Introduce a separate UE capability for the support of simultaneously activated SPS configurations on PCell, PSCell and SCell.

In RAN2#102, it is further agreed that

1	Introduce multiple activated UL SPS configurations in one serving cell only for the same TTI length case.

However, this agreement comes late due to a late RAN1 LS [1] on supporting multiple UL SPS configurations. RAN2 has not properly discussed this feature.

In particular, RAN1 has discussed this feature under the assumption that this multiple SPS configurations scheme is used only when there is an UL SPS repetition configured as well, see details in [1]. Also, a separate UE capability in RAN1 is agreed for the support of UL SPS repetitions for subframe, slot and subslot.  If a separate UE capability is introduced for multiple activated UL SPS configuration, then it means that UE can support multiple SPS configuration without UL repetition. It is not clear from the RAN1 LS that this is the intention.

From RAN1 LS, it is not clear whether a separate UE capability for the support of multiple activated UL SPS configurations in one serving cell for the same TTI length case is needed.

CRs for this proposal 1 are in [2,3].

# 3. Conclusion

In the previous sections we made the following observations:

Observation 1	From RAN1 LS, it is not clear whether a separate UE capability for the support of multiple activated UL SPS configurations in one serving cell for the same TTI length case is needed.

Based on the discussion in the previous sections we propose the following:

Proposal 1	Introduce a separate UE capability for the support of simultaneously activated SPS configurations on PCell, PSCell and SCell.

# 4. References

R2-1808982, LS on Ultra Reliable Low Latency Communication for LTE, RAN2#102

R2-1812123, draft RRC CR UE capability related with SPS, RAN2#103

36306_ CR1633_(Rel-15)_R2-1812122, UE capability related with SPS, RAN2#103