3GPP TSG-RAN WG2 #103	R2-1812284

Gothenburg, Sweden, 20th – 24th August 2018	Update of R2-1809619

Agenda Item:	10.4.5.3

Source:	Ericsson

Title:	Increasing the paging capacity without increasing the “false” page detection rate

Document for:	Discussion, Decision

# Introduction

The essential means for configuration of paging frames and paging occasions have been agreed in RAN2 and are covered in TS 38.304. However, the currently agreed methods do not provide a paging capacity on par with LTE in all scenarios and will also result in increased UE energy consumption through increased number of “false” page detections with some configurations. This document discusses these issues and proposes a solution. The document is associated with the CRs provided for 38.304 [1] and 38.331 [2].

# Discussion

For configuration/allocation of paging frames (PFs), the LTE algorithm will be reused in NR with the addition of an offset which allows shifting of the PFs a desired number of steps in time. For the configuration/allocation of paging occasions (POs) within a PF, the LTE algorithm is however not suitable in its entirety for NR.

In LTE, the configuration of POs within a PF is based on a table where the input parameters are Ns = max(1, nB/T) (i.e. Ns is the number of POs in a PF) and i_s = floor(UE_ID/N) mod Ns (i.e. i_s is an index pointing out a certain UE’s PO within a PF). The table maps POs to specific subframe(s) in the PF as well as allows a UE to derive its specific PO in the PF based on its UE ID.

This table-based approach of LTE is not suitable for NR. In LTE it was simple to map a PO to a subframe and this could easily be done through the table specified for this purpose. However, in NR a PO cannot simply be mapped to a subframe. In terms of transmission resources, a subframe is an unambiguous concept in LTE (with the only variation being normal or extended cyclic prefix). In NR, on the other hand, the transmission resources (in terms of slots and hence OFDM symbols) vary with different numerologies (i.e. subcarrier spacings, SCSs). In addition, the length of a PO in NR is variable and depends on the number of beams needed in a possible beam sweep for the PDCCH paging transmission in combination with the SCS and consequent symbol length. And maybe most importantly, when multiple beams are used for paging transmission in NR, a PO will in essence consist of a set of PDCCH monitoring occasions – one for each beam – and these PDCCH monitoring occasions have to be allocated in time. For these reasons, the table-based PO configuration mechanism of LTE has been replaced by a mechanism based on the paging-SearchSpace in NR. The Ns and i_s parameters are retained (although Ns is configured independently in NR), but they no longer point out subframes in a paging frame, but rather sets of PDCCH monitoring occasions (constituting PDCCH beam sweeps) in a PF, where each set of PDCCH monitoring occasions forms a PO.

The following was agreed in RAN2 with regards to paging on the RAN2 ad hoc 1807 meeting in Montreal:

Agreements:

0		Define a useful paging PDCCH monitoring occasion as a monitoring occasion doesn’t conflict with UL slots/symbols.

1		For non-default association, one PO comprises of ‘N’ useful paging PDCCH monitoring occasion where ‘N’ is equal to number of actual transmitted SSBs.  RAN2 understanding is that the Kth monitoring occasion in the PO is corresponded to the Kth transmitted SSB.

2		For non-default association, (i_s + 1)th PO is a set of N consecutive useful paging PDCCH monitoring occasions for paging starting from the (i_s * N)th PDCCH monitoring occasion. The useful paging PDCCH monitoring occasions starting from 1st useful paging PDCCH monitoring occasion for paging in the paging frame are sequentially numbered from zero. FFS the necessity to introduce additional parameter to indicate the first PDCCH monitoring occasion of each PO in a PF.

3		Support to configure Ns and N value instead of nB.

In addition, it was agreed to increase the maximum number of paging records in a Paging message from 16 in LTE to 32 in NR.

Although the currently agreed algorithm for configuration of POs within a PF is adapted to the circumstances of NR, it does not always achieve the same maximum paging capacity as LTE without increasing the number of UEs being paged by the same Paging message. This applies for both the default case and the non-default case. Lower paging capacity than in LTE may be acceptable for the default case, because a network operator can always choose to use the non-default case, but it is not acceptable for the non-default case. In LTE, the network can always configure POs as densely as 4 in each radio frame (when all radio frames are paging frames). In NR, on the other hand, 4 POs cannot always be fit time-multiplexed in a paging frame, depending on the SCS, the number of beams used for covering the cell and how densely the PDCCH beams for paging can be transmitted. We think that the paging capacity in NR should be at least equal to the paging capacity of LTE.

The current mechanism for configuration of POs does not achieve the same maximum paging capacity as LTE in cases where less than 4 POs can fit in a PF, unless the possibility to page more UEs with the same Paging message than in LTE is utilized.

In addition to the paging capacity issue, the reduction of the maximum PO density also has a negative impact on a UE’s energy consumption in RRC_IDLE or RRC_INACTIVE state. The reason is that fewer POs per paging DRX cycle means that more UEs will be allocated to the same PO. This in turn will increase the number of “false” page detections, i.e. cases where a page monitoring UE detects a page indication on the PDCCH, i.e. a DCI message with a CRC scrambled with the P-RNTI, and goes on to decode the Paging message on the PDSCH, only to find that it is not being paged (i.e. its UE ID is not included in the Paging message).

The reduced maximum PO density increases the UE energy consumption by increasing the number of “false” page detections, i.e. cases where a page monitoring UE detects a page indication on the PDCCH, i.e. a DCI message with a CRC scrambled with the P-RNTI, and goes on to decode the Paging message on the PDSCH, only to find that it is not being paged (i.e. its UE ID is not included in the Paging message).

It has been suggested that the agreed increase of the maximum number of paging records in a Paging message – i.e. the increase of the number of UEs that can be paged in the same PO – from 16 in LTE to 32 in NR compensates for the lower PO density and thereby solves the paging capacity issue.Increasing the number of UE IDs that can be included in a Paging message would indeed lead to an increased paging capacity. However, since the POs are still fewer than in LTE, the UE energy consumption issue with the increased number of “false” page detections still remains.

Increasing the maximum number of UE IDs (paging records) in the Paging message increases the paging capacity, but does not address the UE energy consumption issue with the increased number of “false” page detections.

It is better to use a method where the desired number of POs, i.e. 4, can be configured, so that the UEs can be distributed to more POs. This way, the paging capacity is increased (i.e. the paging capacity of LTE is retained) without increasing the UE energy consumption through increased number of “false” page detections. Therefore we advocate the previously proposed concept of frequency-multiplexing POs.

Frequency-multiplexing of POs addresses the paging capacity issue by ensuring that up to 4 POs can be configured in a PF without increasing the UE energy consumption through increased number of “false” page detections.

Introduce frequency-multiplexing of POs as an optional configuration in NR, so that a maximum paging capacity on par with LTE can be achieved in all scenarios without negative impact on the UE energy consumption caused by more frequent “false” page detections.

When two POs are frequency-multiplexed, they have disjoint CORESETs, while they share the same pagingSearchSpace parameters. Time-multiplexed POs (within the same PF) with the same CORESET could be said to belong to the same “layer” of POs. When frequency-multiplexing of POs is introduced, there could thus be multiple layers of POs, each with its own disjoint CORESET. Assuming that maximum 4 POs should be possible to configure in a PF, there could theoretically be up to 4 layers of POs with 1 PO per layer per PF. However, more realistic is maybe to assume no more than 2 layers, which means that 2 POs would have to be time-multiplexed on each layer to achieve the maximum number of POs, i.e. 4, in a PF. This is illustrated in Figure 1.

Figure 1. Illustration of realization of two layers of POs through two time-multiplexed pairs of POs, where the POs in a pair are configured with different, disjoint CORESETs, i.e. they are frequency-multiplexed.

To support a second layer of POs, it should thus be possible to configure an additional optional CORESET for paging in SIB1.

To support a second layer of frequency-multiplexed POs in a PF, it should be possible to configure an additional optional CORESET for paging in SIB1. The same pagingSearchSpace parameters are associated with the POs on the second layer as with the POs on the first layer. This should be captured in the specifications in accordance with the associated CRs.

When all the POs to be included in a PF (as indicated by the Ns parameter) cannot fit into the same layer – i.e. when time-multiplexing is not enough – then two layers have to be used to accommodate the POs. However, an operator could choose to configure POs on different layers, even if one layer would have been enough, e.g. to make more efficient use of the DL transmission resources when analog DL TX beamforming is used or to facilitate TDD operation. This would be a matter of operator preferences.

It should be possible to configure two frequency-multiplexed layers of POs, even when all the POs would have fit in the PF using only time-multiplexing.

Since it may thus be ambiguous whether one or two layers are used for the POs in a PF, a UE has to have a way of knowing if and how POs are allocated to different PO layers in cases where there are more than one PO per PF. The straightforward way is to let the UE determine this based on the presence or absence of the additional CORESET in SIB1. If the additional CORESET is configured, the POs are distributed to two layers (1 PO on each layer if Ns = 2 and 2 POs on each layer if Ns = 4).

A UE determines from the presence or absence of the additional CORESET for paging in SIB1, whether one or two layers of frequency-multiplexed POs is/are used (i.e. whether frequency-multiplexing or only time-multiplexing of POs is used).

As stated above, the Ns and i_s parameters can still be reused from the LTE PO configuration algorithm (although the Ns parameter is independently configured in NR). With reference to Figure 1, which illustrates a case with Ns = 4 and 2 POs on each layer, the POs are numbered as illustrated, starting at the first layer and continuing at the second layer. The i_s parameter a UE calculates determines which PO it should monitor according to this numbering. That is, if i_s = 0, the UE monitors PO 1, if i_s = 1, the UE monitors PO 2 (i.e. the second time-multiplexed PO on the first layer), if i_s = 2, the UE monitors PO 3 (i.e. the first PO on the second layer, frequency-multiplexed with PO 1), and if i_s = 3, the UE monitors PO 4 (i.e. the second time-multiplexed PO on the second layer, frequency-multiplexed with PO 2). The first layer of POs is the one associated with the first configured CORESET for paging in SIB1 and the second layer of POs is the one associated with the additional (second) configured CORESET for paging in SIB1.

The Ns and i_s parameters are reused from the PO configuration algorithm of LTE (although the Ns parameter is independently configured in NR). The i_s parameter points out POs ordered in the time direction, starting at the first layer and continuing at the second (frequency-multiplexed) layer of POs (if any). The first layer of POs is the one associated with the first configured CORESET for paging provided in SIB1 and the second layer of POs is the one associated with the additional (second) configured CORESET for paging provided in SIB1.

As mentioned above, an operator may choose to configure two frequency-multiplexed layers of POs, even when the POs would have fit within the PF using only time-multiplexing. In general, frequency-multiplexing of POs is an attractive concept in high carrier frequency bands. High carrier frequencies are typically associated with large bandwidths, analog DL TX beamforming and many narrow beams to cover a cell area, and TDD operation. All these properties provide a good match with frequency-multiplexed POs. The large bandwidth provides lots of room in the frequency domain for frequency-multiplexing of both PDCCH and PDSCH transmissions. Since analog DL TX beamforming means that the gNB can only transmit in one direction at a time, the DL transmission resources not used for paging are typically wasted during the paging beam sweep, unless the gNB opportunistically can schedule another pending DL transmission at the same time and in the same direction as the page. And since many beams are used in high carrier frequencies, this resource waste amounts to a great overhead. Frequency-multiplexing of POs makes more efficient use of the transmission resources during the PDCCH beam sweep (taking care of two POs/paging transmissions with a single beam sweep) and thus reduces the overhead. Finally, confining the POs, and thus paging transmissions, to a shorter time interval by frequency-multiplexing of POs makes more time available for TDD UL operation, thus increasing the scheduler’s flexibility.

Frequency-multiplexing of POs is a good match with high carrier frequencies and is thus especially attractive in such scenarios.

It should be pointed out that frequency-multiplexing of POs and increasing the maximum number of UE IDs (paging records) in the Paging message are in no way mutually exclusive alternatives. On the contrary, they may well be used in parallel to achieve even higher paging capacity, i.e. greater than in LTE. Furthermore, when it was proposed (and agreed) to remove the nB parameter and instead let Ns and N be configurable independently of each other, this was under the assumption that a network operator may not want to make all radio frames PFs, but may still want to be able to achieve high paging capacity by configuring multiple POs per PF. In such deployments, the combination of frequency-multiplexed POs and increased maximum number of UE IDs (paging records) in the Paging message may be needed to reach the maximum paging capacity of LTE.

Frequency-multiplexing of POs and increasing the maximum number of UE IDs (paging records) in the Paging message are not mutually exclusive alternatives, but may be used in parallel. This could be beneficial when even higher maximum paging capacity than in LTE is needed or when the network operator wants to achieve high paging capacity without making every radio frame a PF.

# Conclusion

In section 2 we made the following observations:

Observation 1	The current mechanism for configuration of POs does not achieve the same maximum paging capacity as LTE in cases where less than 4 POs can fit in a PF, unless the possibility to page more UEs with the same Paging message than in LTE is utilized.

Observation 2	The reduced maximum PO density increases the UE energy consumption by increasing the number of “false” page detections, i.e. cases where a page monitoring UE detects a page indication on the PDCCH, i.e. a DCI message with a CRC scrambled with the P-RNTI, and goes on to decode the Paging message on the PDSCH, only to find that it is not being paged (i.e. its UE ID is not included in the Paging message).

Observation 3	Increasing the maximum number of UE IDs (paging records) in the Paging message increases the paging capacity, but does not address the UE energy consumption issue with the increased number of “false” page detections.

Observation 4	Frequency-multiplexing of POs addresses the paging capacity issue by ensuring that up to 4 POs can be configured in a PF without increasing the UE energy consumption through increased number of “false” page detections.

Observation 5	Frequency-multiplexing of POs is a good match with high carrier frequencies and is thus especially attractive in such scenarios.

Observation 6	Frequency-multiplexing of POs and increasing the maximum number of UE IDs (paging records) in the Paging message are not mutually exclusive alternatives, but may be used in parallel. This could be beneficial when even higher maximum paging capacity than in LTE is needed or when the network operator wants to achieve high paging capacity without making every radio frame a PF.

Based on the discussion in section 2 we propose the following:

Proposal 1	Introduce frequency-multiplexing of POs as an optional configuration in NR, so that a maximum paging capacity on par with LTE can be achieved in all scenarios without negative impact on the UE energy consumption caused by more frequent “false” page detections.

Proposal 2	To support a second layer of frequency-multiplexed POs in a PF, it should be possible to configure an additional optional CORESET for paging in SIB1. The same pagingSearchSpace parameters are associated with the POs on the second layer as with the POs on the first layer. This should be captured in the specifications in accordance with the associated CRs.

Proposal 3	It should be possible to configure two frequency-multiplexed layers of POs, even when all the POs would have fit in the PF using only time-multiplexing.

Proposal 4	A UE determines from the presence or absence of the additional CORESET for paging in SIB1, whether one or two layers of frequency-multiplexed POs is/are used (i.e. whether frequency-multiplexing or only time-multiplexing of POs is used).

Proposal 5	The Ns and i_s parameters are reused from the PO configuration algorithm of LTE (although the Ns parameter is independently configured in NR). The i_s parameter points out POs ordered in the time direction, starting at the first layer and continuing at the second (frequency-multiplexed) layer of POs (if any). The first layer of POs is the one associated with the first configured CORESET for paging provided in SIB1 and the second layer of POs is the one associated with the additional (second) configured CORESET for paging provided in SIB1.

# References

R2-1812288, Support of frequency-multiplexed paging occasions, CR 38.304, Ericsson, TSG-RAN WG2 #103, 20th – 24th August 2018

R2-1812289, Support of frequency-multiplexed paging occasions, draftCR 38.331, Ericsson, TSG-RAN WG2 #103, 20th – 24th August 2018