3GPP TSG-RAN WG2 Meeting #103 	R2-1812325

# Gothenberg, Sweden, 20th August 2018 – 24th August 2018

| CR-Form-v11.2.1 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.321 | CR | 0322 | rev | - | Current version: | 15.2.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | CR on L field description for MAC subheader |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | OPPO |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R2 |  |  |  |  |  |  |  |  |  |
| Work item code: | NR_newRAT-Core |  |  |  |  |  | Date: |  |  | 2018-08-04 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-15 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) Rel-12 (Release 12) Rel-13 (Release 13) Rel-14 (Release 14) Rel-15 (Release 15) Rel-16 (Release 16) |  |
| Reason for change: |  | In the last RAN2 AH#1807 meeting, it has agreed in principle based on CR R2-1810071 that the second LCID value implies a length of 64 bits for the CCCH SDU and the length field is not included when this LCID value is used. There is another change should be also captured which is the L field interpretation in 6.2.1. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Update the interpretation of L field in 6.2.1 MAC subheader for DL-SCH and UL-SCH. Impact analysis Impacted functionality: MAC PDU format Inter-operability: - If the network is implemented according to the CR and the UE is not there are no inter-operability problems. - If the UE is implemented according to the CR and the network is not, there are no inter-operability problems. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | L field description is not correctly captured by the agreement. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 6.2.1 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  |  | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  |  | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  |  | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |

Start of change

### 6.2.1	MAC subheader for DL-SCH and UL-SCH

The MAC subheader consists of the following fields:

-	LCID: The Logical Channel ID field identifies the logical channel instance of the corresponding MAC SDU or the type of the corresponding MAC CE or padding as described in Tables 6.2.1-1 and 6.2.1-2 for the DL-SCH and UL-SCH respectively. There is one LCID field per MAC subheader. The LCID field size is 6 bits;

-	L: The Length field indicates the length of the corresponding MAC SDU or variable-sized MAC CE in bytes. There is one L field per MAC subheader except for subheaders corresponding to fixed-sized MAC CEs, padding,  and a MAC SDU of CCCH. The size of the L field is indicated by the F field;

-	F: The Format field indicates the size of the Length field. There is one F field per MAC subheader except for subheaders corresponding to fixed-sized MAC CEs and padding. The size of the F field is 1 bit. The value 0 indicates 8 bits of the Length field. The value 1 indicates 16 bits of the Length field;

-	R: Reserved bit, set to zero.

The MAC subheader is octet aligned.

Table 6.2.1-1 Values of LCID for DL-SCH

| Index | LCID values |
| --- | --- |
| 000000 | CCCH |
| 000001–100000 | Identity of the logical channel |
| 100001–101110 | Reserved |
| 101111 | Recommended bit rate |
| 110000 | SP ZP CSI-RS Resource Set Activation/Deactivation |
| 110001 | PUCCH spatial relation Activation/Deactivation |
| 110010 | SP SRS Activation/Deactivation |
| 110011 | SP CSI reporting on PUCCH Activation/Deactivation |
| 110100 | TCI State Indication for UE-specific PDCCH |
| 110101 | TCI States Activation/Deactivation for UE-specific PDSCH |
| 110110 | Aperiodic CSI Trigger State Subselection |
| 110111 | SP CSI-RS / CSI-IM Resource Set Activation/Deactivation |
| 111000 | Duplication Activation/Deactivation |
| 111001 | SCell Activation/Deactivation (four octet) |
| 111010 | SCell Activation/Deactivation (one octet) |
| 111011 | Long DRX Command |
| 111100 | DRX Command |
| 111101 | Timing Advance Command |
| 111110 | UE Contention Resolution Identity |
| 111111 | Padding |

Table 6.2.1-2 Values of LCID for UL-SCH

| Index | LCID values |
| --- | --- |
| 000000 | CCCH of size 64 bits |
| 000001–100000 | Identity of the logical channel |
| 100001 | CCCH of size 48 bits |
| 100010–110100 | Reserved |
| 110101 | Recommended bit rate query |
| 110110 | Multiple Entry PHR (four octet Ci) |
| 110111 | Configured Grant Confirmation |
| 111000 | Multiple Entry PHR (one octet Ci) |
| 111001 | Single Entry PHR |
| 111010 | C-RNTI |
| 111011 | Short Truncated BSR |
| 111100 | Long Truncated BSR |
| 111101 | Short BSR |
| 111110 | Long BSR |
| 111111 | Padding |

End of change