3GPP TSG-RAN WG2 Meeting #103	R2-1812341

Gothenburg, Sweden, 20 - 24 August 2018

Agenda item:	11.1.3

Source:	Nokia, Nokia Shanghai Bell, Qualcomm Incorporated, Huawei, Ericsson

Title:	Topology Adaptation Scenarios

WID/SID:	FS_NR_IAB - Release 16

Document for:	Discussion and Decision

# 1	Introduction

The IAB study item aims to define integrated access and backhauling (IAB) solutions for NR [1]. It has been agreed that IAB supports topology adaptation [2]. Further, two architecture groups have been defined and included in the study [2]. In RAN-3 #100, a text proposal defined the spanning-tree and the directed-acyclic-graph topologies to be supported by IAB [3].

This contribution identifies the possible topology adaptation scenarios for architecture 1a from deployment perspective. Further, it proposes to capture these scenarios in the TR 38.874.

# 2	Discussion

### 2.1 Topology Adaptation Scenarios

In Architecture group 1a, following are the list of possible topology adaptation scenarios.

- Intra-Donor CU, Intra-Donor DU:

In this scenario, the IAB node migrates from one parent node to another parent node under same Donor DU as shown in the below figure.

Figure 1: Topology Adaptation: Intra-Donor CU, Intra-Donor DU

The Donor-CU initiates the topology adaptation based on measurement reports from migrating IAB node’s MT, including other IAB nodes within the configured topology.

This scenario triggers the following:

- Establishment of new route between the migrating IAB node and the Donor DU. This includes the establishment or reconfiguration of RLC channels.

- Release the old routes and contexts.

- Intra-Donor CU, Inter-Donor DU

In this scenario, the IAB node migrates from a parent node served by one Donor DU to a parent node served by a different Donor DU, both connected to same Donor CU as shown in the below figure.

Figure 2: Topology Adaptation: Intra-Donor CU, Inter-Donor DU

The Donor-CU initiates the topology adaptation based on measurement reports from migrating IAB node’s MT, including other IAB nodes within the configured topology.

This scenario triggers the following:

- Establishment of new route between the migrating IAB node and the target Donor DU. This includes the establishment or reconfiguration of RLC channels.

- Reconfiguration of F1-C association and F1-U tunnels, this involves the redirection of F1-U tunnels and F1-AP onto new route.

- Release of old routes and contexts.

- Inter-Donor CU, Inter-Donor DU

In this scenario, the IAB node migrates from a parent node served by one Donor DU to a parent node served by a different Donor DU connected to a different Donor CU, as shown in the below figure.

Figure 3: Topology Adaptation: Inter-Donor CU, Inter-Donor DU

The Donor-CU initiates the topology adaptation based on measurement reports from migrating IAB node’s MT, including other IAB nodes within the configured topology.

This would be an inter-CU HO scenario that would result into the following:

- Establishment of new F1-C association and F1-U tunnels between the migrating IAB node and the target CU, while releasing the existing F1-C associations and F1-U tunnels with serving CU.

- Establishment of new route between the migrating IAB node and the target Donor DU.

- This may result into inter CU HO of all access UEs and subtending IAB nodes connected below the migrating IAB node.

- Release of old contexts and routes.

# 4	Summary

The contribution identifies the possible topology adaptation scenarios for architecture 1a. The related TP for TR 38.874 is also provided.

# References

[1] RP-181349: SI description for Study on Integrated Access and Backhaul for NR. Revision of RP-172219. RAN-Plenary, La Jolla, CA, USA, June 11-15, 2018.

[2] R3-183110: TR 38874 v02.1, 3GPP TSG-RAN WG3 #100 Busan, South Korea, May 21-25, 2018

[3] R3-183577: TP on IAB Topologies, 3GPP TSG-RAN WG3 #100 Busan, South Korea, May 21-25, 2018

# Text Proposal

<<TP start>>

## 9.x Topology Adaptation

### 9.x.x Topology Adaptation Scenarios in Architecture 1a

- Intra-Donor CU, Intra-Donor DU:

In this scenario, the IAB node migrates from one parent node to another parent node under same Donor DU as shown in the below figure.

Figure 1: Topology Adaptation: Intra-Donor CU, Intra-Donor DU

The Donor-CU initiates the topology adaptation based on measurement reports from migrating IAB node’s MT, including other IAB nodes within the configured topology.

This scenario triggers the following:

- Establishment of new route between the migrating IAB node and the Donor DU. This includes the establishment or reconfiguration of RLC channels.

- Release the old routes and contexts.

- Intra-Donor CU, Inter-Donor DU

In this scenario, the IAB node migrates from a parent node served by one Donor DU to a parent node served by a different Donor DU, both connected to same Donor CU as shown in the below figure.

Figure 2: Topology Adaptation: Intra-Donor CU, Inter-Donor DU

The Donor-CU initiates the topology adaptation based on measurement reports from migrating IAB node’s MT, including other IAB nodes within the configured topology.

This scenario triggers the following:

- Establishment of new route between the migrating IAB node and the target Donor DU. This includes the establishment or reconfiguration of RLC channels.

- Reconfiguration of F1-C association and F1-U tunnels, this involves the redirection of F1-U tunnels and F1-AP onto new route.

- Release of old routes and contexts.

- Inter-Donor CU, Inter-Donor DU

In this scenario, the IAB node migrates from a parent node served by one Donor DU to a parent node served by a different Donor DU connected to a different Donor CU, as shown in the below figure.

Figure 3: Topology Adaptation: Inter-Donor CU, Inter-Donor DU

The Donor-CU initiates the topology adaptation based on measurement reports from migrating IAB node’s MT, including other IAB nodes within the configured topology.

This would be an inter-CU HO scenario that would result into the following:

- Establishment of new F1-C association and F1-U tunnels between the migrating IAB node and the target CU, while releasing the existing F1-C associations and F1-U tunnels with serving CU.

- Establishment of new route between the migrating IAB node and the target Donor DU.

- This may result into inter CU HO of all access UEs and subtending IAB nodes connected below the migrating IAB node.

- Release of old contexts and routes.

<<TP end>>