3GPP TSG-RAN WG2 #103	R2-1812419

# Gothenburg, Sweden, 20th – 24th August 2018

Agenda Item:	9.8.5

Source:	Ericsson

Title:	Scheduling and acquisition of SI messages containing positioning SIBs

Document for:	Discussion, Decision

# 1	Introduction

At last RAN2#101 meeting, it was agreed to extend the handling of legacy SI messages by concatenating the list of legacy SI messages and the list of Positioning SI messages. However, in typical configurations of system information and positioning assistance data, the available SI windows are not enough to fit all the needed SI messages. In this paper, we highlight this issue and suggest using a specific SI window for the Positioning SI messages, and also apply an offset rather than concatenation to ensure that there are enough resources for the SI messages with positioning SIBs.

# 2	Discussion

Below is how UE will acquire SI message (with legacy and positioning SIBs) as per the agreement at RAN2#102.

### Acquisition of an SI message

When acquiring an SI message, the UE shall:

1>	determine the start of the SI-window for the concerned SI message as follows:

2>	for the concerned SI message, determine the number n which corresponds to the order of entry in the concatenated list of SI messages configured by schedulingInfoList and pos-schedulingInfoList in SystemInformationBlockType1;

2>	determine the integer value x = (n – 1)*w, where w is the si-WindowLength;

2>	the SI-window starts at the subframe #a, where a = x mod 10, in the radio frame for which SFN mod T = FLOOR(x/10), where T is the si-Periodicity of the concerned SI message;

Considering an example representing a real network deployment with four legacy LTE SI messages with SI1 (containing SIB2 and SIB3) of periodicity 80ms, SI2 (containing SIB4 and SIB5) of 160ms and SI3 and SI4 with much longer periodicity. Furthermore, support for the public warning system (PWS) mandates an si-WindowLength of 10ms.

In addition, we consider the typical case of assistance data, that was introduced by [1], and upon which the assistance data segmentation discussion was based. Furthermore, as was agreed at RAN2#101bis, MAC is suitable to provide observations associated to multiple reference stations, which is the typical case for assistance data broadcast:

As highlighted in [5], there are two options to map the SIB segments to SI message. A) All segments are mapped to one common SI message B) Each segment is mapped to one SI message

Option 1:

- 4 SI messages to represent A-GNSS broadcast

- 7 SI messages to represent GNSS RTK MAC with two GNSS

Option 2:

- 9 SI messages to represent A-GNSS broadcast

- 19 SI messages to represent GNSS RTK MAC with two GNSS

Since this positioning assistance data configuration has been used as basis for other agreements regarding the positioning assistance data broadcast, it is therefore relevant to consider this configuration when assuring that there are enough broadcast resources to support positioning assistance data.

The positioning assistance data broadcast solution in Rel. 15 could support at least 11 unique SI message or 28 unique SI messages with positioning assistance data.

Two solution components have been discussed:

- Schedule SI messages with positioning AD concatenated with legacy SI messages or with an offset

- Use the same si-WindowLength for both positioning AD and legacy, or introduce a new pos-si-WindowLength

|  | Same si-WindowLength | Separate pos-si-WindowLength |
| --- | --- | --- |
| Concatenated SI messages | Section 2.1 | Section 2.2 |
| Pos AD SI message with offset | Section 2.3 | Section 2.4 |

In real network deployments, the SI-WindowLength configured is generally more than 10ms to allow sufficient number of retransmissions [4]. With regard to this, it would benefit to have a separate pos-si-WindowLength which can be set different to the legacy SI-WindowLength.

Below in the following subsections, we will further analyse the configuration options.

The example in below section are provided using Option 2; however similar analysis can be drawn also using Option 1.

## 2.1	Concatenated SI messages

This is the configuration option that has be discussed as a starting point. If we analyse the available SI windows for positioning assistance data SI messages in this typical scenario, see Table 1, we note that there are only four SI windows available for positioning assistance data, which does not support the 28 SI messages needed.

Table 1. SI message resources available for positioning assistance data when the SIB2/3 periodicity is 80ms (only four SI windows).

| SI message | SI1 | SI2 | SI3 | SI4 |  |  |  |  | SI1 |  |  |  |  |  |  |  | SI1 | SI2 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| SFN | 0 | 1 | 2 | 3 | 4 | 5 | 6 | 7 | 8 | 9 | 10 | 11 | 12 | 13 | 14 | 15 | 16 | 17 |

One possibility to increase the number of resources could be to extend the SI period of the legacy SI messages. However, in order to support 28 SI messages with positioning assistance data, the period of the SI messages SI1 and SI2 needs to be extended to 320 ms. To change the periodicity of SIB2 and SIB3 from 80ms to 320ms could have a negative impact on legacy UE behaviour.

In a typical SI configuration, there are not enough resources to handle the positioning assistance data in the example configuration used in the 3GPP WI with concatenated SI messages.

## 2.2	Concatenated SI messages, separate pos-si-WindowLength

With the possibility of a separate pos-si-WindowLength, we can get more SI windows to be used for positioning assistance data SI messages. Since the 1ms si-WindowLength is restricted to five consecutive SI messages due to SIB1 transmission, we only analyse a 2ms pos-si-WindowLength, see Table 2.

Table 2. SI message resources available for positioning assistance data when the SIB2/3 periodicity is 80ms (only four SI windows) and pos-si-WindowLength is 2ms.

| SI message | SI1 | SI2 | SI3 | SI4 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | SI1 |  |  |  |  |  |  |  | SI1 | SI2 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| SFN | 0 | 1 | 2 | 3 | 4 |  |  |  |  | 5 |  |  |  |  |  | 6 |  |  |  |  | 7 |  |  |  |  | 8 | 9 | 10 | 11 | 12 | 13 | 14 | 15 | 16 | 17 |

Even though the number of resources has been significantly increased to 20 SI messages, it is still not enough to support the 28 SI messages with positioning assistance data from the example configuration.

An alternative could be to extend the SI period of SIB2 and SIB3 to 160ms, which could have a negative impact on legacy UE behaviour.

In a typical SI configuration, and a separate pos-si-WindowLength, there are not enough resources to handle the positioning assistance data in the example configuration used in the 3GPP WI, unless the period of legacy SI messages is limited to 160ms or longer, which could have a negative impact on legacy UE behaviour in current deployments.

## 2.3	Scheduling offset between legacy and pos AD SI messages

With the possibility of an offset between the new and legacy SI messages, it is possible to avoid the crowded first set of system frames when scheduling the SI messages.

Table 3. SI message resources available for positioning assistance data when the SIB2/3 periodicity is 80ms (only four SI windows), and pos AD SI messages are scheduled with an offset

| SI message | SI1 | SI2 | SI3 | SI4 |  |  |  |  | SI1 |  |  |  |  |  |  |  | SI1 | SI2 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| SFN | 0 | 1 | 2 | 3 | 4 | 5 | 6 | 7 | 8 | 9 | 10 | 11 | 12 | 13 | 14 | 15 | 16 | 17 |

The scheduling offset avoids all legacy SI messages at different SI periods than the shortest. However, the increase is only from four to seven SI windows, which is not enough to support the 28 SI messages in the configuration example.

An alternative is to change the SI period of SIB 2/3 to 160ms and use an offset of 160ms. That will further increase the number of SI messages to 14. In order to support 28 positioning assistance data SI messages, the period of the legacy SI messages has to be increased to 320ms, which can have a negative impact on the UE behaviour.

In a typical SI configuration, there are not enough resources to handle the positioning assistance data in the example configuration used in the 3GPP WI with only a SI message scheduling offset.

## 2.4	Scheduling offset between legacy and pos AD SI messages, separate pos-si-WindowLength

With the possibility of both a scheduling offset and a separate pos-si-WindowLength, we can get even more SI windows to be used for positioning assistance data SI messages, while leaving the configuration of legacy SI message intact. Again, since the 1ms si-WindowLength is restricted to five consecutive SI messages due to SIB1 transmission, we only analyse a 2ms pos-si-WindowLength, together with an offset of 80ms, see Table 4. In this configuration, there are 35 SI windows for pos AD SI messages in the legacy configuration with SIB2/3 at 80ms. Therefore, the configuration example with 28 pos AD SI messages is supported.

Table 4. SI message resources available for positioning assistance data when the SIB2/3 periodicity is 80ms, the scheduling offset for positioning AD SI messages is 80ms, and the pos-si-WindowLength is 2ms.

| SI message | SI1 | SI2 | SI3 | SI4 |  |  |  |  | SI1 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | SI1 | SI2 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| SFN | 0 | 1 | 2 | 3 | 4 | 5 | 6 | 7 | 8 | 9 |  |  |  |  | 10 |  |  |  |  |  | 11 |  |  |  |  | 12 |  |  |  |  | 13 |  |  |  |  | 14 |  |  |  |  | 15 |  |  |  |  | 16 | 17 |

In a typical SI configuration, there are enough resources to handle the positioning assistance data in the example configuration used in the 3GPP WI with a SI message scheduling offset and a separate pos-si-WindowLength.

## 2.5	Summary

From the analysis, it is clear that the concatenating the SI messages with legacy and pos AD will not provide enough resources to accommodate the positioning assistance data in the example configuration used in this WI. This paper discussed two solution components. If adopted individually, it is not possible to support the positioning assistance data in the example configuration without also changing the configuration of the legacy system information. However, when used in combination, the two solution components will ensure that there are enough resources to accommodate the positioning assistance data in the example configuration.

We therefore have the following proposals

Introduce an 80ms scheduling offset for positioning assistance data broadcast.

Introduce a separate pos-si-WindowLength for positioning assistance data broadcast.

Agree to the text proposal in the appendix based upon agreed CR R2-1808889

# 3.	Conclusions

In this paper, we have analysed the SI scheduling, and concluded that to simply concatenate the legacy and positioning AD SI messages will not provide enough resources to support the example configuration discussed in this WI. We have the following observations and proposals.

Observation 1	The positioning assistance data broadcast solution in Rel. 15 could support at least 11 unique SI message or 28 unique SI messages with positioning assistance data.

Observation 2	In a typical SI configuration, there are not enough resources to handle the positioning assistance data in the example configuration used in the 3GPP WI with concatenated SI messages.

Observation 3	In a typical SI configuration, and a separate pos-si-WindowLength, there are not enough resources to handle the positioning assistance data in the example configuration used in the 3GPP WI, unless the period of legacy SI messages is limited to 160ms or longer, which could have a negative impact on legacy UE behaviour in current deployments.

Observation 4	In a typical SI configuration, there are not enough resources to handle the positioning assistance data in the example configuration used in the 3GPP WI with only a SI message scheduling offset.

Observation 5	In a typical SI configuration, there are enough resources to handle the positioning assistance data in the example configuration used in the 3GPP WI with a SI message scheduling offset and a separate pos-si-WindowLength.

Based on the discussion in the previous sections we propose the following:

Proposal 1	Introduce an 80ms scheduling offset for positioning assistance data broadcast.

Proposal 2	Introduce a separate pos-si-WindowLength for positioning assistance data broadcast.

Proposal 3	Agree to the text proposal in the appendix based upon agreed CR R2-1808889

# References

R2-1804786, “SIB Segmentation”, Qualcomm Incorporated

R2-1808107	, “Addition of broadcast of positioning assistance data” Qualcomm Incorporated, RAN2#102

R2-1808756, “Addition of broadcast of positioning assistance data”, Ericsson, RAN2#102

3GPP TS 36.508 v15.2.0,

R2-1808698 Discussion on the broadcasting of assistance data	Huawei, HiSilicon

# Text Proposal, 36.331

### 5.2.3	Acquisition of an SI message

### Acquisition of an SI message

When acquiring an SI message, the UE shall:

1> if the concerned SI message is configured by the schedulingInfoList determine the start of the SI-window for the concerned SI message as follows:

2> for the concerned SI message, determine the number n which corresponds to the order of entry in the concatenated list of SI messages configured by schedulingInfoList and pos-schedulingInfoList in SystemInformationBlockType1;

2> determine the integer value x = (n – 1)*w, where w is the si-WindowLength;

2> the SI-window starts at the subframe #a, where a = x mod 10, in the radio frame for which SFN mod T = FLOOR(x/10), where T is the si-Periodicity of the concerned SI message;

- else if the concerned SI message is configured by the posSchedulingInfoList and si-posOffset is configured determine the start of the SI-window for the concerned SI message as follows:

2> determine the number m which corresponds to the number of SI messages configured by schedulingInfoList in SystemInformationBlockType1; that are associated to the shortest SI periodicity 8 radio frames (80 ms).

2> for the concerned SI message, determine the number n which corresponds to the order of entry in the list of SI messages for Positioning SIBs configured by posSchedulingInfoList in SystemInformationBlockType1;

2> determine the integer value x = m*w + (n – 1)*wp, where w is the si-WindowLength and wp is the pos-si-WindowLength.

2> the SI-window with the concerned SI message of Positioning SIBs starts at the subframe #a, where a = x mod 10, in the radio frame for which SFN mod T = FLOOR(x/10) + 8, where T is the si-posPeriodicity of the concerned SI message;

### 6.2.2	Message definitions

#### […]

#### SystemInformationBlockType1

SystemInformationBlockType1 contains information relevant when evaluating if a UE is allowed to access a cell and defines the scheduling of other system information. SystemInformationBlockType1-BR uses the same structure as SystemInformationBlockType1.

Signalling radio bearer: N/A

RLC-SAP: TM

Logical channels: BCCH and BR-BCCH

Direction: E-UTRAN to UE

SystemInformationBlockType1 message

-- ASN1START

SystemInformationBlockType1-BR-r13 ::=	SystemInformationBlockType1

SystemInformationBlockType1 ::=		SEQUENCE {

cellAccessRelatedInfo				SEQUENCE {

plmn-IdentityList					PLMN-IdentityList,

trackingAreaCode					TrackingAreaCode,

cellIdentity						CellIdentity,

cellBarred							ENUMERATED {barred, notBarred},

intraFreqReselection				ENUMERATED {allowed, notAllowed},

csg-Indication						BOOLEAN,

csg-Identity						CSG-Identity			OPTIONAL	-- Need OR

},

cellSelectionInfo					SEQUENCE {

q-RxLevMin							Q-RxLevMin,

q-RxLevMinOffset					INTEGER (1..8)			OPTIONAL	-- Need OP

},

p-Max								P-Max						OPTIONAL,			-- Need OP

freqBandIndicator					FreqBandIndicator,

schedulingInfoList					SchedulingInfoList,

tdd-Config							TDD-Config					OPTIONAL,	-- Cond TDD

si-WindowLength						ENUMERATED {

ms1, ms2, ms5, ms10, ms15, ms20,

ms40},

systemInfoValueTag					INTEGER (0..31),

nonCriticalExtension				SystemInformationBlockType1-v890-IEs	OPTIONAL

}

SystemInformationBlockType1-v890-IEs::=	SEQUENCE {

lateNonCriticalExtension			OCTET STRING (CONTAINING SystemInformationBlockType1-v8h0-IEs)			OPTIONAL,

nonCriticalExtension				SystemInformationBlockType1-v920-IEs	OPTIONAL

}

-- Late non critical extensions

SystemInformationBlockType1-v8h0-IEs ::=	SEQUENCE {

multiBandInfoList					MultiBandInfoList		OPTIONAL,	-- Need OR

nonCriticalExtension				SystemInformationBlockType1-v9e0-IEs	OPTIONAL

}

SystemInformationBlockType1-v9e0-IEs ::= SEQUENCE {

freqBandIndicator-v9e0				FreqBandIndicator-v9e0		OPTIONAL,	-- Cond FBI-max

multiBandInfoList-v9e0				MultiBandInfoList-v9e0		OPTIONAL,	-- Cond mFBI-max

nonCriticalExtension				SystemInformationBlockType1-v10j0-IEs	OPTIONAL

}

SystemInformationBlockType1-v10j0-IEs ::= SEQUENCE {

freqBandInfo-r10					NS-PmaxList-r10				OPTIONAL,	-- Need OR

multiBandInfoList-v10j0				MultiBandInfoList-v10j0		OPTIONAL,	-- Need OR

nonCriticalExtension				SystemInformationBlockType1-v10l0-IEs					OPTIONAL

}

SystemInformationBlockType1-v10l0-IEs ::= SEQUENCE {

freqBandInfo-v10l0					NS-PmaxList-v10l0			OPTIONAL,	-- Need OR

multiBandInfoList-v10l0				MultiBandInfoList-v10l0		OPTIONAL,	-- Need OR

nonCriticalExtension				SEQUENCE {}					OPTIONAL

}

-- Regular non critical extensions

SystemInformationBlockType1-v920-IEs ::=	SEQUENCE {

ims-EmergencySupport-r9				ENUMERATED {true}			OPTIONAL,	-- Need OR

cellSelectionInfo-v920				CellSelectionInfo-v920		OPTIONAL,	-- Cond RSRQ

nonCriticalExtension				SystemInformationBlockType1-v1130-IEs	OPTIONAL

}

SystemInformationBlockType1-v1130-IEs ::=	SEQUENCE {

tdd-Config-v1130				TDD-Config-v1130			OPTIONAL,	-- Cond TDD-OR

cellSelectionInfo-v1130			CellSelectionInfo-v1130		OPTIONAL,	-- Cond WB-RSRQ

nonCriticalExtension			SystemInformationBlockType1-v1250-IEs	OPTIONAL

}

SystemInformationBlockType1-v1250-IEs ::=	SEQUENCE {

cellAccessRelatedInfo-v1250					SEQUENCE {

category0Allowed-r12						ENUMERATED {true}		OPTIONAL	-- Need OP

},

cellSelectionInfo-v1250					CellSelectionInfo-v1250		OPTIONAL,	-- Cond RSRQ2

freqBandIndicatorPriority-r12			ENUMERATED {true}			OPTIONAL,	-- Cond mFBI

nonCriticalExtension			SystemInformationBlockType1-v1310-IEs	OPTIONAL

}

SystemInformationBlockType1-v1310-IEs ::=	SEQUENCE {

hyperSFN-r13								BIT STRING (SIZE (10))		OPTIONAL,	-- Need OR

eDRX-Allowed-r13							ENUMERATED {true}			OPTIONAL,	-- Need OR

cellSelectionInfoCE-r13					CellSelectionInfoCE-r13	OPTIONAL,	-- Need OP

bandwidthReducedAccessRelatedInfo-r13	SEQUENCE {

si-WindowLength-BR-r13					ENUMERATED {

ms20, ms40, ms60, ms80, ms120,

ms160, ms200, spare},

si-RepetitionPattern-r13				ENUMERATED {everyRF, every2ndRF, every4thRF,

every8thRF},

schedulingInfoList-BR-r13				SchedulingInfoList-BR-r13	OPTIONAL,	-- Cond SI-BR

fdd-DownlinkOrTddSubframeBitmapBR-r13	CHOICE {

subframePattern10-r13					BIT STRING (SIZE (10)),

subframePattern40-r13					BIT STRING (SIZE (40))

}																	OPTIONAL, 	-- Need OP

fdd-UplinkSubframeBitmapBR-r13			BIT STRING (SIZE (10)) 		OPTIONAL, 	-- Need OP

startSymbolBR-r13						INTEGER (1..4),

si-HoppingConfigCommon-r13				ENUMERATED {on,off},

si-ValidityTime-r13						ENUMERATED {true}	OPTIONAL,			-- Need OP

systemInfoValueTagList-r13				SystemInfoValueTagList-r13	OPTIONAL	-- Need OR

}																OPTIONAL,	-- Cond BW-reduced

nonCriticalExtension						SystemInformationBlockType1-v1320-IEs	OPTIONAL

}

SystemInformationBlockType1-v1320-IEs ::=	SEQUENCE {

freqHoppingParametersDL-r13				SEQUENCE {

mpdcch-pdsch-HoppingNB-r13				ENUMERATED {nb2, nb4}		OPTIONAL,	-- Need OR

interval-DLHoppingConfigCommonModeA-r13	CHOICE {

interval-FDD-r13					ENUMERATED {int1, int2, int4, int8},

interval-TDD-r13					ENUMERATED {int1, int5, int10, int20}

}																	OPTIONAL,	-- Need OR

interval-DLHoppingConfigCommonModeB-r13	CHOICE {

interval-FDD-r13					ENUMERATED {int2, int4, int8, int16},

interval-TDD-r13					ENUMERATED { int5, int10, int20, int40}

}																	OPTIONAL,	-- Need OR

mpdcch-pdsch-HoppingOffset-r13			INTEGER (1..maxAvailNarrowBands-r13)	OPTIONAL	-- Need OR

}																OPTIONAL,	-- Cond Hopping

nonCriticalExtension						SystemInformationBlockType1-v1350-IEs					OPTIONAL

}

SystemInformationBlockType1-v1350-IEs ::=	SEQUENCE {

cellSelectionInfoCE1-r13				CellSelectionInfoCE1-r13	OPTIONAL,	-- Need OP

nonCriticalExtension					SystemInformationBlockType1-v1360-IEs				OPTIONAL

}

SystemInformationBlockType1-v1360-IEs ::=	SEQUENCE {

cellSelectionInfoCE1-v1360				CellSelectionInfoCE1-v1360	OPTIONAL, 	-- Cond QrxlevminCE1

nonCriticalExtension						SystemInformationBlockType1-v1430-IEs		OPTIONAL

}

SystemInformationBlockType1-v1430-IEs ::=	SEQUENCE {

eCallOverIMS-Support-r14				ENUMERATED {true}			OPTIONAL,	-- Need OR

tdd-Config-v1430						TDD-Config-v1430			OPTIONAL,	-- Cond TDD-OR

cellAccessRelatedInfoList-r14			SEQUENCE (SIZE (1..maxPLMN-1-r14)) OF

CellAccessRelatedInfo-r14	OPTIONAL,	-- Need OR

nonCriticalExtension					SystemInformationBlockType1-v1450-IEs				OPTIONAL

}

SystemInformationBlockType1-v1450-IEs ::=	SEQUENCE {

tdd-Config-v1450						TDD-Config-v1450		OPTIONAL,	-- Cond TDD-OR

nonCriticalExtension					SystemInformationBlockType1-v15xy-IEs																						OPTIONAL

}

SystemInformationBlockType1-v15xy-IEs ::= 	SEQUENCE {

pos-schedulingInfoList-r15				Pos-SchedulingInfoList-r15,

si-posOffset-r15						ENUMERATED {true}		OPTIONAL,	-- Need OP

si-posWindowLength-r15					ENUMERATED {ms1, ms2, ms5, ms10, ms15,  ms20, ms40}

nonCriticalExtension					SEQUENCE {}					OPTIONAL

}

PLMN-IdentityList ::=					SEQUENCE (SIZE (1..maxPLMN-r11)) OF PLMN-IdentityInfo

PLMN-IdentityInfo ::=					SEQUENCE {

plmn-Identity							PLMN-Identity,

cellReservedForOperatorUse				ENUMERATED {reserved, notReserved}

}

SchedulingInfoList ::= SEQUENCE (SIZE (1..maxSI-Message)) OF SchedulingInfo

SchedulingInfo ::=	SEQUENCE {

si-Periodicity						ENUMERATED {

rf8, rf16, rf32, rf64, rf128, rf256, rf512},

sib-MappingInfo						SIB-MappingInfo

}

SchedulingInfoList-BR-r13 ::= SEQUENCE (SIZE (1..maxSI-Message)) OF SchedulingInfo-BR-r13

SchedulingInfo-BR-r13 ::=	SEQUENCE {

si-Narrowband-r13				INTEGER (1..maxAvailNarrowBands-r13),

si-TBS-r13						ENUMERATED {b152, b208, b256, b328, b408, b504, b600, b712,

b808, b936}

}

SIB-MappingInfo ::= SEQUENCE (SIZE (0..maxSIB-1)) OF SIB-Type

SIB-Type ::=						ENUMERATED {

sibType3, sibType4, sibType5, sibType6,

sibType7, sibType8, sibType9, sibType10,

sibType11, sibType12-v920, sibType13-v920,

sibType14-v1130, sibType15-v1130,

sibType16-v1130, sibType17-v1250, sibType18-v1250,

..., sibType19-v1250, sibType20-v1310, sibType21-v1430}

SystemInfoValueTagList-r13 ::=		SEQUENCE (SIZE (1..maxSI-Message)) OF SystemInfoValueTagSI-r13

SystemInfoValueTagSI-r13 ::=		INTEGER (0..3)

CellSelectionInfo-v920 ::=			SEQUENCE {

q-QualMin-r9						Q-QualMin-r9,

q-QualMinOffset-r9					INTEGER (1..8)						OPTIONAL	-- Need OP

}

CellSelectionInfo-v1130 ::=			SEQUENCE {

q-QualMinWB-r11						Q-QualMin-r9

}

CellSelectionInfo-v1250 ::=			SEQUENCE {

q-QualMinRSRQ-OnAllSymbols-r12						Q-QualMin-r9

}

CellAccessRelatedInfo-r14 ::=	SEQUENCE {

plmn-IdentityList-r14				PLMN-IdentityList,

trackingAreaCode-r14				TrackingAreaCode,

cellIdentity-r14					CellIdentity

}

Pos-SchedulingInfoList-r15 ::= SEQUENCE (SIZE (1..maxSI-Message)) OF Pos-SchedulingInfo-r15

Pos-SchedulingInfo-r15 ::=	SEQUENCE {

pos-si-Periodicity-r15		ENUMERATED {rf8, rf16, rf32, rf64, rf128, rf256, rf512},

pos-sib-MappingInfo-r15		Pos-SIB-MappingInfo-r15

}

Pos-SIB-MappingInfo-r15 ::= SEQUENCE (SIZE (1..maxSIB)) OF Pos-SIB-Type-r15

Pos-SIB-Type-r15 ::= SEQUENCE {

encrypted-r15		ENUMERATED { true }				OPTIONAL,		-- Need OP

gnss-id-r15				GNSS-ID-r15						OPTIONAL,		-- Need OP

sbas-id-r15				SBAS-ID-r15						OPTIONAL,		-- Need OP

pos-sib-type-r15		ENUMERATED { 	posSibType1-1,

posSibType1-2,

posSibType1-3,

posSibType1-4,

posSibType1-5,

posSibType1-6,

posSibType1-7,

posSibType2-1,

posSibType2-2,

posSibType2-3,

posSibType2-4,

posSibType2-5,

posSibType2-6,

posSibType2-7,

posSibType2-8,

posSibType2-9,

posSibType2-10,

posSibType2-11,

posSibType2-12,

posSibType2-13,

posSibType2-14,

posSibType2-15,

posSibType2-16,

posSibType2-17,

posSibType2-18,

posSibType2-19,

posSibType3-1,

... }

}

-- ASN1STOP

| SystemInformationBlockType1 field descriptions |
| --- |
| bandwithReducedAccessRelatedInfo Access related information for BL UEs and UEs in CE. NOTE 3. |
| category0Allowed The presence of this field indicates category 0 UEs are allowed to access the cell. |
| cellAccessRelatedInfoList This field contains a list allowing signalling of access related information per PLMN. One PLMN can be included in only one entry of this list. NOTE 4. |
| cellBarred barred means the cell is barred, as defined in TS 36.304 [4]. |
| cellIdentity Indicates the cell identity. NOTE 2. |
| cellReservedForOperatorUse As defined in TS 36.304 [4]. |
| cellSelectionInfoCE Cell selection information for BL UEs and UEs in CE. If absent, coverage enhancement S criteria is not applicable. NOTE 3. |
| cellSelectionInfoCE1 Cell selection information for BL UEs and UEs in CE supporting CE Mode B. E-UTRAN includes this IE only if cellSelectionInfoCE is present in SystemInformationBlockType1-BR. NOTE 3. |  |
| csg-Identity Identity of the Closed Subscriber Group the cell belongs to. |
| csg-Indication If set to TRUE the UE is only allowed to access the cell if it is a CSG member cell, if selected during manual CSG selection or to obtain limited service, see TS 36.304 [4]. |
| eCallOverIMS-Support Indicates whether the cell supports eCall over IMS services for UEs as defined in TS 23.401 [41]. If absent, eCall over IMS is not supported by the network in the cell. NOTE 2. |
| eDRX-Allowed The presence of this field indicates if idle mode extended DRX is allowed in the cell. The UE shall stop using extended DRX in idle mode if eDRX-Allowed is not present. |
| encrypted The presence of this field indicates that the pos-sib-type is encrypted as specified in TS 36.355 [54]. |
| fdd-DownlinkOrTddSubframeBitmapBR The set of valid subframes for FDD downlink or TDD transmissions, see TS 36.213 [23]. If this field is present, SystemInformationBlockType1-BR-r13 is transmitted in RRCConnectionReconfiguration, and if RRCConnectionReconfiguration does not include systemInformationBlockType2Dedicated, UE may assume the valid subframes in fdd-DownlinkOrTddSubframeBitmapBR are not indicated as MBSFN subframes. If this field is not present, the set of valid subframes is the set of non-MBSFN subframes as indicated by mbsfn-SubframeConfigList. If neither this field nor mbsfn-SubframeConfigList is present, all subframes are considered as valid subframes for FDD downlink transmission, all DL subframes according to the uplink-downlink configuration (see TS 36.211 [21]) are considered as valid subframes for TDD DL transmission, and all UL subframes according to the uplink-downlink configuration (see TS 36.211 [21]) are considered as valid subframes for TDD UL transmission. The first/leftmost bit corresponds to the subframe #0 of the radio frame satisfying SFN mod x = 0, where x is the size of the bit string divided by 10. Value 0 in the bitmap indicates that the corresponding subframe is invalid for transmission. Value 1 in the bitmap indicates that the corresponding subframe is valid for transmission. |
| fdd-UplinkSubframeBitmapBR The set of valid subframes for FDD uplink transmissions for BL UEs, see TS 36.213 [23]. If the field is not present, then UE considers all uplink subframes as valid subframes for FDD uplink transmissions. The first/leftmost bit corresponds to the subframe #0 of the radio frame satisfying SFN mod x = 0, where x is the size of the bit string divided by 10. Value 0 in the bitmap indicates that the corresponding subframe is invalid for transmission. Value 1 in the bitmap indicates that the corresponding subframe is valid for transmission. |
| freqBandIndicatorPriority If the field is present and supported by the UE, the UE shall prioritize the frequency bands in the multiBandInfoList field in decreasing priority order. Only if the UE does not support any of the frequency band in multiBandInfoList, the UE shall use the value in freqBandIndicator field. Otherwise, the UE applies frequency band according to the rules defined in multiBandInfoList. NOTE 2. |
| freqBandInfo A list of additionalPmax and additionalSpectrumEmission values, as defined in TS 36.101 [42, table 6.2.4-1] for UEs neither in CE nor BL UEs and TS 36.101 [42, table 6.2.4E-1] for UEs in CE or BL UEs, for the frequency band in freqBandIndicator. If E-UTRAN includes freqBandInfo-v10l0 it includes the same number of entries, and listed in the same order, as in freqBandInfo-r10. |
| freqHoppingParametersDL Downlink frequency hopping parameters for BR versions of SI messages, MPDCCH/PDSCH of paging, MPDCCH/PDSCH of RAR/Msg4 and unicast MPDCCH/PDSCH. If not present, the UE is not configured downlink frequency hopping. |
| gnss-ID The presence of this field indicates that the positioning SIB type is for a specific GNSS. |
| hyperSFN Indicates hyper SFN which increments by one when the SFN wraps around. |
| ims-EmergencySupport Indicates whether the cell supports IMS emergency bearer services for UEs in limited service mode. If absent, IMS emergency call is not supported by the network in the cell for UEs in limited service mode. NOTE 2. |
| intraFreqReselection Used to control cell reselection to intra-frequency cells when the highest ranked cell is barred, or treated as barred by the UE, as specified in TS 36.304 [4]. NOTE 2. |
| multiBandInfoList A list of additional frequency band indicators, as defined in TS 36.101 [42, table 5.5-1] that the cell belongs to. If the UE supports the frequency band in the freqBandIndicator field it shall apply that frequency band. Otherwise, the UE shall apply the first listed band which it supports in the multiBandInfoList field. If E-UTRAN includes multiBandInfoList-v9e0 it includes the same number of entries, and listed in the same order, as in multiBandInfoList (i.e. without suffix). See Annex D for more descriptions. The UE shall ignore the rule defined in this field description if freqBandIndicatorPriority is present and supported by the UE. |
| multiBandInfoList-v10j0 A list of additionalPmax and additionalSpectrumEmission values, as defined in TS 36.101 [42, table 6.2.4-1] for UEs neither in CE nor BL UEs and TS 36.101 [42, table 6.2.4E-1] for UEs in CE or BL UEs, for the frequency bands in multiBandInfoList (i.e. without suffix) and multiBandInfoList-v9e0. If E-UTRAN includes multiBandInfoList-v10j0, it includes the same number of entries, and listed in the same order, as in multiBandInfoList (i.e. without suffix). If E-UTRAN includes multiBandInfoList-v10l0 it includes the same number of entries, and listed in the same order, as in multiBandInfoList-v10j0. |
| plmn-IdentityList List of PLMN identities. The first listed PLMN-Identity is the primary PLMN. NOTE 2. |
| p-Max Value applicable for the cell. If absent the UE applies the maximum power according to the UE capability. NOTE 2. |
| pos-sib-MappingInfo List of the posSIBs mapped to this SystemInformation message. |
| pos-sib-type The positioning SIB type is defined in TS 36.355 [54]. |
| q-QualMin Parameter “Qqualmin” in TS 36.304 [4]. If cellSelectionInfo-v920 is not present, the UE applies the (default) value of negative infinity for Qqualmin. NOTE 1. |
| q-QualMinRSRQ-OnAllSymbols If this field is present and supported by the UE, the UE shall, when performing RSRQ measurements, perform RSRQ measurement on all OFDM symbols in accordance with TS 36.214 [48]. NOTE 1. |
| q-QualMinOffset Parameter “Qqualminoffset” in TS 36.304 [4]. Actual value Qqualminoffset = field value [dB]. If cellSelectionInfo-v920 is not present or the field is not present, the UE applies the (default) value of 0 dB for Qqualminoffset. Affects the minimum required quality level in the cell. |
| q-QualMinWB If this field is present and supported by the UE, the UE shall, when performing RSRQ measurements, use a wider bandwidth in accordance with TS 36.133 [16]. NOTE 1. |
| q-RxLevMinOffset Parameter Qrxlevminoffset in TS 36.304 [4]. Actual value Qrxlevminoffset = field value * 2 [dB]. If absent, the UE applies the (default) value of 0 dB for Qrxlevminoffset. Affects the minimum required Rx level in the cell. |
| sbas-ID The presence of this field indicates that the positioning SIB type is for a specific SBAS. |
| sib-MappingInfo List of the SIBs mapped to this SystemInformation message. There is no mapping information of SIB2; it is always present in the first SystemInformation message listed in the schedulingInfoList list. |
| si-HoppingConfigCommon Frequency hopping activation/deactivation for BR versions of SI messages and MPDCCH/PDSCH of paging. |
| si-Narrowband This field indicates the index of a narrowband used to broadcast the SI message towards BL UEs and UEs in CE, see TS 36.211 [21, 6.4.1] and TS 36.213 [23, 7.1.6]. Field values (1..maxAvailNarrowBands-r13) correspond to narrowband indices (0..[maxAvailNarrowBands-r13-1]) as specified in TS 36.211 [21]. |
| si-RepetitionPattern Indicates the radio frames within the SI window used for SI message transmission. Value everyRF corresponds to every radio frame, value every2ndRF corresponds to every 2 radio frames, and so on. The first transmission of the SI message is transmitted from the first radio frame of the SI window. |
| si-Periodicity, pos-si-Periodicity Periodicity of the SI-message in radio frames, such that rf8 denotes 8 radio frames, rf16 denotes 16 radio frames, and so on. |
| si-posOffset This field, if present and set to true indicates that the SI messages with positioning SIBs are scheduled with an offset compared to the all other SI message. The offset is the shortest configured SI periodicity 80ms. |
| si-posWindowLength Common SI scheduling window for all Positioning SIs. Unit in milliseconds, where ms1 denotes 1 millisecond, ms2 denotes 2 milliseconds and so on. |
| si-TBS This field indicates the transport block size information used to broadcast the SI message towards BL UEs and UEs in CE, see TS 36.213 [23, Table 7.1.7.2.1-1] for a 6 PRB bandwidth and a QPSK modulation. |
| schedulingInfoList-BR Indicates additional scheduling information of SI messages for BL UEs and UEs in CE. It includes the same number of entries, and listed in the same order, as in schedulingInfoList (without suffix). |
| si-ValidityTime Indicates system information validity timer. If set to TRUE, the timer is set to 3h, otherwise the timer is set to 24h. |
| si-WindowLength, si-WindowLength-BR Common SI scheduling window for all SIs. Unit in milliseconds, where ms1 denotes 1 millisecond, ms2 denotes 2 milliseconds and so on. In case si-WindowLength-BR-r13 is present and the UE is a BL UE or a UE in CE, the UE shall use si-WindowLength-BR-r13 and ignore the original field si-WindowLength (without suffix). UEs other than BL UEs or UEs in CE shall ignore the extension field si-WindowLength-BR-r13. |
| startSymbolBR For BL UEs and UEs in CE, indicates the OFDM starting symbol for any MPDCCH, PDSCH scheduled on the same cell except the PDSCH carrying SystemInformationBlockType1-BR, see TS 36.213 [23]. Values 1, 2, and 3 are applicable for dl-Bandwidth greater than 10 resource blocks. Values 2, 3, and 4 are applicable otherwise. |
| systemInfoValueTagList Indicates SI message specific value tags for BL UEs and UEs in CE. It includes the same number of entries, and listed in the same order, as in schedulingInfoList (without suffix). |
| systemInfoValueTagSI SI message specific value tag as specified in subclause 5.2.1.3. Common for all SIBs within the SI message other than MIB, SIB1, SIB10, SIB11, SIB12 and SIB14. |
| systemInfoValueTag Common for all SIBs other than MIB, MIB-MBMS, SIB1, SIB1-MBMS, SIB10, SIB11, SIB12 and SIB14. Change of MIB, MIB-MBMS, SIB1 and SIB1-MBMS is detected by acquisition of the corresponding message. |
| tdd-Config Specifies the TDD specific physical channel configurations. NOTE 2. |
| trackingAreaCode A trackingAreaCode that is common for all the PLMNs listed. NOTE2. NOTE 5. |

NOTE 1:	The value the UE applies for parameter “Qqualmin” in TS 36.304 [4] depends on the q-QualMin fields signalled by E-UTRAN and supported by the UE. In case multiple candidate options are available, the UE shall select the highest priority candidate option according to the priority order indicated by the following table (top row is highest priority).

| q-QualMinRSRQ-OnAllSymbols | q-QualMinWB | Value of parameter “Qqualmin” in TS 36.304 [4] |
| --- | --- | --- |
| Included | Included | q-QualMinRSRQ-OnAllSymbols – (q-QualMin – q-QualMinWB) |
| Included | Not included | q-QualMinRSRQ-OnAllSymbols |
| Not included | Included | q-QualMinWB |
| Not included | Not included | q-QualMin |

NOTE 2:	E-UTRAN sets this field to the same value for all instances of SIB1 message that are broadcasted within the same cell.

NOTE 3:	E-UTRAN configures this field only in the BR version of SIB1 message.

NOTE 4:	E-UTRAN configures at most 6 PLMNs in total (i.e. across all the PLMN lists in SIB1).

NOTE 5:	E-UTRAN configures only one value for this parameter per PLMN.

| Conditional presence | Explanation |
| --- | --- |

The below change is only applicable if new SIB for Positioning scheduling information has been agreed.

## 6.3	RRC information elements

### 6.3.1	System information blocks

#### PositioningSchedulingInformationBlockType

The IE PositioningSchedulingInformationBlockType contains scheduling information for Positioning SIBs.

PositioningSchedulingInformationBlockType message

-- ASN1START

-- ASN1START

PositioningSchedulingInformationBlockType-r15 ::=	SEQUENCE {

pos-SchedulingInfoList-r15 	Pos-SchedulingInfoList-r15,

si-posOffset-r15			ENUMERATED {true}					OPTIONAL,	-- Need OP

si-posWindowLength-r15		ENUMERATED {ms1, ms2, ms5, ms10, ms15,  ms20, ms40}

}

Pos-SchedulingInfoList-r15 ::= SEQUENCE (SIZE (1..maxSI-Message)) OF Pos-SchedulingInfo-r15

Pos-SchedulingInfo-r15 ::=	SEQUENCE {

pos-si-Periodicity-r15		ENUMERATED {rf8, rf16, rf32, rf64, rf128, rf256, rf512},

pos-sib-MappingInfo-r15		Pos-SIB-MappingInfo-r15

}

Pos-SIB-MappingInfo-r15 ::= SEQUENCE (SIZE (1..maxSIB)) OF Pos-SIB-Type-r15

Pos-SIB-Type-r15 ::= SEQUENCE {

encrypted-r15		ENUMERATED { true }				OPTIONAL,		-- Need OP

gnss-id-r15				GNSS-ID-r15						OPTIONAL,		-- Need OP

sbas-id-r15				SBAS-ID-r15						OPTIONAL,		-- Need OP

pos-sib-type-r15		ENUMERATED { 	posSibType1-1,

posSibType1-2,

posSibType1-3,

posSibType1-4,

posSibType1-5,

posSibType1-6,

posSibType1-7,

posSibType2-1,

posSibType2-2,

posSibType2-3,

posSibType2-4,

posSibType2-5,

posSibType2-6,

posSibType2-7,

posSibType2-8,

posSibType2-9,

posSibType2-10,

posSibType2-11,

posSibType2-12,

posSibType2-13,

posSibType2-14,

posSibType2-15,

posSibType2-16,

posSibType2-17,

posSibType2-18,

posSibType2-19,

posSibType3-1,

... }

}

-- ASN1STOP

PositioningSchedulingInformationBlockType field descriptions

encrypted The presence of this field indicates that the pos-sib-type is encrypted as specified in TS 36.355 [54].

gnss-ID The presence of this field indicates that the positioning SIB type is for a specific GNSS.

cellAccessRelatedInfoList This field contains a list allowing signalling of access related information per PLMN. One PLMN can be included in only one entry of this list. NOTE 4.

pos-si-Periodicity Periodicity of the posSI-message in radio frames, such that rf8 denotes 8 radio frames, rf16 denotes 16 radio frames, and so on.

pos-sib-MappingInfo List of the posSIBs mapped to this SystemInformation message.

pos-sib-type The positioning SIB type is defined in TS 36.355 [54].

sbas-ID The presence of this field indicates that the positioning SIB type is for a specific SBAS.

si-posOffset This field, if present and set to true indicates that the SI messages with positioning SIBs are scheduled with an offset compared to the all other SI message. The offset is the shortest configured SI periodicity 80ms.

si-posWindowLength Common SI scheduling window for all Positioning SIs. Unit in milliseconds, where ms1 denotes 1 millisecond, ms2 denotes 2 milliseconds and so on.