3GPP TSG-RAN WG2 Meeting #103	R2-1812790

Gothenburg, Sweden, 20th - 24th August 2018

Agenda Item: 	10.2.2.1

Source:	Apple

Title:	Clarification on LCH-to-cell restriction

Document for:		Discussion and Decision

# Introduction

In last RAN2 meeting, it was agreed to extend LCH-to-cell restriction as a general functionality, not limit it in duplication.

Agreements

1	LCH-to-cell restriction is not restricted to be only applicable to LCH(s) associated with a radio bearer configured for duplication.

2	Add a capability for the LCH-to-cell restriction. Needs to be supported if duplication is supported.

This contribution gives our further clarification on this feature.

# Discussion

According to current ASN.1 (copied below), allowedServingCells is configured per LCH.

Observaiton 1: One LCH has one LCH-to-cell restriction configuration.

At first, LCH-to-cell restriction is introduced by CA duplication, and the purpose is to avoid the duplicated packets transmitting on the same carrier when the duplication is activated.

Observation 2: In CA duplication, LCH-to-cell restriction is applied when CA duplication is activated.

With the latest agreement, the restriction is extended to a general functionality, and the potential purpose is to link the transmission of some services to the serving cell that provides different QoS garantuees. In addition to activated CA duplication, this restriction can be applied in the case of non-duplication, EN-DC duplication (NR leg), and deactivated CA duplication.

Following is our understanding on each extended use case.

- Use case 1: non-duplication

- For one non-duplicated DRB, if LCH-to-cell restriction is configured on one LCH by RRC, the restriction will be applied on the data transmission of this LCH, until the restriction is deconfigured by RRC.

- Use case 2: EN-DC duplication (NR leg)

- For EN-DC duplicated DRB, if LCH-to-cell restriction is configured on one LCH in NR leg by RRC, the restriction will be applied on the LCH regardless of the duplication activated or deactivated. It is different from the CA duplication since the purpose of the restriction is different.

- Use case 3: deactivated CA duplication

- In current ASN.1, one LCH have only one LCH-to-cell restriction configuration. If it is allowed to apply the different restrictions in duplication deactivated state and activated state, one LCH has to have two LCH-to-cell restriction configurations. Since we donot see the benefit of such configuration, considering the ASN.1 impact, such configuration should not be supported. If this use case has to be considered, we could consider to keep same restriction configuration for activated and deactivated state, and not disable the restriction when the CA duplication is deactivated.

According to our understanding, we propose

Proposal 1: For non-duplicated DRB, LCH-to-cell restriction should be applied for the LCH during LCP procedure.

Proposal 2: For EN-DC duplicated DRB, LCH-to-cell restriction should be applied for the LCH during LCP procedure, regardless of the duplication activated or not.

Proposal 3: If LCH-to-cell restriction is applied when CA duplication is deactivated, the restriction should be same as that in activated state.

# Text Proposal (38.331)

#### -	LogicalChannelConfig

The IE LogicalChannelConfig is used to configure the logical channel parameters.

LogicalChannelConfig information element

-- ASN1START

-- TAG-LOGICAL-CHANNEL-CONFIG-START

LogicalChannelConfig ::=			SEQUENCE {

ul-SpecificParameters				SEQUENCE {

priority							INTEGER (1..16),

prioritisedBitRate					ENUMERATED {kBps0, kBps8, kBps16, kBps32, kBps64, kBps128, kBps256, kBps512,

kBps1024, kBps2048, kBps4096, kBps8192, kBps16384, kBps32768, kBps65536, infinity},

bucketSizeDuration					ENUMERATED {ms5, ms10, ms20, ms50, ms100, ms150, ms300, ms500, ms1000,

spare7, spare6, spare5, spare4, spare3,spare2, spare1},

allowedServingCells					SEQUENCE (SIZE (1..maxNrofServingCells-1)) OF ServCellIndex					OPTIONAL,	-- Need R

allowedSCS-List						SEQUENCE (SIZE (1..maxSCSs)) OF SubcarrierSpacing							OPTIONAL,	-- Need R

maxPUSCH-Duration					ENUMERATED { ms0p02, ms0p04, ms0p0625, ms0p125, ms0p25, ms0p5, spare2, spare1 }

OPTIONAL,	-- Need R

configuredGrantType1Allowed			ENUMERATED {true}															OPTIONAL,	-- Need R

logicalChannelGroup					INTEGER (0..maxLCG-ID)														OPTIONAL, 	-- Need R

schedulingRequestID					SchedulingRequestId															OPTIONAL,	-- Need R

logicalChannelSR-Mask				BOOLEAN,

logicalChannelSR-DelayTimerApplied	BOOLEAN,

KeepAllowedServingCells			    BOOLEAN,

...

}																													OPTIONAL,	-- Cond UL

...

}

-- TAG-LOGICAL-CHANNEL-CONFIG-STOP

-- ASN1STOP

LogicalChannelConfig field descriptions

allowedSCS-List If present, UL MAC SDUs from this logical channel can only be mapped to the indicated numerology. Otherwise, UL MAC SDUs from this logical channel can be mapped to any configured numerology. Only the values 15/30/60 KHz (for FR1) and 60/120 KHz (for FR2) are applicable. Corresponds to ‘allowedSCS-List’ as specified in TS 38.321 [3].

allowedServingCells If present, UL MAC SDUs from this logical channel can only be mapped to the serving cells indicated in this list. Otherwise, UL MAC SDUs from this logical channel can be mapped to any configured serving cell of this cell group. Corresponds to 'allowedServingCells' in TS 38.321 [3]. This field is applied for logical channels belonging to the radio bearer disabled for pdcp-Duplication and for the logical channels belonging to the different cellGroup being associated with a radio bearer enabled for pdcp-Duplication.

bucketSizeDuration Value in ms. ms5 corresponds to 5ms, ms10 corresponds to 10ms, and so on.

configuredGrantType1Allowed If present, UL MAC SDUs from this logical channel can be transmitted on a configured grant type 1. Corresponds to 'configuredGrantType1Allowed' in TS 38.321 [3].

logicalChannelGroup ID of the logical channel group, as specified in TS 38.321 [3], which the logical channel belongs to.

logicalChannelSR-Mask Indicates whether SR masking is configured for this logical channel. See TS 38.321 [3]

logicalChannelSR-DelayTimerApplied Indicates whether to apply the delay timer for SR transmission for this logical channel. Set to FALSE if logicalChannelSR-DelayTimer is not included in BSR-Config.

maxPUSCH-Duration If present, UL MAC SDUs from this logical channel can only be transmitted using uplink grants that result in a PUSCH duration shorter than or equal to the the duration indicated by this field. Otherwise, UL MAC SDUs from this logical channel can be transmitted using an uplink grant resulting in any PUSCH duration. Corresponds to "maxPUSCH-Duration' in TS 38.321 [3].

priority Logical channel priority, as specified in TS 38.321 [3].

prioritisedBitRate Value in kiloBytes/s. 0kBps corresponds to 0, 8kBps corresponds to 8 kiloBytes/s,16 kBps corresponds to 16 kiloBytes/s, and so on. For SRBs, the value can only be set to infinity.

schedulingRequestId If present, it indicates the scheduling request configuration applicable for this logical channel, as specified in TS 38.321 [3].

KeepAllowedServingCells Indicates whether allowedServingCells is applied for this logical channel when duplication is deactivated. See TS 38.321 [3]

# Text Proposal (38.321)

## 5.10	Activation/Deactivation of PDCP duplication

If one or more DRBs are configured with PDCP duplication, the network may activate and deactivate the PDCP duplication for the configured DRB(s).

The PDCP duplication for the configured DRB(s) is activated and deactivated by:

-	receiving the Duplication Activation/Deactivation MAC CE described in subclause 6.1.3.11.

The MAC entity shall for each DRB configured with PDCP duplication:

1>	if a Duplication Activation/Deactivation MAC CE is received activating the PDCP duplication of the DRB:

2>	indicate the activation of PDCP duplication of the DRB to upper layers;

2>	apply the allowedServingCells to the logical channels of the DRB.

1>	if a Duplication Activation/Deactivation MAC CE is received deactivating the PDCP duplication of the DRB:

2>	indicate the deactivation of PDCP duplication of the DRB to upper layers;

2>	If KeepAllowedServingCells is configured by upper layers, not apply the allowedServingCells to the logical channels of the DRB.

# Conclusion

Proposal 1: For non-duplicated DRB, LCH-to-cell restriction should be applied for the LCH during LCP procedure.

Proposal 2: For EN-DC duplicated DRB, LCH-to-cell restriction should be applied for the LCH during LCP procedure, regardless of the duplication activated or not.

Proposal 3: If LCH-to-cell restriction is applied when CA duplication is deactivated, the restriction should be same as that in activated state.

Proposal 4: adopt the corresponding TPs.