3GPP TSG-RAN WG2 Meeting #103						R2-1812986

Gothenburg, Sweden, 20th - 24th August 2018

Title:	Reply LS on cell selection issue in idle mode for 5G NR HPUE

Response to:	R4-1808203

Release:	Rel-15

Work Item:	NR_newRAT-Core

Source:	RAN WG2

To:	RAN WG4

Cc:

Contact Person:

Name:	Xiaoran ZHANG

E-mail Address:	zhangxiaoran@chinamobile.com

1. Overall Description:

RAN2 would like to thank RAN4 for the LS on cell selection issue in idle mode for 5G NR HPUE (R4-1808203). At RAN2#103 meeting, RAN2 discussed the identified issue and following potential solutions in RAN4 LS:

- Alt.1: Resolve this issue in RAN2 under the assumption that HPUE shall assume 23dBm of output power (i.e. default  PC) at maximum if gNB does NOT signal any Pmax value

- Alt.2: PC2 UE can assume 26dBm of output power at maximum if gNB does NOT signal any Pmax value at least for band n41, n77, n78 and n79

The conclusions are:

- Alt. 1 has RAN2 impact and RAN2 did not reach any consensus on the solution to resolve the issue raised in RAN4 LS.

- Alt. 2 has no RAN2 impact and is feasible from RAN2 perspective.

- RAN2 agreed to capture in 38.331 that when p-Max is absent in SIB2 or SIB4 that UE applies the maximum power according to TS 38.101.

2. Actions:

To RAN2 WG:

RAN2 kindly asks RAN4 to take the above information into consideration.

3. Date of Next RAN2 Meetings:

TSG-RAN2 Meeting #103bis	October 08 – 12, 2018		Chengdu, CN

TSG-RAN2 Meeting#104 	November 12 – 16, 2018		Spokane, US