3GPP TSG-RAN WG2 Meeting #103	R2-1813038

Gothenburg, Sweden, 20th - 24th August 2018

Agenda Item: 	10.2.2.1

Source:	Rapporteur (Apple)

Title:	Summary of offline 006 - Clarification on LCH-to-cell restriction

Document for:		Discussion and Decision

# Introduction

This contribution reports the conclusions of the following offline from RAN2#103 - 006, UP session:

=>	Offline discussion to a/ understand the full implications of the decision taken at the last meeting b/ whether the non-duplication restriction can also be applied when duplication bearer when duplication is deactivated (Apple, Offline discussion 06)

# Discussion

According to current ASN.1 (copied below), allowedServingCells is configured per LCH.

Observaiton 1: One LCH has one LCH-to-cell restriction configuration.

LCH-to-cell restriction was initially introduced by CA duplication, and the purpose is to avoid the duplicated packets transmitting on the same carrier when the duplication is activated.

Observation 2: In current specification, LCH-to-cell restriction is applied when CA duplication is activated.

In last RAN2 meeting, following agreements were achieved to extend the use case of LCH-to-cell restriction, not limited in CA duplication, and the potential purpose is to link the transmission of different services to the different serving cells that provide different QoS garantuees.

Agreements

1	LCH-to-cell restriction is not restricted to be only applicable to LCH(s) associated with a radio bearer configured for duplication.

2	Add a capability for the LCH-to-cell restriction. Needs to be supported if duplication is supported.

According to the agreement in last meeting, in order to apply the LCH-to-cell restriction to other use cases than activated CA duplication, further clarification is required.

## Q1: Do companies agree that further clarification on the LCH-to-cell restriction is needed?

| Company | Yes/No | Comments |
| --- | --- | --- |
| Qualcomm | Yes | It may be needed to clarify the handling of CA duplication deactivation case since it was already agreed to lift the restriction when deactivation happened, which is not the same case as the generalized LCH-to-cell restriction. |
| OPPO | Yes |  |
| MediaTek | Yes | It will be good to clarify that LCH to cell restrictions for non-duplication case are configured and activated/deactivated via RRC only. |
| LG | Yes | We think following cell restrictions may be used. - Cell restriction for non-duplicated bearer - Cell restriction for CA duplication bearer - Cell restriction for DC duplication bearer In our understanding, those three cell restriction can be used like below: - Cell restriction for non-duplicated bearer can be configured and this restriction is not related to activation/deactivation of packet duplication. - Cell restriction for CA duplication bearer can be configured and this restriction should be applied only after CA duplication is activated. If CA duplication is deactiveated, cell restriction for CA duplication should be lifted. Cell restriction for DC duplication bearer can be treated same as cell restriction for non-duplicated bearer. This means that cell restriction for DC duplication bearer, if configured, should be applied regardless of whether packet duplication is activated or deactivated because, unlike CA duplication, different MAC entity is used to transmit the duplicated PDU. |
| Sharp | Yes | Base on the agreement last meeting , LCH-to-cell restriction could be used for different use cases,and hence further clarification would be helpful. |
| Ericsson | Yes | The In-Principal agreed changes would address most needed changes apart from the case of a CA duplicated bearers for where duplication is deactivated etc. |
| ZTE | Yes |  |
| Lenovo/ MotM | Yes | We agree with the reasoning given by Qualcomm |
| NTT DOCOMO | Yes |  |
| Huawei | Yes | The use cases of this LCH-to-cell restriction have impact on whether to apply this parameter after duplication deactivation. |
| Samsung | Yes |  |
| Nokia | ? | Our understanding of the current specification is that for duplication case, the restrictions are configured by RRC and controlled by MAC, whereas for non-duplication case, it is configured and controlled by RRC. |
| Fujitsu | Yes | It alignes with the previous agreement. |
| Apple | Yes |  |
| CATT | Yes | At least clarify the case of LCH-to-cell restriction in DC duplication |

Summary: Almost all companies agree that further clarification on the LCH-to-cell restriction is needed.

If the ansqwer to Q1 is yes, there are three potential use cases for discussion.

- Use case 1: non-duplicated bearer [1]

- For one non-duplicated bearer, if LCH-to-cell restriction is configured on the LCH by RRC, the restriction will be applied on the data transmission of this LCH, until the restriction is deconfigured by RRC.

## Q2-1: Do companies agree to apply the LCH-to-cell restriction to non-duplicated bearer if configured by RRC?

| Company | Yes/No | Comments |
| --- | --- | --- |
| Qualcomm | Yes | We do not see a strong use case but we are okay to follow the previous agreement if other companies want to generalize the LCH-to-cell restriction to non-duplicated bearer. |
| OPPO | Yes | In our understanding, this is the intention of the previous agreement. |
| MediaTek | Yes |  |
| LG | Yes | As commented in Q1, cell restriction for non-duplicated bearer can be configured and this restriction is not related to activation/deactivation of packet duplication. |
| Sharp | Yes | Match the previous agreement. |
| Ericsson | Yes | As agreed |
| ZTE | Yes |  |
| Lenovo/ MotM | Yes | Agreement was already made in the last meeting. |
| NTT DOCOMO | Yes |  |
| Huawei | Yes | This is a clear and normal use case, when we extend this restrition to be used in non-duplication purpose. |
| Samsung | Yes |  |
| Nokia | Yes | We do not see the need to rediscuss this. |
| Fujitsu | Yes | It alignes with the previous agreement. |
| Apple | Yes |  |
| CATT | Yes |  |

Summary: All companies agree to apply the LCH-to-cell restriction to non-duplicated bearer if configured by RRC.

- Use case 2: EN-DC split bearer in NR leg

- For EN-DC split bearer, if LCH-to-cell restriction is configured on the LCH in NR leg by RRC, the restriction will be applied on the LCH.

## Q2-2: Do companies agree to apply the LCH-to-cell restriction to the NR leg of EN-DC split bearer if configured by RRC?

| Company | Yes/No | Comments |
| --- | --- | --- |
| Qualcomm | Yes | Yes for non-duplication split bearer only. |
| OPPO | Yes but open to discuss |  |
| MediaTek | Yes | This is true only if the bearer is not configured for duplication. |
| LG | Yes/No | Cell restriction for EN-DC split bearer can be treated same as cell restriction for non-duplicated bearer as commented in Q1. |
| Sharp | Yes | It aligns with the intention of the previous agreement. |
| Ericsson | Yes | Agree with LG on the handling |
| ZTE | Yes | The LCH-to-cell restriction can be configured to all kinds of LCHs in NR side. |
| Lenovo/ MotM | Yes | This was the agreement from last meeting (even though this is not our preference). Also, as in the RRC specification, the cell restrictions can be applied to any LCH irrespective of if this LCH is a part of a duplicated bearer or a split bearer or otherwise as long as the concerned PDCP is an NR PDCP |
| NTT DOCOMO | Yes |  |
| Huawei | Yes | Same view in Q2-1. |
| Samsung | Yes |  |
| Nokia | Yes |  |
| Fujitsu | Yes | It alignes with the previous agreement. |
| Apple | Yes |  |
| CATT | Yes |  |

Summary: All companies agree to apply the LCH-to-cell restriction to the NR leg of EN-DC split bearer if configured by RRC.

- Use case 3: EN-DC duplicated bearer in NR leg [1][5]

- For EN-DC duplicated bearer, if LCH-to-cell restriction is configured on the LCH in NR leg by RRC, the restriction will be applied on the LCH when the duplication is activated.

## Q2-3a: Do companies agree to apply the LCH-to-cell restriction to the NR leg of EN-DC duplicated bearer when DC duplication is activated?

| Company | Yes/No | Comments |
| --- | --- | --- |
| Qualcomm | No | There is no need to change the existing behavior. |
| OPPO | Yes | It can be up to network configuration whether the cell restriction is applied to this case or not, i.e., an indication can be added besides the allowedServingCells |
| MediaTek | No | There is no clear use case for this kind of behavior. In our understanding, duplicated bearers in DC case should be not be restricted from any transmission opportunity. There is no clear use case for changing this behavior. |
| LG | Yes/No | As commented in Q1, we think that cell restriction only for DC duplication bearer is not needed. If there is cell restriction for DC duplication bearer, this should be treated same as cell restriction for non-duplicated bearer. This means that cell restriction should be applied regardless of whether packet duplication is activated or deactivated. |
| Sharp | Yes | If the EN-DC duplicatied bearer is considered as the bearer configured with duplication. It could be either LTE PDCP or NR PDCP If it is LTE PDCP, LCH to cell restriction could be applied in NR leg not for NR duplication but for NR cell restriction If it is NR PDCP, LCH to cell restriction could be applied in NR leg for both NR duplication and NR cell restriction. |
| Ericsson | Yes/No | Agree with LG |
| ZTE | Yes | The LCH-to-cell restriction can be configured to all kinds of LCHs in NR side. |
| Lenovo/ MotM | Yes | This was the agreement from last meeting (even though this is not our preference). Also, as in the RRC specification, the cell restrictions can be applied to any LCH irrespective of if this LCH is a part of a duplicated bearer or a split bearer or otherwise as long as the concerned PDCP is an NR PDCP |
| NTT DOCOMO | Yes |  |
| Huawei | Yes | No need to exclude this use case. |
| Samsung | Yes | We don’t see any big problem for allowing this as not allowing this would restrict NW flexibility. |
| Nokia | Yes | We would like to avoid unnecessary limitations which would resut in a more complex handling. |
| Fujitsu | Yes | It alignes with the previous agreement. And we agree with LGE – there is no need to have restrict separation. |
| Apple | Yes |  |
| CATT | Yes | Cell restriction implements duplication for CA duplication only. Hence, if configured for an LCH of a DC duplication bearer, it can be treated as for a non-duplicated bearer. That is, it applies irrespective of duplication activation / deactivation. And this comes for free, no new parameter needs to be introduced. |

## Q2-3b: Do companies agree to apply the LCH-to-cell restriction to the NR leg of EN-DC duplicated bearer when DC duplication is deactivated?

| Company | Yes/No | Comments |
| --- | --- | --- |
| Qualcomm | No | Same answer as in Q2-2. There is no need to change the existing behavior. |
| OPPO | Yes | See above |
| MediaTek | No | We see no clear reason to deviate from existing specification. |
| LG | Yes/No | As commented in Q1, we think that cell restriction only for DC duplication bearer is not needed. If there is cell restriction for DC duplication bearer, this should be treated same as cell restriction for non-duplicated bearer. This means that cell restriction should be applied regardless of whether packet duplication is activated or deactivated. |
| Sharp | Yes | When PDCP duplication deactivated, EN-DC duplicated bearer is just like a split bearer, the application of LCH-to-cell could be decided by network. It could be an option to apply the LCH-to-cell retriction. |
| Ericsson | Yes/No | Agree with LG |
| ZTE | Yes | The LCH-to-cell restriction can be configured to the NR leg of EN-DC no matter the duplication is activated or not. |
| Lenovo/ MotM | Yes | This was the agreement from last meeting (even though this is not our preference). Also, as in the RRC specification, the cell restrictions can be applied to any LCH irrespective of if this LCH is a part of a duplicated bearer or a split bearer or otherwise as long as the concerned PDCP is an NR PDCP |
| NTT DOCOMO | Yes |  |
| Huawei | Yes | For EN-DC duplicated bearer, there is no reason to configure the LCH-to-cell restriction for duplication purpose. So, this parameter can be configured for non-duplciation purpose. Then, even if the duplication is deactivated, this restriction shall be kept. |
| Samsung | Yes |  |
| Nokia | Yes | We would like to avoid unnecessary limitations which would resut in a more complex handling. |
| Fujitsu | Yes | It alignes with the previous agreement. And we agree with LGE – there is no need to have restrict separation. |
| Apple | Yes |  |
| CATT | Yes | See Q2-3a |

Summary: Majority agrees to apply the LCH-to-cell restriction to the NR leg of EN-DC duplicated bearer regardless of the duplication activated or deactivated.

- Use case 4: CA duplicated bearer when the duplication deactivated [1][2][4]

- If the restriction on one LCH is applicable for both CA duplication activated and deactivated, there are two options:

  - Option 1: Keep same restriction for one LCH regardless of CA duplication activated or deactivated. (The ASN.1 impact is to introduce a parameter Indicates whether allowedServingCells is applied for this logical channel when duplication is deactivated.)

  - Option 2: Allow different restriction for one LCH for CA duplication activated and deactivtation. (The ASN.1 impact is to introduce one additional restriction configuration for this logical channel for CA duplication deactivated state.)

## Q2-4a: Do companies agree to apply the LCH-to-cell restriction for one LCH when CA duplication is deactivated?

| Company | Yes/No | Comments |
| --- | --- | --- |
| Qualcomm | No | The simplest solution is option 3. As previously agreed: if a bearer is configured as CA duplication bearer, at deactivation the previously configured LCH-to-cell restriction is lifted to allow the bearer to fully utilize all carriers. This condition can be clarified in RRC spec in field description but there is no need to introduce any additional parameter. In other words, the LCH-to-cell restriction applies in two cases: (1) for CA duplication bearer, when duplication is activated. (2) for non-duplication bearer. |
| OPPO | Yes | In the last RAN2 AH#1807 meeting, in the main session, it has been agreed: R2-1809963 cell restriction during LCP Lenovo, Motorola Mobility, Qualcomm discussion Rel-15 NR_newRAT-Core - Huawei have the same understanding and support the proposal. MediaTek also have the same view. - Nokia think this can be used a general functionality and there are good use cases for it. Ericsson have the same view as Nokia and it will also be useful in unlicensed. DOCOMO also agree and the stage 3 has specified this an independent functionality. ZTE also agree. Vivo think it should be general functionality. LG also agree. Samsung also agree. - Qualcomm see no use case apart from duplication. Agreements 1 LCH-to-cell restriction is not restricted to be only applicable to LCH(s) associated with a radio bearer configured for duplication. 2 Add a capability for the LCH-to-cell restriction. Needs to be supported if duplication is supported. In the UP session, the CR R2-1810181 is agreed in principle based on the previous agreement that the cell retriction is not applied when duplication is deactivated. During the online discussion, lots of companies see benefits for introducing the cell restriction to be used as a general functionality other than only in duplication, e.g., MDBV or unlicensed operation. However, given the latest CR, it seems for the DRB configured with CA duplication, the benefits are gone when the duplication is deactivated since the cell restriction is not applied anymore unless the DRB is reconfigured. In order to make the benefits of cell restriction equal to all DRBs (no matter whether CA duplication is configured or not), and also to make less network reconfiguration when cell restriction is still needed when CA duplication is deactivated, it’s better to indicate the UE whether to apply the cell restriction or not when the duplication is deactivated. |
| MediaTek | No | Agree with Qualcomm’s comment. In this late stage of the Release we prefer not to make such changes without adequate discussion. We think this issue can be discussed in Rel-16 if needed. |
| LG | No | Agree with QC. We also think that option 3 should be considred. If cell restriction is configured for CA duplication bearer, cell restriction should be lifted after CA duplication is deactivated as commented in Q1. |
| Sharp | Yes | For a bearer configured with CA duplication, when duplication is deactivated, it could be considered as a non-duplication bearer. From this perspective, LCH-to-cell restriction could be applied to both. |
| Ericsson | Yes | This point is remaining to decide upon. Preference to make a complete restriction function to resolve the non-restricted LCH at duplication deactivation. |
| ZTE | Yes | We prefer to have a separate IE “allowedServingCells-NonDuplication” to provide the LCH-to-cell restriction in non-CA duplication cases (i.e. the original IE will still be used for the purpose of CA duplication). |
| Lenovo/ MotM | Yes | This way we can avoid frequent reconfiguration, i.e. configuration of LCH-to cell restriction when duplication was deactivated by MAC CE, of a duplicated bearer if the network would anyway like to restrict the bearer to be served by certain cell/ carrier only. |
| NTT DOCOMO | Yes/No | No preference |
| Huawei | Yes | For EN-DC duplicated bearer, there is no reason to configure the LCH-to-cell restriction for duplication purpose. So, this parameter can be configured for non-duplciation purpose. Then, even if the duplication is deactivated, this restriction shall be kept. |
| Samsung | No | If NW wants to keep the cell restriction at deactivation, RRC configuration can be used. Thus, we don’t think either of two options proposed above are needed. |
| Nokia | No | Based on our understanding of the current spec (see Q1), we agree with Qualcomm. We should avoid ASN.1 impacts at this late stage. |
| Fujitsu | No | Agree with QC – we understood that configured LCH-to-cell restriction is lifted. |
| Apple | Yes |  |
| CATT | No | We are reluctant to introduce a new RRC parameter for this usecase. |

Summary: There is no consensus on whether to apply the restriction when CA duplication is deactivated.

## Q2-4b: If the answer to Q2-4a is Yes, which option is your preference, Option 1 or Option 2?

| Company | Option# | Comments |
| --- | --- | --- |
| Qualcomm | 3 | See comment in Q2-4a |
| OPPO | 1 | See R2-1811753 and R2-1811754 |
| LG | 3 | See comment in Q2-4a |
| Sharp | 1 | See comment in Q2-4a |
| Ericsson | 2 |  |
| ZTE | 2 |  |
| Lenovo/ MotM | 1 or 2 | Our preference would be option 2, since it has the benefit that gNB could configure different cell restrictions for the duplication case and for the case when duplication is deactivated. As cell restriction for duplication is done to ensure that the same PDCP PDU doesn’t end up twice on the same carrier/cell whereas cell restriction for the non-duplication case is mainly motivated by QoS/scheduling, it makes sense to allow having a different configuration for such both cases. Option 1 would be also acceptable. |
| NTT DOCOMO | 1?? | We prefer to avoid additional ASN.1 impact as much as possible, i.e. the MAC can maintain the same restriction even when packet duplication is deactivated. |
| Huawei | 1 or 2 | if needed, both options have the similar use cases. We can go with either of the options. |
| Fujitsu | 3 | We understood that configured LCH-to-cell restriction is lifted. |
| Apple | 1 or 2 |  |

Summary: If apply restriction in CA duplication deactivates state, there is no consensus on whether the same or differnet restriction is applied in deactivated state.

## Q3: Do companies propose any new use case else or have any comments?

| Company | Comments |
| --- | --- |
| OPPO | We think the discussion for the EN-DC case can be extended to other DC case, e.g., NR-NR DC, which is generalized to DC based duplication. |
| Huawei | Similar view with OPPO |

Summary: The conclusion for EN-DC case can be applied in NR-DC and MR-DC.

# Conclusion

During the offline discussion, all companies would like to clarify that the restriction can be applied on non-duplicated bearer and EN-DC duplicated/split bearer. There is no consensus on whether to apply the restriction when CA duplication is deactivated.

During the offline discussion, one company indicated that there is no use case discussed in RAN2 (and maybe none exists) for configuring cell restrictions for DC duplication.

Based on the offline discussion comments, the discussion rapporteur suggests the following as way forward for RAN2:

Proposal 1: The LCH-to-cell restriction should be further clarified to apply on the LCH in the follow cases:

- Non-duplicated bearer;

- NR leg of EN-DC split bearer;

- NR leg of EN-DC duplicated bearer, for both dupliation activated and deactivated state.

Proposal 2: When CA duplication is deactivated, RAN2 is proposed to decide which option should be adopted.

- Option 1: Apply same restriction for CA duplication activated and deactivated state.

- Option 2: Apply different restriction for CA duplication activated and deactivated state.

- Option 3: Not apply the restriction for CA duplication deactivated state.

Proposal 3: LCH-to-cell restriction can be applied on NR-NR DC split/duplicated bearer.

The corresponding TPs for 38.331, 38.321, 38.300 for 3 options are proposed below.

# Reference

R2-1812790	Clarification on LCH-to-cell restriction				Apple		Disc

R2-1811753	CR on cell restriction in LCP for PDCP duplication	OPPO		draftCR	38.331

R2-1811754 CR on cell restriction in LCP for PDCP duplication	OPPO	CR 38.321

R2-1812208	Cell restriction for a DRB configured for duplication	Lenovo, Motorola Mobility	discussion

R2-1811381	Further consideration on LCH-to-Cell restriction	Huawei, HiSilicon	discussion

R2-1811382	Correction on LCH-to-Cell restriction	Huawei, HiSilicon	CR	Rel-15	38.321

# Option 1: Apply same restriction in deactivated state

## 5.1. Text Proposal (38.331)

#### -	LogicalChannelConfig

The IE LogicalChannelConfig is used to configure the logical channel parameters.

LogicalChannelConfig information element

-- ASN1START

-- TAG-LOGICAL-CHANNEL-CONFIG-START

LogicalChannelConfig ::=			SEQUENCE {

ul-SpecificParameters				SEQUENCE {

priority							INTEGER (1..16),

prioritisedBitRate					ENUMERATED {kBps0, kBps8, kBps16, kBps32, kBps64, kBps128, kBps256, kBps512,

kBps1024, kBps2048, kBps4096, kBps8192, kBps16384, kBps32768, kBps65536, infinity},

bucketSizeDuration					ENUMERATED {ms5, ms10, ms20, ms50, ms100, ms150, ms300, ms500, ms1000,

spare7, spare6, spare5, spare4, spare3,spare2, spare1},

allowedServingCells					SEQUENCE (SIZE (1..maxNrofServingCells-1)) OF ServCellIndex					OPTIONAL,	-- Need R

allowedSCS-List						SEQUENCE (SIZE (1..maxSCSs)) OF SubcarrierSpacing							OPTIONAL,	-- Need R

maxPUSCH-Duration					ENUMERATED { ms0p02, ms0p04, ms0p0625, ms0p125, ms0p25, ms0p5, spare2, spare1 }

OPTIONAL,	-- Need R

configuredGrantType1Allowed			ENUMERATED {true}															OPTIONAL,	-- Need R

logicalChannelGroup					INTEGER (0..maxLCG-ID)														OPTIONAL, 	-- Need R

schedulingRequestID					SchedulingRequestId															OPTIONAL,	-- Need R

logicalChannelSR-Mask				BOOLEAN,

logicalChannelSR-DelayTimerApplied	BOOLEAN,

KeepAllowedServingCells			    BOOLEAN,

...

}																													OPTIONAL,	-- Cond UL

...

}

-- TAG-LOGICAL-CHANNEL-CONFIG-STOP

-- ASN1STOP

LogicalChannelConfig field descriptions

allowedSCS-List If present, UL MAC SDUs from this logical channel can only be mapped to the indicated numerology. Otherwise, UL MAC SDUs from this logical channel can be mapped to any configured numerology. Only the values 15/30/60 KHz (for FR1) and 60/120 KHz (for FR2) are applicable. Corresponds to ‘allowedSCS-List’ as specified in TS 38.321 [3].

allowedServingCells If present, UL MAC SDUs from this logical channel can only be mapped to the serving cells indicated in this list. Otherwise, UL MAC SDUs from this logical channel can be mapped to any configured serving cell of this cell group. Corresponds to 'allowedServingCells' in TS 38.321 [3]. This field can be applied for logical channels belonging to a radio bearer regardless of the pdcp-Duplication is configured or not.

bucketSizeDuration Value in ms. ms5 corresponds to 5ms, ms10 corresponds to 10ms, and so on.

configuredGrantType1Allowed If present, UL MAC SDUs from this logical channel can be transmitted on a configured grant type 1. Corresponds to 'configuredGrantType1Allowed' in TS 38.321 [3].

logicalChannelGroup ID of the logical channel group, as specified in TS 38.321 [3], which the logical channel belongs to.

logicalChannelSR-Mask Indicates whether SR masking is configured for this logical channel. See TS 38.321 [3]

logicalChannelSR-DelayTimerApplied Indicates whether to apply the delay timer for SR transmission for this logical channel. Set to FALSE if logicalChannelSR-DelayTimer is not included in BSR-Config.

maxPUSCH-Duration If present, UL MAC SDUs from this logical channel can only be transmitted using uplink grants that result in a PUSCH duration shorter than or equal to the the duration indicated by this field. Otherwise, UL MAC SDUs from this logical channel can be transmitted using an uplink grant resulting in any PUSCH duration. Corresponds to "maxPUSCH-Duration' in TS 38.321 [3].

priority Logical channel priority, as specified in TS 38.321 [3].

prioritisedBitRate Value in kiloBytes/s. 0kBps corresponds to 0, 8kBps corresponds to 8 kiloBytes/s,16 kBps corresponds to 16 kiloBytes/s, and so on. For SRBs, the value can only be set to infinity.

schedulingRequestId If present, it indicates the scheduling request configuration applicable for this logical channel, as specified in TS 38.321 [3].

KeepAllowedServingCells Indicates whether allowedServingCells is applied for this logical channel when duplication is deactivated. See TS 38.321 [3].

## 5.2. Text Proposal (38.321)

Note: change is based on in-principle-agreed MAC CR (R2-1812139), and the change on change is marked in yellow.

##### 5.4.3.1.1	General

The Logical Channel Prioritization procedure is applied whenever a new transmission is performed.

RRC controls the scheduling of uplink data by signalling for each logical channel per MAC entity:

-	priority where an increasing priority value indicates a lower priority level;

-	prioritisedBitRate which sets the Prioritized Bit Rate (PBR);

-	bucketSizeDuration which sets the Bucket Size Duration (BSD).

RRC additionally controls the LCP procedure by configuring mapping restrictions for each logical channel:

-	allowedSCS-List which sets the allowed Subcarrier Spacing(s) for transmission;

-	maxPUSCH-Duration which sets the maximum PUSCH duration allowed for transmission;

-	configuredGrantType1Allowed which sets whether a configured grant Type 1 can be used for transmission;

-	allowedServingCells which sets the allowed cell(s) for transmission.

-	KeepAllowedServingCells which indicates whether allowedServingCells is applied to the logical channel.

The following UE variable is used for the Logical channel prioritization procedure:

-	Bj which is maintained for each logical channel j.

The MAC entity shall initialize Bj of the logical channel to zero when the logical channel is established.

For each logical channel j, the MAC entity shall:

1>	increment Bj by the product PBR × T before every instance of the LCP procedure, where T is the time elapsed since Bj was last incremented;

1>	if the value of Bj is greater than the bucket size (i.e. PBR × BSD):

2>	set Bj to the bucket size.

NOTE:	The exact moment(s) when the UE updates Bj between LCP procedures is up to UE implementation, as long as Bj is up to date at the time when a grant is processed by LCP.

##### 5.4.3.1.2	Selection of logical channels

The MAC entity shall, when a new transmission is performed:

- select the logical channels for each UL grant that satisfy all the following conditions:

2>	the set of allowed Subcarrier Spacing index values in allowedSCS-List, if configured, includes the Subcarrier Spacing index associated to the UL grant; and

2>	maxPUSCH-Duration, if configured, is larger than or equal to the PUSCH transmission duration associated to the UL grant; and

2>	configuredGrantType1Allowed, if configured, is set to TRUE in case the UL grant is a Configured Grant Type 1; and

2>	allowedServingCells, if configured, includes the Cell information associated to the UL grant. Does not apply to logical channels associated with a DRB configured with PDCP duplication within same cell group for which PDCP duplication is deactivated if KeepAllowedServingCells is set to FALSE.

NOTE:	The Subcarrier Spacing index, PUSCH transmission duration and Cell information are included in Uplink transmission information received from lower layers for the corresponding scheduled uplink transmission.

# Option 2: Apply different restriction in deactivated state

## 6.1. Text Proposal (38.331)

#### -	LogicalChannelConfig

The IE LogicalChannelConfig is used to configure the logical channel parameters.

LogicalChannelConfig information element

-- ASN1START

-- TAG-LOGICAL-CHANNEL-CONFIG-START

LogicalChannelConfig ::=			SEQUENCE {

ul-SpecificParameters				SEQUENCE {

priority							INTEGER (1..16),

prioritisedBitRate					ENUMERATED {kBps0, kBps8, kBps16, kBps32, kBps64, kBps128, kBps256, kBps512,

kBps1024, kBps2048, kBps4096, kBps8192, kBps16384, kBps32768, kBps65536, infinity},

bucketSizeDuration					ENUMERATED {ms5, ms10, ms20, ms50, ms100, ms150, ms300, ms500, ms1000,

spare7, spare6, spare5, spare4, spare3,spare2, spare1},

allowedServingCells					SEQUENCE (SIZE (1..maxNrofServingCells-1)) OF ServCellIndex					OPTIONAL,	-- Need R

allowedSCS-List						SEQUENCE (SIZE (1..maxSCSs)) OF SubcarrierSpacing							OPTIONAL,	-- Need R

maxPUSCH-Duration					ENUMERATED { ms0p02, ms0p04, ms0p0625, ms0p125, ms0p25, ms0p5, spare2, spare1 }

OPTIONAL,	-- Need R

configuredGrantType1Allowed			ENUMERATED {true}															OPTIONAL,	-- Need R

logicalChannelGroup					INTEGER (0..maxLCG-ID)														OPTIONAL, 	-- Need R

schedulingRequestID					SchedulingRequestId															OPTIONAL,	-- Need R

logicalChannelSR-Mask				BOOLEAN,

logicalChannelSR-DelayTimerApplied	BOOLEAN,

allowedServingCells1				SEQUENCE (SIZE (1..maxNrofServingCells-1)) OF ServCellIndex					OPTIONAL,	-- Need R

...

}																													OPTIONAL,	-- Cond UL

...

}

-- TAG-LOGICAL-CHANNEL-CONFIG-STOP

-- ASN1STOP

LogicalChannelConfig field descriptions

allowedSCS-List If present, UL MAC SDUs from this logical channel can only be mapped to the indicated numerology. Otherwise, UL MAC SDUs from this logical channel can be mapped to any configured numerology. Only the values 15/30/60 KHz (for FR1) and 60/120 KHz (for FR2) are applicable. Corresponds to ‘allowedSCS-List’ as specified in TS 38.321 [3].

allowedServingCells If present, UL MAC SDUs from this logical channel can only be mapped to the serving cells indicated in this list. Otherwise, UL MAC SDUs from this logical channel can be mapped to any configured serving cell of this cell group. Corresponds to 'allowedServingCells' in TS 38.321 [3]. This field can be applied for logical channels belonging to a radio bearer regardless of the pdcp-Duplication is configured or not.

bucketSizeDuration Value in ms. ms5 corresponds to 5ms, ms10 corresponds to 10ms, and so on.

configuredGrantType1Allowed If present, UL MAC SDUs from this logical channel can be transmitted on a configured grant type 1. Corresponds to 'configuredGrantType1Allowed' in TS 38.321 [3].

logicalChannelGroup ID of the logical channel group, as specified in TS 38.321 [3], which the logical channel belongs to.

logicalChannelSR-Mask Indicates whether SR masking is configured for this logical channel. See TS 38.321 [3]

logicalChannelSR-DelayTimerApplied Indicates whether to apply the delay timer for SR transmission for this logical channel. Set to FALSE if logicalChannelSR-DelayTimer is not included in BSR-Config.

maxPUSCH-Duration If present, UL MAC SDUs from this logical channel can only be transmitted using uplink grants that result in a PUSCH duration shorter than or equal to the the duration indicated by this field. Otherwise, UL MAC SDUs from this logical channel can be transmitted using an uplink grant resulting in any PUSCH duration. Corresponds to "maxPUSCH-Duration' in TS 38.321 [3].

priority Logical channel priority, as specified in TS 38.321 [3].

prioritisedBitRate Value in kiloBytes/s. 0kBps corresponds to 0, 8kBps corresponds to 8 kiloBytes/s,16 kBps corresponds to 16 kiloBytes/s, and so on. For SRBs, the value can only be set to infinity.

schedulingRequestId If present, it indicates the scheduling request configuration applicable for this logical channel, as specified in TS 38.321 [3].

allowedServingCells1 If present, UL MAC SDUs from this logical channel can only be mapped to the serving cells indicated in this list if the CA duplication is deactivated. Corresponds to 'allowedServingCells1' in TS 38.321 [3].

## 6.2. Text Proposal (38.321)

##### 5.4.3.1.1	General

The Logical Channel Prioritization procedure is applied whenever a new transmission is performed.

RRC controls the scheduling of uplink data by signalling for each logical channel per MAC entity:

-	priority where an increasing priority value indicates a lower priority level;

-	prioritisedBitRate which sets the Prioritized Bit Rate (PBR);

-	bucketSizeDuration which sets the Bucket Size Duration (BSD).

RRC additionally controls the LCP procedure by configuring mapping restrictions for each logical channel:

-	allowedSCS-List which sets the allowed Subcarrier Spacing(s) for transmission;

-	maxPUSCH-Duration which sets the maximum PUSCH duration allowed for transmission;

-	configuredGrantType1Allowed which sets whether a configured grant Type 1 can be used for transmission;

-	allowedServingCells which sets the allowed cell(s) for transmission except when CA duplication is deactivated;

-	allowedServingCells1 which sets the allowed cell(s) for transmission when CA duplication is deactivated.

The following UE variable is used for the Logical channel prioritization procedure:

-	Bj which is maintained for each logical channel j.

The MAC entity shall initialize Bj of the logical channel to zero when the logical channel is established.

For each logical channel j, the MAC entity shall:

1>	increment Bj by the product PBR × T before every instance of the LCP procedure, where T is the time elapsed since Bj was last incremented;

1>	if the value of Bj is greater than the bucket size (i.e. PBR × BSD):

2>	set Bj to the bucket size.

NOTE:	The exact moment(s) when the UE updates Bj between LCP procedures is up to UE implementation, as long as Bj is up to date at the time when a grant is processed by LCP.

##### 5.4.3.1.2	Selection of logical channels

The MAC entity shall, when a new transmission is performed:

- select the logical channels for each UL grant that satisfy all the following conditions:

2>	the set of allowed Subcarrier Spacing index values in allowedSCS-List, if configured, includes the Subcarrier Spacing index associated to the UL grant; and

2>	maxPUSCH-Duration, if configured, is larger than or equal to the PUSCH transmission duration associated to the UL grant; and

2>	configuredGrantType1Allowed, if configured, is set to TRUE in case the UL grant is a Configured Grant Type 1; and

2>	allowedServingCells, if configured, includes the Cell information associated to the UL grant. Does not apply to logical channels associated with a DRB configured with PDCP duplication within same cell group for which PDCP duplication is deactivated.

2>	allowedServingCells1, if configured, includes the Cell information associated to the UL grant. Apply to logical channels associated with a DRB configured with PDCP duplication within same cell group for which PDCP duplication is deactivated.

NOTE:	The Subcarrier Spacing index, PUSCH transmission duration and Cell information are included in Uplink transmission information received from lower layers for the corresponding scheduled uplink transmission.

# Option 3: Not support restriction in deactivated state

## 7.1. Text Proposal (38.331)

#### -	LogicalChannelConfig

The IE LogicalChannelConfig is used to configure the logical channel parameters.

LogicalChannelConfig information element

-- ASN1START

-- TAG-LOGICAL-CHANNEL-CONFIG-START

LogicalChannelConfig ::=			SEQUENCE {

ul-SpecificParameters				SEQUENCE {

priority							INTEGER (1..16),

prioritisedBitRate					ENUMERATED {kBps0, kBps8, kBps16, kBps32, kBps64, kBps128, kBps256, kBps512,

kBps1024, kBps2048, kBps4096, kBps8192, kBps16384, kBps32768, kBps65536, infinity},

bucketSizeDuration					ENUMERATED {ms5, ms10, ms20, ms50, ms100, ms150, ms300, ms500, ms1000,

spare7, spare6, spare5, spare4, spare3,spare2, spare1},

allowedServingCells					SEQUENCE (SIZE (1..maxNrofServingCells-1)) OF ServCellIndex					OPTIONAL,	-- Need R

allowedSCS-List						SEQUENCE (SIZE (1..maxSCSs)) OF SubcarrierSpacing							OPTIONAL,	-- Need R

maxPUSCH-Duration					ENUMERATED { ms0p02, ms0p04, ms0p0625, ms0p125, ms0p25, ms0p5, spare2, spare1 }

OPTIONAL,	-- Need R

configuredGrantType1Allowed			ENUMERATED {true}															OPTIONAL,	-- Need R

logicalChannelGroup					INTEGER (0..maxLCG-ID)														OPTIONAL, 	-- Need R

schedulingRequestID					SchedulingRequestId															OPTIONAL,	-- Need R

logicalChannelSR-Mask				BOOLEAN,

logicalChannelSR-DelayTimerApplied	BOOLEAN,

...

}																													OPTIONAL,	-- Cond UL

...

}

-- TAG-LOGICAL-CHANNEL-CONFIG-STOP

-- ASN1STOP

LogicalChannelConfig field descriptions

allowedSCS-List If present, UL MAC SDUs from this logical channel can only be mapped to the indicated numerology. Otherwise, UL MAC SDUs from this logical channel can be mapped to any configured numerology. Only the values 15/30/60 KHz (for FR1) and 60/120 KHz (for FR2) are applicable. Corresponds to ‘allowedSCS-List’ as specified in TS 38.321 [3].

allowedServingCells If present, UL MAC SDUs from this logical channel can only be mapped to the serving cells indicated in this list. Otherwise, UL MAC SDUs from this logical channel can be mapped to any configured serving cell of this cell group. Corresponds to 'allowedServingCells' in TS 38.321 [3]. This field can be applied for logical channels belonging to a radio bearer regardless of the pdcp-Duplication is configured or not.

bucketSizeDuration Value in ms. ms5 corresponds to 5ms, ms10 corresponds to 10ms, and so on.

configuredGrantType1Allowed If present, UL MAC SDUs from this logical channel can be transmitted on a configured grant type 1. Corresponds to 'configuredGrantType1Allowed' in TS 38.321 [3].

logicalChannelGroup ID of the logical channel group, as specified in TS 38.321 [3], which the logical channel belongs to.

logicalChannelSR-Mask Indicates whether SR masking is configured for this logical channel. See TS 38.321 [3]

logicalChannelSR-DelayTimerApplied Indicates whether to apply the delay timer for SR transmission for this logical channel. Set to FALSE if logicalChannelSR-DelayTimer is not included in BSR-Config.

maxPUSCH-Duration If present, UL MAC SDUs from this logical channel can only be transmitted using uplink grants that result in a PUSCH duration shorter than or equal to the the duration indicated by this field. Otherwise, UL MAC SDUs from this logical channel can be transmitted using an uplink grant resulting in any PUSCH duration. Corresponds to "maxPUSCH-Duration' in TS 38.321 [3].

priority Logical channel priority, as specified in TS 38.321 [3].

prioritisedBitRate Value in kiloBytes/s. 0kBps corresponds to 0, 8kBps corresponds to 8 kiloBytes/s,16 kBps corresponds to 16 kiloBytes/s, and so on. For SRBs, the value can only be set to infinity.

schedulingRequestId If present, it indicates the scheduling request configuration applicable for this logical channel, as specified in TS 38.321 [3].

## 7.2. Text Proposal (38.331)

Note: change is based on in-principle-agreed MAC CR (R2-1812139), and the change on change is marked in yellow.

##### 5.4.3.1.2	Selection of logical channels

The MAC entity shall, when a new transmission is performed:

- select the logical channels for each UL grant that satisfy all the following conditions:

2>	the set of allowed Subcarrier Spacing index values in allowedSCS-List, if configured, includes the Subcarrier Spacing index associated to the UL grant; and

2>	maxPUSCH-Duration, if configured, is larger than or equal to the PUSCH transmission duration associated to the UL grant; and

2>	configuredGrantType1Allowed, if configured, is set to TRUE in case the UL grant is a Configured Grant Type 1; and

2>	allowedServingCells, if configured, includes the Cell information associated to the UL grant. Does not apply to logical channels associated with a DRB configured with PDCP duplication within same cell group for which PDCP duplication is deactivated.

NOTE:	The Subcarrier Spacing index, PUSCH transmission duration and Cell information are included in Uplink transmission information received from lower layers for the corresponding scheduled uplink transmission.

## 5.10	Activation/Deactivation of PDCP duplication

If one or more DRBs are configured with PDCP duplication, the network may activate and deactivate the PDCP duplication for the configured DRB(s).

The PDCP duplication for the configured DRB(s) is activated and deactivated by:

-	receiving the Duplication Activation/Deactivation MAC CE described in subclause 6.1.3.11.

-	indication by RRC.

The MAC entity shall for each DRB configured with PDCP duplication:

1>	if a Duplication Activation/Deactivation MAC CE is received activating the PDCP duplication of the DRB:

2>	indicate the activation of PDCP duplication of the DRB to upper layers;

1>	if a Duplication Activation/Deactivation MAC CE is received deactivating the PDCP duplication of the DRB:

2>	indicate the deactivation of PDCP duplication of the DRB to upper layers;

## 7.3. Text Proposal (38.300)

### 16.1.3	Packet Duplication

When duplication is configured for a radio bearer by RRC, a secondary RLC entity and a secondary logical channel are added to the radio bearer to handle the duplicated PDCP PDUs. Duplication at PDCP therefore consists in submitting the same PDCP PDUs twice: once to the primary RLC entity and a second time to the secondary RLC entity. With two independent transmission paths, packet duplication therefore increases reliability and reduces latency and is especially beneficial for URLLC services.

NOTE:	PDCP control PDUs are not duplicated and always submitted to the primary RLC entity.

When duplication is activated, the original PDCP PDU and the corresponding duplicate shall not be transmitted on the same carrier. The two different logical channels can either belong to the same MAC entity (CA) or to different ones (DC). In the former case, logical channel mapping restrictions are used in MAC to ensure that the logical channel carrying the original PDCP PDUs and logical channel carrying the corresponding duplicates are not sent on the same carrier.

When an RLC entity acknowledges the transmission of a PDCP PDU, the PDCP entity shall indicate to the other RLC entity to discard it; and when the secondary RLC entity reaches the maximum number of retransmissions for a PDCP PDU, the UE informs the gNB but does not trigger RLF.

When configuring duplication for a DRB, RRC also sets the initial state (either activated or deactivated). After the configuration, the state can then be dynamically controlled by means of a MAC control element and in DC, the UE applies the MAC CE commands regardless of their origin (MCG or SCG). When duplication is deactivated for a DRB, the secondary RLC entity is not re-established, the HARQ buffers are not flushed but the corresponding logical channel mapping restrictions – if any – are lifted in CA, and the transmitting PDCP entity should indicate to the secondary RLC entity to discard all duplicated PDCP PDUs.

When duplication is configured for an SRB the state is always active and cannot be dynamically controlled.

When activating duplication for a DRB, NG-RAN should ensure that at least one serving cell is activated for each logical channel of the DRB; and when the deactivation of SCells leaves no serving cells activated for a logical channel of the DRB, NG-RAN should ensure that duplication is also deactivated.