3GPP TSG-WG2 Meeting #103	R2-1813082

# Gothenburg, Sweden, August 20th – 24th  2018

| CR-Form-v11.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.331 | CR |  | rev |  | Current version: | 15.2.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | X | Radio Access Network | X | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | PTRS-UplinkConfig NBC Correction. |
| --- | --- |
| Source to WG: | Ericsson |  |  |  |  |  |  |  |  |  |
| Source to TSG: | n.a. |  |  |  |  |  |  |  |  |  |
| Work item code: | NR_newRAT-Core |  |  |  |  |  |  | Date: |  | 2018-08-24 |
| Category: | F |  |  |  |  |  |  | Release: |  | Rel-15 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) Rel-12 (Release 12) Rel-13 (Release 13) Rel-14 (Release 14) Rel-15 (Release 15) Rel-16 (Release 16) |  |
| Reason for change: |  | The PTRS-UplinkConfig IE separates parameters for PTRS with and without transform precoding into two branches of a CHOICE structure. Hence, the NW has to choose which of the two modes to configure. However, like for DMRS, it is supposed to be possible to configure both modes simultaneously. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Remove the CHOICE structure and make the two SEQUENCEs of mode specific parameters OPTIONAL “Need R” so that the network can setup and release them independently. The change is non-backwards-compatible as it changes the sequence of coded ASN.1 bits. Impact Analysis Impacted functionality: Uplink Phase-Tracking Reference Signal configuration Inter-operability: If the NW implements the change but the UE does not, the UE cannot comprehend the received PTRS-UplinkConfig IE and hence fails to parse the RRCReconfiguration message containing the IE. If the UE implements the change but the NW does not, the UE cannot comprehend the received PTRS-UplinkConfig IE and hence fails to parse the RRCReconfiguration message containing the IE. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | The NW cannot configure UL PTRS with and without transform precoding simultaneously. |  |  |  |  |  |  |  |  |
| Clauses affected: |  |  |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  |  | Other core specifications |  |  | TS/TR ... CR ... |  |  |  |
| affected: |  |  |  | Test specifications |  |  | TS/TR ... CR ... |  |  |  |
| (show related CRs) |  |  |  | O&M Specifications |  |  | TS/TR ... CR ... |  |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |

#### –	PTRS-UplinkConfig

The IE PTRS-UplinkConfig is used to configure uplink Phase-Tracking-Reference-Signals (PTRS).

PTRS-UplinkConfig information element

-- ASN1START

-- TAG-PTRS-UPLINKCONFIG-START

PTRS-UplinkConfig ::= 					SEQUENCE {

transformPrecoderDisabled									SEQUENCE {

frequencyDensity						SEQUENCE (SIZE (2)) OF INTEGER (1..276)					OPTIONAL,	-- Need S

timeDensity								SEQUENCE (SIZE (3)) OF INTEGER (0..29)					OPTIONAL, 	-- Need S

maxNrofPorts							ENUMERATED {n1, n2},

resourceElementOffset					ENUMERATED {offset01, offset10, offset11 }				OPTIONAL,	-- Need S

ptrs-Power								ENUMERATED {p00, p01, p10, p11}

}																										OPTIONAL,	-- Need R

transformPrecoderEnabled								SEQUENCE {

sampleDensity							SEQUENCE (SIZE (5)) OF INTEGER (1..276),

timeDensityTransformPrecoding			ENUMERATED {d2}												OPTIONAL	-- Need S

}																										OPTIONAL,	-- Need R

...

}

-- TAG-PTRS-UPLINKCONFIG-STOP

-- ASN1STOP

PTRS-UplinkConfig field descriptions

frequencyDensity Presence and frequency density of UL PT-RS for CP-OFDM waveform as a function of scheduled BW If the field is absent, the UE uses K_PT-RS = 2. Corresponds to L1 parameter 'UL-PTRS-frequency-density-table' (see 38.214, section 6.1)

maxNrofPorts The maximum number of UL PTRS ports for CP-OFDM. Corresponds to L1 parameter 'UL-PTRS-ports' (see 38.214, section 6.2.3.1)

ptrs-Power UL PTRS power boosting factor per PTRS port. Corresponds to L1 parameter 'UL-PTRS-power' (see 38.214, section 6.1, table 6.2.3-5)

resourceElementOffset Indicates the subcarrier offset for UL PTRS for CP-OFDM. If the field is absent, the UE applies the value offset00. Corresponds to L1 parameter 'UL-PTRS-RE-offset' (see 38.214, section 6.1)

sampleDensity Sample density of PT-RS for DFT-s-OFDM, pre-DFT, indicating a set of thresholds T={NRBn,n=0,1,2,3,4}, that indicates dependency between presence of PT-RS and scheduled BW and the values of X and K the UE should use depending on the scheduled BW according to the table in 38.214 FFS_Section. Corresponds to L1 parameter 'UL-PTRS-pre-DFT-density' (see 38.214, section 6.1, 6.2.3-3)

timeDensity Presence and time density of UL PT-RS for CP-OFDM waveform as a function of MCS If the field is absent, the UE uses L_PT-RS = 1. Corresponds to L1 parameter 'UL-PTRS-time-density-table' (see 38.214, section 6.1)

timeDensityTransformPrecoding Time density (OFDM symbol level) of PT-RS for DFT-s-OFDM. If the field is absent, the UE applies value d1. Corresponds to L1 parameter 'UL-PTRS-time-density-transform-precoding' (see 38.214, section 6.1)

transformPrecoderDisabled Configuration of UL PTRS without transform precoder (with CP-OFDM).

transformPrecoderEnabled Configuration of UL PTRS with transform precoder (DFT-S-OFDM).