3GPP TSG-RAN WG2 Meeting #103	R2-1812756

Gothenburg, Sweden, 20 - 24 August 2018

Title:	[DRAFT] LS on gap configuration clarification for CSI-RS measurement

Response to:	R4-1808017

Release:	Release 15

Work Item:	NR_newRAT-Core

Source:	Nokia [TSG RAN WG2]

To:	RAN WG4

Cc:

Contact Person:

#### Name:	Jing He

E-mail Address:	jing.1.he@nokia-sbell.com

Attachments:	-

1. Overall Description:

RAN2 thanks RAN4 LS to clarify it is possible that CSI-RS measurement occurs outside the SMTC window for RRM measurement of the same cell. Currently RAN2 understand that defined measurement gap configuration in RRC signaling only can be used to support gap-based SSB measurement only or CSI-RS measurement only, or both configured but the CSI-RS occasion fully falling into SMTC window or vice versa.

GapConfig ::=						SEQUENCE {

gapOffset 							INTEGER (0..159),

mgl 								ENUMERATED {ms1dot5, ms3, ms3dot5, ms4, ms5dot5, ms6},

mgrp 								ENUMERATED {ms20, ms40, ms80, ms160},

mgta								ENUMERATED {ms0, ms0dot25, ms0dot5},

...

}

For case of CSI-RS location partially overlapped with SSB location, it needs RAN4 clarification on if current defined value in mgl (gap duration) and mgrp (gap periodicity) needs extension. For case of CSI-RS location separated with SSB location, it needs RAN4 clarification on if two kinds of gap pattern with different duration and periodicity are needed to configure to UE.

2. Actions:

RAN2 respectfully requests RAN4 to clarify if current defined value in mgl (gap duration) and mgrp (gap periodicity) needs extension to support the overlapped case and if two kinds of gap pattern with different duration and periodicity are needed to support separated case.

3. Date of Next TSG-RAN WG2 Meetings:

3GPP TSG RAN WG2#103bis	8 - 12 Oct 2018   	 	Chengdu

3GPPRAN2#104 		12 - 16 Nov 2018    		Spokane