3GPP TSG-RAN2 Meeting #103 bis	R2-1813629

# Chengdu, China, 8th Oct 2018 - 12th Oct 2018

| CR-Form-v11.2.1 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.321 | CR | 0407 | rev | - | Current version: | 15.3.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | x | Radio Access Network | x | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | CR on carrier to use for the Random Access procedure |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | OPPO |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R2 |  |  |  |  |  |  |  |  |  |
| Work item code: | NR_newRAT-Core |  |  |  |  |  | Date: |  |  | 2018-09-25 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-15 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) Rel-12 (Release 12) Rel-13 (Release 13) Rel-14 (Release 14) Rel-15 (Release 15) Rel-16 (Release 16) |  |
| Reason for change: |  | In the latest MAC specification, for the following condition when UE selects the carrier to be used for random access procedure, it’s not clear where the carrier is signalled, RRC signalling or DCI. To make the statement clearer, it’s better to indicate where the carrier is signalled, in this case, the carrier is indicated by PDCCH order. 1> if the carrier to use for the Random Access procedure is explicitly signalled: 2> select the signalled carrier for performing Random Access procedure; 2> set the PCMAX to PCMAX,f,c of the signalled carrier. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Clearly indicate the carrier to use for Random Access procedure is explicitly signalled by PDCCH order. Impact analysis Impacted functionality: Random Access procedure initialization Inter-operability: - If the network is implemented according to the CR and the UE is not there are no inter-operability problems. - If the UE is implemented according to the CR and the network is not, there are no inter-operability problems. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Not clear whether the carrier to use for RA is from RRC or DCI. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 5.1.1 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | x | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | x | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |

Start of change

5.1	Random Access procedure

5.1.1	Random Access procedure initialization

The Random Access procedure described in this subclause is initiated by a PDCCH order, by the MAC entity itself, or by RRC for the events in accordance with TS 38.300 [2]. There is only one Random Access procedure ongoing at any point in time in a MAC entity. The Random Access procedure on an SCell shall only be initiated by a PDCCH order with ra-PreambleIndex different from 0b000000.

NOTE 1:	If the MAC entity receives a request for a new Random Access procedure while another is already ongoing in the MAC entity, it is up to UE implementation whether to continue with the ongoing procedure or start with the new procedure (e.g. for SI request).

RRC configures the following parameters for the Random Access procedure:

-	prach-ConfigurationIndex: the available set of PRACH occasions for the transmission of the Random Access Preamble;

-	preambleReceivedTargetPower: initial Random Access Preamble power;

-	rsrp-ThresholdSSB: an RSRP threshold for the selection of the SSB and corresponding Random Access Preamble and/or PRACH occasion. If the Random Access procedure is initiated for beam failure recovery, rsrp-ThresholdSSB used for the selection of the SSB within candidateBeamRSList refers to rsrp-ThresholdSSB in BeamFailureRecoveryConfig IE;

-	rsrp-ThresholdCSI-RS: an RSRP threshold for the selection of CSI-RS and corresponding Random Access Preamble and/or PRACH occasion. If the Random Access procedure is initiated for beam failure recovery, rsrp-ThresholdCSI-RS shall be set to a value calculated by multiplying rsrp-ThresholdSSB in BeamFailureRecoveryConfig IE by powerControlOffset as specified in TS 38.214 [6];

-	rsrp-ThresholdSSB-SUL: an RSRP threshold for the selection between the NUL carrier and the SUL carrier;

-	candidateBeamRSList: a list of reference signals (CSI-RS and/or SSB) identifying the candidate beams for recovery and the associated Random Access parameters;

-	powerControlOffset: a power offset between rsrp-ThresholdSSB and rsrp-ThresholdCSI-RS to be used when the Random Access procedure is initiated for beam failure recovery;

-	powerRampingStep: the power-ramping factor;

-	powerRampingStepHighPriority: the power-ramping factor in case of differentiated Random Access procedure;

-	scalingFactorBI: a scaling factor for differentiated Random Access procedure;

-	ra-PreambleIndex: Random Access Preamble;

-	ra-ssb-OccasionMaskIndex: defines PRACH occasion(s) associated with an SSB in which the MAC entity may transmit a Random Access Preamble (see subclause 7.4);

-	ra-OccasionList: defines PRACH occasion(s) associated with a CSI-RS in which the MAC entity may transmit a Random Access Preamble;

-	ra-PreambleStartIndex: the starting index of Random Access Preamble(s) for on-demand SI request;

-	preambleTransMax: the maximum number of Random Access Preamble transmission;

-	ssb-perRACH-OccasionAndCB-PreamblesPerSSB (SpCell only): defines the number of SSBs mapped to each PRACH occasion and the number of Random Access Preambles mapped to each SSB;

-	if groupBconfigured is configured, then Random Access Preambles group B is configured.

-	Amongst the contention-based Random Access Preambles associated with an SSB (as defined in TS 38.213 [6]), the first numberOfRA-PreamblesGroupA Random Access Preambles belong to Random Access Preambles group A. The remaining Random Access Preambles associated with the SSB belong to Random Access Preambles group B (if configured).;

NOTE 2:	If Random Access Preambles group B is supported by the cell and SSBs are mapped to Random Access Preambles, Random Access Preambles group B is included in each SSB.

-	if Random Access Preambles group B is configured:

-	ra-Msg3SizeGroupA (per cell): the threshold to determine the groups of Random Access Preambles;

-	msg3-DeltaPreamble: ∆PREAMBLE_Msg3 in TS 38.213 [6];

-	messagePowerOffsetGroupB: the power offset for preamble selection;

-	numberOfRA-PreamblesGroupA (SpCell only): defines the number of Random Access Preambles in Random Access Preamble group A for each SSB.

-	the set of Random Access Preambles and/or PRACH occasions for SI request, if any;

-	the set of Random Access Preambles and/or PRACH occasions for beam failure recovery request, if any;

-	ra-ResponseWindow: the time window to monitor RA response(s) (SpCell only);

-	ra-ContentionResolutionTimer: the Contention Resolution Timer (SpCell only).

In addition, the following information for related Serving Cell is assumed to be available for UEs:

-	if Random Access Preambles group B is configured:

-	if the Serving Cell for the Random Access procedure is configured with supplementaryUplink, and SUL carrier is selected for performing Random Access Procedure:

-	PCMAX,f,c of the SUL carrier as specified in TS 38.101 [10].

-	else:

-	PCMAX,f,c of the NUL carrier as specified in TS 38.101 [10].

The following UE variables are used for the Random Access procedure:

-	PREAMBLE_INDEX;

-	PREAMBLE_TRANSMISSION_COUNTER;

-	PREAMBLE_POWER_RAMPING_COUNTER;

-	PREAMBLE_POWER_RAMPING_STEP;

-	PREAMBLE_RECEIVED_TARGET_POWER;

-	PREAMBLE_BACKOFF;

-	PCMAX;

-	SCALING_FACTOR_BI;

-	TEMPORARY_C-RNTI.

When the Random Access procedure is initiated on a Serving Cell, the MAC entity shall:

1>	flush the Msg3 buffer;

1>	set the PREAMBLE_TRANSMISSION_COUNTER to 1;

1>	set the PREAMBLE_POWER_RAMPING_COUNTER to 1;

1>	set the PREAMBLE_BACKOFF to 0 ms;

1>	if the carrier to use for the Random Access procedure is explicitly signalled by PDCCH order:

2>	select the signalled carrier for performing Random Access procedure;

2>	set the PCMAX to PCMAX,f,c of the signalled carrier.

1>	else if the carrier to use for the Random Access procedure is not explicitly signalled; and

1>	if the Serving Cell for the Random Access procedure is configured with supplementaryUplink; and

1>	if the RSRP of the downlink pathloss reference is less than rsrp-ThresholdSSB-SUL:

2>	select the SUL carrier for performing Random Access procedure;

2>	set the PCMAX to PCMAX,f,c of the SUL carrier.

1>	else:

2>	select the NUL carrier for performing Random Access procedure;

2>	set the PCMAX to PCMAX,f,c of the NUL carrier.

1>	set PREAMBLE_POWER_RAMPING_STEP to powerRampingStep;

1>	if powerRampingStepHighPriority is configured:

2>	if the Random Access procedure was initiated for beam failure recovery (as specified in subclause 5.17); or

2>	if the Random Access procedure was initiated for handover:

3>	set the PREAMBLE_POWER_RAMPING_STEP to powerRampingStepHighPriority;

1>	set SCALING_FACTOR_BI to 1;

1>	if scalingFactorBI is configured:

2>	if the Random Access procedure was initiated for beam failure recovery (as specified in subclause 5.17); or

2>	if the Random Access procedure was initiated for handover:

3>	set the SCALING_FACTOR_BI to scalingFactorBI;

1>	perform the Random Access Resource selection procedure (see subclause 5.1.2).

End of change