3GPP TSG-RAN2#103-BIS meeting	 Tdoc R2-1813679

Chengdu, China, 8 - 13 October 2018

Agenda Item:	11.2.1.3

Souce:	MediaTek Inc.,

Title:	LBT and CAPC for Uplink Control Channels in NR-U

Document for:		Discussion and decision

# 1 Introduction

Discussions on supporting New Radio over Unlicensed (NR-U) frequency bands is now underway in 3GPP standard meetings. Both gNB and UEs in NR-U need to perform Listen-Before-Talk (LBT) and also determine the Channel Access Priority Class (CAPC) for every transmission, as unlicensed frequencies are also used by other networks, like WiFi. While there has been recent proposals for LBT and CAPC for user plane (UL and DL) data transmission, LBT and CAPC for Control Channels is not yet discussed and resolved.

In this contribution, we introduce suitable LBT mechanisms and methods for determining CAPC for uplink control channels (PUCCH) over NR-U.

# 2 Uplink Control Channels (PUCCH) in NR-U

PUCCH is an uplink physical channel that carries UCI (Uplink Control Information). The major information transmitted over PUCCH are categorized as follows:

- Scheduling Request (SR)

- ACK/NAK (when no UL data to transmit)

- Channel State Information (CSI) Report (periodic/semi-persistent)

3GPP 38.213 [1] mentions multiple PUCCH formats for carrying SR, HARQ Ack/Nack, CSI Report or a combination of these information. The size of the UCI associated also varies from one format to another. In NR-U, UE and gNB needs to perform LBT and CAPC for transmission over PUCCH. Thus, suitable LBT mechanism and efficient CAPC determination needs to be performed for PUCCH in NR-U.

Observation 1: UE needs to perform suitable LBT mechanism and for category 4 LBT determine CAPC for transmission over PUCCH in NR-U.

Based on this observation, we would like to introduce suitable LBT mechanisms and methods for determining CAPC for PUCCH transmission in NR-U.

# 3 LBT and CAPC for PUCCH

In this section we will first design an approach to determine the LBT type for PUCCH and subsequently estimate the CAPC of the PUCCH.

## 3.1 LBT for PUCCH in NR-U

Listen-before-talk (LBT) [1] is defined as a mechanism, using which an equipment applies a clear channel assessment (CCA) check before using the channel. While LBT for DL and UL data transmission are largely discussed in 3GPP, LBT for PUCCH is not yet discussed. Thus, we observe that there is an impending need for suitable selection of LBT for different messages involved in the random access process.

As mentioned before in Section 2, PUCCH typically carries SR, HARQ Ack/Nack, CSI or any combination of these information. Depending on the size of PUCCH (number of OFDMA symbols) and the size of Uplink Control Information (UCI) carried, 3GPP 38.300 [2] and 38.213 [1] has mentioned different formats of PUCCH. Naturally, depending on the different formats and UCI size, the LBT and CAPC for PUCCH could be different.

Proposal 1: LBT type for PUCCH message transmissions should be based on the PUCCH format. The final details of LBT parameters (e.g. MCOT) should be decided by RAN 1.

Among all LBT types, category 4 LBT scheme is expected to provide efficient contention resolution, while ensuring fairness with WiFi [3]. Thus, in NR-U PUCCH, category 4 LBT is a natural and fair choice. However, category 2 LBT could also be used for short PUCCH, carrying small UCI payloads (Format #1), like only SR or HARQ Ack/Nack. RAN1 to needs make the final decision regarding the LBT category for PUCCH in NR-U.

| # | PUCCH Format | Format Description | LBT Category |  |  |
| --- | --- | --- | --- | --- | --- |
| 1 | Format #0 | Short PUCCH (1 or 2 symbols), small UCI payloads (< 2 bits) | Cat. 4 | Cat. 2 | Mix of Cat. 2 and Cat. 4 |
| 2 | Format #1 | Long PUCCH (4 ~ 14 symbols), small UCI payloads (< 2 bits) |  |  |  |
| 3 | Format #2 | Short PUCCH (1 or 2 symbols), large UCI payloads (> 2 bits) |  |  |  |
| 4 | Format #3 | Long PUCCH (4 ~ 14 symbols), large UCI payloads (> 2 bits) |  |  |  |
| 5 | Format #4 | Long PUCCH (4 ~ 14 symbols), moderate UCI payloads (> 2 bits) |  |  |  |

Table 1: LBT for PUCCH messages in NR-U

Observation 2: There could be three possible options for LBT categories in PUCCH:

- All PUCCH messages use LBT Category 4 LBT

- All PUCCH messages use LBT Category 2 LBT

- PUCCH messages use a mix of Category 2 and Category 4.

Proposal 2: RAN2 assumed that RAN1 decide the LBT type.

## 3.2 CAPC Estimation for PUCCH in NR-U

Category 4 LBT requires determination of CAPC, where lower CAPC values reflect higher priority. We propose to explore UL Logical Channel Priorities (LCP) to estimate the CAPC values for UL SR transmissions. Alternatively, instead of LCH, uplink QCI could also be used to estimate the CAPC of SR in NR-U. As a third option, the network can also explicitly configure the CAPC values for SR.

- Such a table, mapping ULL LCP or QCI to UL CAPC for SR could be configured and signalled by RRC.

- Alternatively, it could be hardcoded and used in specifications

Proposal 3: CAPC for UL SR should be based on the UL LCP values. Alternatively, UL QCI could also be used to determine the CAPC for SR. As a third option, the network can also explicitly configure CAPC for UL SR transmissions.

If DL HARQ decoding fails, the UE could not have any information about the DL LCP values. Thus, UE could be unable to use the DL LCP values for determining the UL CAPC for HARQ Ack/Nack transmissions. Hence, considering the time constraints of small HARQ Ack/Nack messages, we propose to use the highest priority CAPC (CAPC = 1) for HARQ Ack/Nack transmission. On the other hand, the CSI messages are relatively bigger and does not have time constraints. Hence, we propose the network (gNB) to configure the CAPC for CSI messages.

Proposal 4: HARQ Ack/Nack in PUCCH is assigned with CAPC 1 (the highest priority). The network should configure the CAPC for CSI in PUCCH.

# 4 Extension over PUCCH Carrying Multiple Information

As mentioned in 3GPP 38.213, PUCCH can also carry a combination SR, HARQ Ack/Nack and CSI. In that case, CAPC corresponding having most strict requirements could be used, i.e. CAPCPUCCH = min (Ci). However, the final decision if left to RAN2 and RAN1 for estimating and agreeing on how to determine CAPC for a PUCCH carrying a combination of SR, HARQ Ack/Nack and CSI.

Proposal 5: For PUCCH carrying multiple information, the CAPC corresponding to the highest priority among all components PUCCH should be used.

The final decision should be left to RAN 2 and RAN 1.

# 5 Conclusions

In this contribution, we first pointed out the necessity of a suitable LBT and CAPC procedure for PUCCH in NR-U. Subsequently, we proposed some solutions for LBT process and CAPC estimation methods for PUCCH in NR-U. Our observations are proposal are summarized below.

Observation 1: Suitable LBT mechanism and CAPC determination needs to be performed for efficient transmission over PUCCH in NR-U.

Proposal 1: LBT type for PUCCH message transmissions should be based on the PUCCH format. The final details of LBT parameters (e.g. MCOT) should be decided by RAN 1.

Observation 2: There could be three possible options for LBT categories in PUCCH:

- All PUCCH messages use LBT Category 4 LBT

- All PUCCH messages use LBT Category 2 LBT

- PUCCH messages use a mix of Category 2 and Category 4.

Proposal 2: RAN2 assumed that RAN1 decide the LBT type

Proposal 3: CAPC for UL SR should be based on the UL LCP values. Alternatively, UL QCI could also be used to determine the CAPC for SR. As a third option, the network can also explicitly configure CAPC for UL SR transmissions.

Proposal 4: HARQ Ack/Nack in PUCCH is assigned with CAPC 1 (the highest priority). The network should configure the CAPC for CSI in PUCCH.

Proposal 5: For PUCCH carrying multiple information, the CAPC corresponding to the highest priority among all components PUCCH should be used.

# 6 References

- 3GPP TS 38.213. Physical layer procedures for control (Release 15)

- 3GPP TS 38.300, Technical Specification Group Radio Access Network; NR; NR and NG-RAN Overall Description (Release 15)

- 3GPP TS 36.889. Study on Licensed-Assisted Access to Unlicensed Spectrum

- R2-1811065 - Channel access type for SR for NR-U, Oppo