3GPP TSG-RAN WG2 Meeting #103bis                                                  R2-1813727

Chengdu, China, 8th - 12th Oct 2018

Source:	CATT

Title:	NR Sidelink Protocol Stack and Functions

Agenda Item:	10.x.x

Document for:	Discussion and Decision

Introduction

At 3GPP RAN#80 meeting, a new SI “Study on NR V2X” was approved. One of the objectives is to study the sidelink L2/L3 protocols. In this contribution, we will discuss the NR sidelink UP and CP protocol stacks and the functions of each layer.

Discussion

NR sidelink UP protocol stack

During the LTE D2D discussion, LTE sidelink UP protocol stack is designed based on the LTE Uu UP protocol stack. The same principle can be used in NR, that is to say the NR sidelink UP protocol stack can be designed based on the NR Uu UP protocol stack.

The NR Uu UP protocol stack is shown in Figure-1. It includes five protocol layers: SDAP, PDCP, RLC, MAC, and PHY.

Figure-1 NR Uu UP stack

In this contribution, we will analyze whether all the four RAN2-related protocol layers, SDAP/PDCP/RLC/MAC, should be supported in NR sidelink and what functions are needed in each layer.

SDAP

The main function of SDAP is to perform Flow/DRB mapping. For LTE Uu interface, the mapping can be configured by RRC or using reflective QoS. But for V2X Sidelink, same as in LTE, the mapping between Flow and DRB can also be UE implementation and there is no specification effort. Hence it is unnecessary to introduce the SDAP layer in NR sidelink.

Proposal 1: For NR sidelink UP protocol stack, SDAP layer does not needs to be supported.

PDCP

The main functions of PDCP are listed below, and we will analyze whether they are also applicable to NR sidelink one-by-one.

- PDCP SN maintenance

There are two main usages for PDCP SN: perform reordering and duplication detection. From the perspective reordering, since there is timestamp in the V2X packet, in-order delivery is not mandatory. AndFrom the perspectvie of duplication detection, considering PDCP duplication needs to be supported for V2X in order to satisy the low latency and high reliablity requirement, PDCP SN is necessary for duplication detection. Hence PDCP SN manitainence should be supported in NR sidelink PDCP layer.

- Header compression/decompression

Header compression/decompression is used to reduce the overhead of IP header. For V2X services, there is IP packet and non IP packet. For IP packet, header compression/decompression is useful. Hence this function should be supported in NR sidelink PDCP layer.

- Ciphering/deciphering&& Intergrity protection/verification

In LTE, RAN does need to provide security for V2X services, and the secuirty is provided by upper layer. In NR, we think the same rule can be applied. Hence PDCP in sidelink does not need to support security related functions including ciphering/deciphering and intergrity protection/verification.

- Timer based SDU discard

For V2X services, it is clear that end-to-end latency is one of the QoS parameters. Hence timer based SDU discard can be supported in NR sidelink in order to avoid the unnecessary transmission.

- Routing for split bearer

This function is used for DC case. It is not suitable for sidelink.

- Duplication/Duplication detection

In Rel-15 eV2X, PDCP duplication is introduced to reduce the latency and improve the reliability. In NR, this function is also useful. Hence, it should be supported in NR sidelink.

- Reordering and in-order delivery/out-of-order delivery

As discussed above, considering there is timestamp in the V2X packet, at least out-of-order delivery can be supported. Whether in-order delivery needs to be supported can be disussed further.

Proposal 2: For NR sidelink UP protocol stack, PDCP layer needs to be supported with at least the following functions:

- PDCP SN maintenance;

- Header compression/decompression;

- Timer based SDU discard;

- Duplication/Duplication detection;

- Out-of-order delivery.

Proposal 3: For NR sidelink, whether PDCP should support reordering and in-order delivery can be further discussed.

RLC

Before discussing the RLC functions in NR sidelink, it should first make clear which RLC mode needs to be supported in NR sidelink.

- RLC UM

Same as in LTE, RLC UM defintely needs to be supported in NR sidelink.

- RLC AM

According to the SID, unicast should be supported. For unicast with strict end-to-end latency requirement, RLC AM cannot be supported because if ARQ is used, it cannot meet the end-to-end latency requirement. But there are also some V2X services with less tight end-to-end latency requirement, e.g. partially/conditionally automated driving with 100ms end-to-end latency requirement. For these V2X services, whether ARQ needs to be supported can be further discussed.

- RLC TM

For V2X service transmission, RLC TM mode can be not supported considering segementation may be needed. But for unicast, whether RLC TM mode will be used during the unicast connection setup procedure needs further study.

And then, we can discuss which RLC functions need to be supported in NR sidelink one by one.

- RLC SN maintenance

RLC SN maintenance should be supported considering segement and reassembly of RLC SDU is mandatory for V2X when the packet is large.

- ARQ (AM mode)

For groupcast and broadcast, it can not be supported. But for unicast, whether it needs to be supported depends on whether RLC AM mode needs to be supported.

- Segmentation/reassembly of RLC SDUs (UM/AM mode)

In order to improve the resource efficiency, it is no doubt that segmentation and reassembly of RLC SDU should be supported at least for RLC UM mode.

- Re-segmentation of RLC SDU segments (AM mode)

For groupcast and broadcast, it can not be supported. But for unicast, whether it needs to be supported depends on whether RLC AM mode needs to be supported.

- Duplicate detection (AM mode)

RLC layer duplication detection mainly used to detect the duplicated packet due to HARQ ACK is misunderstood as HARQ NACK in MAC layer. For NR Uu RLC UM mode, duplicate detection is only performed in PDCP layer. Hence in NR Uu, RLC duplicate detection is only for RLC AM.

For groupcast and broadcast, it can not be supported. But for unicast, whether it needs to be supported depends on whether RLC AM mode needs to be supported.

- RLC SDU discard(UM /AM mode)

RLC SDU discard is initiated by PDCP due to PDCP discard timer expiry. Since timer based SDU discard is supported in PDCP, RLC should also support RLC SDU discard.

Proposal 4: For NR sidelink, RLC needs to be supported with at least the following functions:

- RLC SN maintenance;

- Segmentation/reassembly of RLC SDUs;

- RLC SDU discard.

Proposal 5: For NR sidelink, whether RLC TM and RLC AM mode needs to be supported can be further discussed.

Proposal 6: If RLC AM needs to be supported, RLC should further support the following functions:

- ARQ;

- Re-segmentation of RLC SDU segments;

- Duplicate detection.

MAC

The main functions of MAC are listed below, and we will analyze whether it is applicable for sidelink one-by-one.

- Logical/transport channels mapping

This is one basic MAC function. Hence it is no doubt that it should be supported in MAC.

- MAC SDUs multiplexing/demultiplexing

Same as in LTE, data multiplexing/demultiplexing from different logical channels of one destination shoud be supported in NR.

- scheduling information reporting

Scheduling information includes SR, BSR and PHR. For network sheduled resource allocation, at least SR/BSR should be supported in order to let gNB perform the Sidelink resource allocation.

- HARQ

For sidelink unicast, HARQ should be supported in order to improve the realibility. But it can not be supported for groupcast and broadcast.

- Logical channel prioritisation

When receiving Sidelink grant, if there are multiple logical channels have data avaliable, LCP is mandatory in order to allocate the sidelink resource amongst these different logical channels.

Proposal 7: For NR sidelink, MAC needs to be supported with at least the following functions:

- Logical/transport channels mapping;

- MAC SDUs multiplexing/demultiplexing;

- Scheduling information reporting(only for network sheduled resource allocation);

- HARQ；

- Logical channel prioritisation.

Based on the above analysis, it is obvious that the LTE sidelink UP protocol stack, PC5-U, can be reused for NR sidelink.

Figure-2 NR Sidelink UP stack

The function of each NR Sidelink protocol layer is summarized in the following Table-1:

Table-1 NR Sidelink functions

| Protocol layer | Functions supported by Uu | Sidelink unicast | Sidelink groupcast/multicast |
| --- | --- | --- | --- |
| PDCP | PDCP SN maintenance | Y | Y |
|  | Header compression/decompression | Y | Y |
|  | Ciphering/deciphering | N | N |
|  | Integrity protection/verification | N | N |
|  | Timer based SDU discard | Y | Y |
|  | Routing for split bearer | N | N |
|  | Duplication/Duplication detection | Y | Y |
|  | Reordering and in-order delivery | FFS | FFS |
|  | Out-of-order delivery | Y | Y |
| RLC | RLC SN maintenance | Y | Y |
|  | ARQ ( AM mode) | FFS | N |
|  | Segmentation/reassembly of RLC SDUs (UM /AM mode) | Y | Y |
|  | re-segmentation (AM mode) | FFS | N |
|  | duplicate detection (AM mode) | FFS | N |
|  | RLC SDU discard (UM /AM mode) | Y | Y |
| MAC | Logical/transport channels mapping | Y | Y |
|  | MAC SDUs multiplexing/demultiplexing | Y | Y |
|  | scheduling information reporting | Y | Y |
|  | HARQ | Y | N |
|  | logical channel prioritisation | Y | Y |

Proposal 8: LTE PC5-U protocol stack can be reused in NR sidelink.

NR Sidelink CP protocol stack

In LTE, there are two control plane protocol stacks, which are shown in the following Figure-3:

Figure-3 NR Uu CP stack

UE does not establish and maintain a logical connection to receiving UEs prior to one-to-many sidelink communication. Higher layer establishes and maintains a logical connection for one-to-one sidelink communication. Hence the function split of the above two control plane protocol stack is as below:

- PC5-C: only used for for SBCCH, which includes the information transmitted by a UE transmitting sidelink synchronisation signal, i.e. acting as synchronisation reference, via SL-BCH.

- PC5 signalling protocol stack: used for establishing, maintaining and releasing the logical connection for one-to-one sidelink communication.

Considering NR also support out-of-coverage scenario, from the requirement of synchronization, PC5-C should be supported.

Proposal 9: NR Sidelink should support PC5-C at least for synchronization.

For connection management of one-to-one communication, whether it is supported by PC5-C or PC5 signaling protocol stack depends on SA2 design.

Proposal 10: Send LS to SA2 to confirm whether upper layer will be in charge of sideink connection management in NR, e.g., establishing, maintaining and releasing the logical connection.

Conclusion

According to the analysis in section 2, it is proposed:

Proposal 1: For NR sidelink UP protocol stack, SDAP layer does not needs to be supported.

Proposal 2: For NR sidelink UP protocol stack, PDCP layer needs to be supported with at least the following functions:

- PDCP SN maintenance;

- Header compression/decompression;

- Timer based SDU discard;

- Duplication/Duplication detection;

- Out-of-order delivery.

Proposal 3: For NR sidelink, whether PDCP should support reordering and in-order delivery can be further discussed.

Proposal 4: For NR sidelink, RLC needs to be supported with at least the following functions:

- RLC SN maintenance;

- Segmentation/reassembly of RLC SDUs;

- RLC SDU discard.

Proposal 5: For NR sidelink, whether RLC TM and RLC AM mode needs to be supported can be further discussed.

Proposal 6: If RLC AM needs to be supported, RLC should further support the following functions:

- ARQ;

- Re-segmentation of RLC SDU segments;

- Duplicate detection.

Proposal 7: For NR sidelink, MAC needs to be supported with at least the following functions:

- Logical/transport channels mapping;

- MAC SDUs multiplexing/demultiplexing;

- Scheduling information reporting(only for network sheduled resource allocation);

- HARQ；

- Logical channel prioritisation.

Proposal 8: LTE PC5-U protocol stack can be reused in NR sidelink.

Proposal 9: NR Sidelink should support PC5-C at least for synchronization.

Proposal 10: Send LS to SA2 to confirm whether upper layer will be in charge of sideink connection management in NR, e.g., establishing, maintaining and releasing the logical connection.

Reference

- RP-181480  New SID: Study on NR V2X