3GPP TSG-RAN WG2 Meeting #103bis                                                  R2-1813735

Chengdu, China, 8th - 12th Oct 2018

Source:	CATT

Title:	Some consideration of platooning

Agenda Item:	10.x.x

Document for:	Discussion and Decision

Introduction

At 3GPP RAN#80 meeting, a new SI “Study on NR V2X” was approved. One of the objectives is to study sidelink resource allocation mechanism:

Vehicles Platooning enables the vehicles to dynamically form a platoon travelling together. All the vehicles in the platoon obtain information from the leading vehicle to manage this platoon. These information allow the vehicles to drive closer than normal in a coordinated manner, going to the same direction and travelling together.

In this contribution, we will discuss some high level issues of platooning.

Discussion

In NR V2X, vehicle platooning is a very essential feature as identified in SID. According to RAN1 agreements, four sub-modes of Mode 2 are defined till now:

  - Mode-2 definition covers potential sidelink radio-layer functionality or resource allocation sub-modes (subject to further refinement including merging of some or all of them) where

    - UE autonomously selects sidelink resource for transmission

    - UE assists sidelink resource selection for other UE(s)

    - UE is configured with NR configured grant (type-1 like) for sidelink transmission

    - UE schedules sidelink transmissions of other UEs

In this paper, we will further study how these 4 sub modes of mode 2 works, and which scenario these sub modes should be used.

Sub-mode a)

- Sub-mode a)

In our understanding, this sub-mode is similar as the LTE V2X Mode 4 resource allocation. UE should first acquire the Tx resource pool, and then UE can perform the autonomously resource selection.

Sub-mode a) actually inherits how mode 4 works in LTE V2X, and the other sub-modes are considered as enhancement for legacy LTE V2X mode 4. Hence in NR V2X, sub mode a) can be considered as the baseline of NR V2X mode 2.

Proposal 1: Sub-mode a) as LTE V2X mode 4 can be used as the baseline of NR V2X mode 2.

Sub-mode b) / d)

In a vehicle platoon, there is a header/manager that is responsible to manage the radio resource for the whole platoon. In the scenario of in coverage, the header/manager should be authorized by the network, to ensure the safety of a platoon, in order to avoid a malicious vehicle controlling the platoon. Besides, the header/manager vehicle is responsible for deeper resource allocation, as per the radio resource achieved from gNB, gNB also requires the authorization of the header/manager vehicle to allocate radio resource for this platoon.

Likewise, the member of the vehicle platoon should also be controlled. The admission control for the member of platoon can be performed by the network or the header/manager of the platoon. If member of platoon is controlled by network, this would align with the control of the header/manager of the platoon; if the member of platoon is controlled by the header/manager, the procedure can be reused by the out of coverage UE, since there is no network to control the member of the platoon in the scenario of out of coverage. RAN2 is asked to discuss whether network or header/manager shall control the admission of platooning member.

Proposal 2: In the scenario of in coverage, the header/manager should be authorized by the network.

Proposal 3: RAN2 is asked to discuss whether network or header/manager shall control the admission of platooning member.

In sub-mode b), our understanding is the manager/header vehicle will firstly be granted with a Tx resource pool by the network, and then separate the resource pool to each member of the vehicle platoon.

In sub-mode d), our understanding is the manager/header vehicle will firstly be granted with a Tx resource pool by the network, and then play the role of gNB internally in this vehicle platoon.

Both sub-mode b) and d) enables the header/manager to allocate the radio resources from the network granted to be further allocated to platoon members. But if sub-mode d) function mounted on the header/manager vehicle has to implement very complex radio resource scheduling algorithm, which increase the cost and complexity of NR V2X UE dramatically. Besides, if sub-mode d) is introduced, every NR V2X UE probably plays the role of header/manager vehicle, so every V2X UE is required to support sub-mode 4, which will increase the cost and complexity for all NR V2X UE.

Hence we think the modes that UE support should be reported to the network, so that the network can decides the mode that the header/manager can work in.

Proposal 4: UE reports the modes supported to the network.

Proposal 5: The network configures the mode for the header/manager.

Likewise, in the scenario of out of coverage, the header/manager should also be authorized, but by the member of the vehicle platoon, for the same purpose of safety. Since there is no gNB who provides authorization for the vehicle platoon, then the discovery procedure has to be introduced to enable the discovery internal platoon.. In legacy D2D discovery, the discovery message is actually specified by SA2, which is transparent to RAN. In another word, AS layer only provide radio resources for discovery. So we believe in out of coverage case, sidelink discovery procedure can be leveraged to establish a platoon, and we can send an LS to SA2 to confirm if they will implement the discovery message in upper layer to establish a platoon.

Proposal 6: Discovery procedure can be leveraged to establish a platoon in out of coverage scenario.

Sub-mode c) grant free

In LTE V2X, we introduced SPS resource allocation mode. In NR, we introduced a new grant allocation mode-grant free, in order to satisfy the extreme low latency service, URLLC, etc., For type 1 grant free, RRC configures the grant, the grant is valid right after the configuration; on the contrary, type 2 grant free require the activation of PDCCH, which almost makes no difference with LTE SPS. In NR grant free mode, UE doesn’t have to acquire the grant from the network before transmission. The benefit of grant free is to satisfy the extreme low latency service, but introduces radio resource collision, and unnecessary radio resources consumption, especially for type 1 grant free mode. So type 1 and type 2 grant free modes both can be used for different scenarios.

Observation 1: NR V2X grant free type 2 makes no difference with SPS radio resource selection in legacy LTE V2X.

Proposal 7: RAN2 to discuss the V2X scenarios for type 1 and type 2 grant free.

Conclusion

As the first meeting of NR V2X in RAN2, we discussed the resource allocation mythology, regarding the establishment of a platoon, and we have the following observation and proposals:

Proposal 1: Sub-mode a) as LTE V2X mode 4 can be used as the baseline of NR V2X mode 2.

Proposal 2: In the scenario of in coverage, the header/manager should be authorized by the network.

Proposal 3: RAN2 is asked to discuss whether network or header/manager shall control the admission of platooning member.

Proposal 4: UE reports the modes supported to the network.

Proposal 5: The network configures the mode for the header/manager.

Proposal 6: Discovery procedure can be leveraged to establish a platoon in out of coverage scenario.

Observation 1: NR V2X grant free type 2 makes no difference with SPS radio resource selection in legacy LTE V2X.

Proposal 7: RAN2 to discuss the V2X scenarios for type 1 and type 2 grant free.

Reference

- RP-181480  New SID: Study on NR V2X

- R1-1809069	 Resource allocation mechanism	AT&T

- R1-1809159	 Resource allocation mechanism	NTT DOCOMO, INC.

- 3GPP TR22.886

- 3GPP TR37.885