3GPP TSG-RAN WG2 meeting #103bis												R2-1813736

Chengdu, China, 8 - 12 October, 2018                    Revision of R2-1811286

Agenda item:	11.2.2.2

Source:	ZTE

Title:	Simulation and Evaluation for RLM/RLF on NR-U

Document for:	Discussion and Decision

Introduction

On NR-U, LBT failure may cause the reduced transmission for signals, such as RLM-RS. When UE monitors the downlink link quality based on the reference signal in the configured RLM-RS resource(s), the missing RLM-RS may introduce more out-of-sync indication, and then trigger RLF. In RAN1#93[1], the agreement about RLM/RRM has been reached as follows:

Agreement: Potential modifications to RLM/RRM procedures due to reduced transmission opportunities for DL signals and channels due to LBT failure should be identified and studied

In this contribution, at the first step, we figures out how much impact on RLF will missing RLM-RS have. And then, we will find the solutions if the impact can’t be negligible.

Discussion

RLM and RLF

On NR, according to 38.133[2] and 38.213[3], the UE shall monitor the downlink link quality based on the configured RLM-RS resource(s) for the purpose of indicating out-of-sync/in-sync status to higher layers. The configured RLM-RS resources can be all SSBs, or all CSI-RSs, or a mix of SSBs and CSI-RSs.

UE shall evaluate whether the downlink radio link quality on the configured RLM-RS resource estimated over the last TEvaluate_out_SSB [ms] evaluation period becomes worse than the threshold Qout_SSB. When the radio link quality is worse than the threshold Qout for all resources in configured RLM-RS resource, the physical layer in the UE indicates out-of-sync to RRC.

UE shall evaluate whether the downlink radio link quality on the configured RLM-RS resource estimated over the last TEvaluate_in_SSB [ms] evaluation period becomes better than the threshold Qin_SSB. When the radio link quality is better than the threshold Qin for any resource in configured RLM-RS resource, the physical layer in the UE indicates, in-sync to RRC.

According to 38.331[4], if receiving N310 consecutive "out-of-sync" indications from the physical layer, UE shall start timer T310. Upon receiving N311 consecutive "in-sync" indications from the physical layer while T310 is running, the UE shall stop timer T310. Upon T310 expiry, UE shall declare RLF and initiate the connection re-establishment procedure. An example of RLM/RLF procedure is shown in Figure 1.

Figure 1 An example of RLM/RLF procedure

Compared with the periodic and constant transmission of RLM-RS on NR, the LBT failure will result in the missing RLM-RS on NR-U. In the RLM procedure of NR, UE will treat the missing RLM-RS as very poor channel quality. However, considering that UE has the capability of false alarm detection, UE is able to distinguish the missing RLM-RS. In the RLM procedure of NR-U, how to deal with the missing RLM-RS still is a problem.

In the following, we will clarify the impact of missing RLM-RS on RLF through simulation and find out the solutions.

Simulation and analysis

First of all, in order to distinguish the reason triggering RLF, we adopts two cases to simulate the RLM procedure separately:

- Case 1: treating the measurement result of missing RLM-RS as very poor channel quality

- Case 2: ignoring the measurement result of missing RLM-RS

The following table summarizes the results of RLF probability and LBT failure probability. More details of the simulation assumptions can be found in Annex.

Table 1. Summary of simulation results

| Simulation result | Lamda=0.2 |  |  | Lamda=0.13 |  |  | Lamda=0.1 |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Case 1 | Case 2 | Gain | Case 1 | Case 2 | Gain | Case 1 | Case 2 | Gain |
| LBT failure probability | 36.92% |  |  | 19.37% |  |  | 5.76% |  |  |
| RLF probability for cell edge UE | 33.35% | 25.00% | +32.58% | 27.80% | 22.20% | +25.23% | 11.10% | 11.10% | 0% |
| RLF probability for cell center UE | 9.03% | 7.64% | +18.30% | 5.55% | 4.16% | +33.33% | 0.00% | 0.00% | 0% |
| RB usage ratio | 45.45% |  |  | 34.34% |  |  | 15.58% |  |  |

As shown in Table 1, the increment of traffic volume will result in intense contention among nodes, and then increase the LBT failure probability correspondingly. Although the RLF probability increases along with the raising of LBT failure probability, we should distinguish the impact of LBT failure and heavy interference.

Under the light load (lamda=0.1), RLF probability of Case 1 and Case 2 is the same, so LBT failure has no impact on RLF; Under the medium load (lamda=0.13), the increment of RLF probability between Case 1 and Case 2 has increased to 25.23% for cell edge UEs and 33.33% for cell center UEs; Under the high load (lamda=0.2) the increment of RLF probability between Case 1 and Case 2 has increased to 32.58% for cell edge UE and 18.30% for cell center UEs.  Since the increment reflects the impact of LBT failure on RLF, we can see that the LBT failure has important impact on RLF probability under the medium and high load from the simulation results.

Observation 1:  On NR-U, the impact of LBT failure on RLF is not negligible under the medium and high load.

It is seen that the increment of RLF probability increases along with the rising of LBT failure. This is because LBT failure is considered as poor link quality in Case 1 so that OOS condition is easier to be met.

Observation 2: RLF probability will increase along with the rising of LBT failure probability

In the conclusion, LBT failure has unnegligible impact on triggering OOS indication in the RLF procedure.

So, RAN2 should discuss how to deal with the impact of LBT failure on RLF procedure.

Proposal 1:  RAN2 should further discuss the scheme to treat the impact of LBT failure on the RLM procedure on NR-U.

When gNB has little chance to serve UE due to LBT failures, it is better for UEs to switch to other carriers once realizing this condition. So, if UE can distinguish LBT failures from poor channel quality condition for transmission of RLM-RS, a mechanism to trigger carrier selection when necessary due to LBT failures can be considered. For example, when the percentage of LBT failures for transmission of RLM-RS within a certain period reaches a threshold, an RRC reestablishement procedure can be triggered.

Proposal 2:  RAN2 should discuss a mechanism to trigger carrier selection when necessary due to LBT failures.

Conclusion

The proposals are as follows:

Observation 1:  On NR-U, the impact of LBT failure on RLF is not negligible under the medium and high load.

Observation 2: RLF probability will increase along with the raising of LBT failure probability

Proposal 1:  RAN2 should further discuss the scheme to treat the impact of LBT failure on the RLM procedure on NR-U.

Proposal 2:  RAN2 should discuss a mechanism to trigger carrier selection when necessary due to LBT failures.

References

- RAN1#93, chairman notes

- 38.133-f10; Requirements for support of radio resource management (Release 15)

- 38.213-f20; Physical layer procedures for control (Release 15)

- 38.331-f21, Radio Resource Control (RRC) protocol specification (Release 15)

Annex

Simulation assumptions for system-level simulation

Table 2 Key parameters in RLM/RLF procedure

| The key parameter | value |
| --- | --- |
| N310 | 4 |
| N311 | 4 |
| T310 | 200 ms |
| the periodicity of SSB | 10 ms |
| SINR for threshold Qout_SSB | -7 dB |
| SINR for threshold Qin_SSB | -6 dB |

Table 3. System parameters of system-level simulation

| Parameters | Indoor Sub-7GHz |
| --- | --- |
| Carrier Frequency | 5GHz |
| Carrier Channel Bandwidth | 20MHz |
| Number of carriers | 1 |
| Number of users per operator | 15 per gNB per 20MHz |
| Channel Model | NR InH Mixed Office model |
| BS/AP Tx Power | 23dBm (total across all TX antennas) |
| UE/STA Tx Power | 18dBm (total across all TX antennas) |
| BS/AP Antenna gain | 0dBi |
| UE/STA Antenna gain | 0 dBi |
| BS/AP Noise Figure | 5dB |
| UE/STA Receiver Noise Figure | 9dB |
| Minimum received power from serving cell for UE dropping | -82dBm |
| BS/AP antenna Array configuration | (M, N, P, Mg, Ng) = (1, 2, 2, 1, 1), dH = dV = 0.5 λ |
| UE/STA antenna Array configuration | Tx/Rx: (M, N, P, Mg, Ng) = (1, 1, 2, 1, 1), dH = dV = 0.5 λ |
| Traffic model | Use 36.889 Table A.1.1 |