3GPP TSG RAN WG2 Meeting #103bis                                            	R2-1813844

Chengdu, China, 8th – 12th October, 2018

Source:	OPPO

Title:	Discussion on intra-UE prioritization/multiplexing

Agenda Item:	11.7.2

Document for:	Discussion and Decision

Introduction

In RAN #81, a revised SID on Study on NR Industrial Internet of Things (IoT) is approved. The potential enhancements on UL/DL intra-UE prioritization/multiplexing are discussed and captured in this study item, as shown in the following [1]:

b)  UL/DL intra-UE prioritization/multiplexing, i.e. prioritization (for example dropping, delaying or puncturing lower priority service) between different categories of traffic in the UE, including both data and control channels and considering (RAN2/RAN1):

i)   different latency and reliability requirements

ii)  Different types of resource allocation for example grant-free and grant-based allocations

Based on the draft work plan for NR IIoT [2], initial conclusion is referred to capture for helping RAN1 to start related work.

•	Agreeing on the scenarios, which should be addressed and initial discussion on the solutions with a focus on identifying potential impacts to RAN1

In this contribution, we shall show our opinions on the optional impacts on RAN1/2.

Discussion

UE behavior on UL/DL data transfer is described and separated in RAN1/RAN2 specification. Take UL data transfer as an example:

- Step 1: UL Grant prioritization

When an uplink grant has been received on PDCCH or configured uplink grant is available, UE need to do these operation: confirm NDI status, choose one uplink grant, and then deliver the chosen uplink grant and the associated HARQ information to the HARQ entity.

- This is implemented by MAC and thus in RAN2 scope.

- Step 2: HARQ operation

For each chosen uplink grant, the HARQ entity shall identify the HARQ process associated with this grant, and deliver the related information to the identified HARQ process. In addition, for new transmission, logical channel prioritization procedure should be applied, and the related information is MAC PDU, uplink grant, and the HARQ information. For re-transmission, the related information is uplink grant and the HARQ information (redundancy version). Then, HARQ process will instruct the physical layer to generate a transmission according to the related information.

- This is implemented by MAC and thus in RAN2 scope.

- Step 3: RAN1 operation

RAN1 transfer the stored uplink grant at the indicated time-frequency resource.

Specially, for eMBB and URLLC coexisting scenario in Rel-15 URLLC, pre-emption is introduced in downlink for inter-UE prioritization. If pre-emption indication is received, UE can know whether to decode on the special time-frequency resource. For inter-UE prioritization of uplink, it has been addressed in Rel-16 URLLC WI.

In NR, similar mechanism about pre-emption enhancement might be considered in RAN1, but applied to intra-UE case, for both UL and DL, e.g. for prioritization between PUSCHs, between PUCCHs, and between PUSCH and PUCCH.

- This is implemnted by PHY and thus in RAN1 scope.

As the analysis above, we can simply separate the potential solutions impacted to RAN1/RAN2, as shown in Figure 1:

Figure 1: solutions impacted to RAN1 and RAN2

Proposal1: RAN2 study solution on UL grant prioritization and LCP for intra-UE prioritization.

Proposal2: RAN1 study solution on prioritization between physical channels, including data and control channel, for UL and DL.

Conclusions

In this contribution, we show our opinions on the optional impacts on RAN1/2 with the following proposals:

Proposal1: RAN2 study solution on UL grant prioritization and LCP for intra-UE prioritization.

Proposal2: RAN1 study solution on prioritization between physical channels, including data and control channel, for UL and DL.

References

- RP-182068, Revised SID: Study on NR Industrial Internet of Things (IoT)

- R2-181xxx0, Draft Work Plan for NR IIoT, Nokia (Rapporteur)