3GPP TSG-RAN WG2 Meeting 103bis                                                  R2-1813853

Chengdu, China, 08 October– 12 October 2018              revision of R2-1811214

Source:	CATT

Title:	More collision issues with HARQ Process #0

Agenda Item:	10.3.1.2

Document for:	Discussion and Decision

- Introduction

In RAN2#103 meeting, the issue of Msg3 HARQ process #0 colliding with configured grant was discussed. Various kinds of solutions were provided and compared. A show of hands eliminated step by step the various solution candidates which resulted in the end in no consensus for solving the problem [1]:

CG and RACH MSG3 R2-1812842 Issues of sharing HARQ buffer between configured grants and Msg3 ASUSTeK, Nokia, Lenovo CR Rel-15 38.321 15.2.0 0396 - F NR_newRAT-Core Not Pursued DISCSUSSION […] 1: avoid CG usage of harq process 0 by using the CG timer (ASUStek & Co) 2: avoid CG usage of HARQ process 0 during RACH or part of the RACH procedure by a text condition (CATT, Ericsson, LG) 3: restrict the usage of HARQ processes for CG such that CG doesn’t use HARQ process 0 at all (Samsung Vivo) 1st Poll 2st Poll 3st Poll 1: 5 2: 10 11 9 3: 6 8 Do nothing 8 12 13 No agreement for a solution

As can be seen above, the various solutions where grouped into three different (main) options. The show of hands eliminated options 1 and 3 consecutively so that option 2 was the only surviving candidate. But it didn’t get sufficient support in the end compared to “doing nothing”.

In this contribution, we highlight new issues not discussed so far of collisions with HARQ process #0 and show that for most of them they cannot be prevented by gNB and result in an error case in the UE (bug). Hence they need to be fixed. We therefore suggest an improved solution along the lines of option 2 for solving these issues, as well as a new approach, much simpler, for preventing such collisions while not restricting any transmission.

- Discussion

2.1 Description of the issues

- Issue 1: HARQ Process#0 for dynamic or configured grants overridden by Msg3 PDU

This issue was initially pointed out in [2] and occurs when the Msg3 is transmitted after a new transmission of the configured grant. We also include the case where it is a dynamic grant. The scenario is shown in Figure 1.

- At time t1, the UE transmits dynamic or configured scheduling MAC PDU for the first time, using HARQ process #0. The HARQ process stores the MAC PDU from the multiplexing and assembly entity in HARQ process #0 buffer; this transmission fails.

- At time t2, the UE transmits Msg3 for the first time. And the HARQ process stores the MAC PDU for Msg3 in HARQ process #0 buffer. Since, the grant was received in RAR, and MAC PDU is obtained from Msg3 buffer, it is the initial transmission for Msg3 and it overrides the previous PDU in the HARQ buffer;

- At time t3, comes the dynamic uplink grant for retransmission of the previous MAC PDU from the multiplexing and assembly entity, scheduled at t4. However, at t4, a different MAC PDU (of potentially different size) is in the HARQ process buffer. Therefore an error occurs in the UE.

Note: it can be argued that the case of the dynamic scheduling grant can be avoided by the gNB. But that would mean that gNB cannot schedule any UE in the cell on HARQ Process #0, even for re-transmissions, during any on-going Random Access procedure. This sounds to us like a restrictive requirement for the MAC scheduler.

Figure 1: HARQ Process #0 of dynamic or configured grant overridden by HARQ Process #0 of Msg3

From the analysis above, we can derive the below observation:

Observation 1: MAC PDU from multiplexing and assembly entity in HARQ process #0 may be overridden by MAC PDU from Msg3 during dynamic scheduling as well as configured grant scheduling.

A very first comment is that upon receiving Msg3 gNB identifies the UE (via MAC CE C-RNTI) and is therefore aware that this UE has an on-going collision between the CG and Msg3, so it can predict the HARQ failure and take corresponding decisions e.g. not request a re-Tx or request it with an UL grant that matches Msg3 PDU.

Observation 2: Upon receiving Msg3 gNB identifies the UE and can prevent the error case (PDU size mismatch) at the UE.

The issue remains though that the MAC PDU from the multiplexing and assembly entity will be lost when it is overridden by Msg3 MAC PDU. If this is not for an URLLC channel, the issue is not so severe and the system can live with it. If it is an URLLC channel where the “ultra-reliable” prevails the “low latency”, then the traffic will likely be mapped on RLC AM mode and ARQ will recover the HARQ failure. If the “low latency” prevails the “ultra-reliable”, then it will likely be mapped on RLC UM mode, and the RLC cannot recover the HARQ failure. But then it is also useless to delay the transmission by blocking the CG/DS transmission in first place. If both “low latency” and “ultra-reliable” are required, then other means for recovering the PDU should be foreseen e.g. PDCP duplication, CG bundling and/or the use of the new MCS table to minimize the risk of initial transmission failure.

From the above analysis, it can be conclude that:

Proposal 1: The issue that MAC PDU from multiplexing and assembly entity is overridden by Msg3 is not a severe issue and does not need to be fixed.

- Issue 2: HARQ Process#0 for Msg3 PDU overridden by dynamic or configured grant

This issue, also spotted by [2], occurs when the HARQ Process #0 for Msg3 is overridden by other kinds of grants. The scenario can be described as follows referring to Figure 2.

- At time t1, the UE transmits Msg3 for the first time. And the HARQ process stores the MAC PDU for Msg3 in HARQ process #0 buffer. This transmission fails.

- At time t2, comes the dynamic or configured uplink grant on HARQ process #0. Since it is a new transmission, the MAC PDU is obtained from the multiplexing and assembly entity which will override the Msg3 PDU in the HARQ buffer of HARQ process #0.

- At time t3, Msg3 retransmission is scheduled to occur at t4. However, Msg3 MAC PDU is no longer in the HARQ process buffer. With Msg3 MAC PDU and the overriding new MAC PDU having potentially different sizes, an error occurs in the UE.

The same note on the dynamic scheduling grant as for issue #1 equally applies here.

Figure 2: HARQ Process #0 of Msg3 overridden by HARQ Process #0 of dynamic or configured grant

From the analysis above it can be seen that the issue exists in dynamic scheduling as well as configured grant scheduling scenarios.

Observation 2: Msg3 PDU may be overridden by MAC PDU from multiplexing and assembly entity during dynamic scheduling as well as configured grant scheduling. This will result in an error case in the UE of UL grant/MAC PDU size mismatch.

One key difference with the issue #1 is that since gNB did not receive the Msg3, it cannot identify the RACH’ing UE and prevent from the error case of UL grant/MAC PDU size mismatch in the UE. Hence this is a bug. In addition, the loss of Msg3 is problematic as it will result in contention resolution failure which will delay the Random Access procedure which is always an issue e.g. for CBRA BFR.

Observation 3: If Msg3 is overridden by MAC PDU of dynamic scheduling or configured grant, the error case in the UE of UL grant/MAC PDU size mismatch upon Msg3 retransmission cannot be anticipated by the gNB.

Observation 4: Msg3 loss cannot be recovered and leaves no other option but RA re-attempt.

Proposal 2: The issue of Msg3 overridden by MAC PDU of dynamic scheduling or configured grant shall be fixed.

- Issue 3: HARQ Process#0 for a transmission bundle overridden by Msg3

In this sub-section, we analyze new issues not discussed so far.

As shown in Figure 3, the scenario is when Msg3 transmission occurs within a transmission bundle of either a dynamic or configured grant on HARQ Process #0. In this case, there is no problem for the initial transmission of the bundle as well as any repetition occurring before Msg3. The case of a collision between Msg3 grant and a bundle grant in the slot where Msg3 is transmitted is not fully clear in current MAC specification (which we also address in [3]), however it is expected that there will be cases where Msg3 overrides the bundle grant. However, once Msg3 is transmitted, the Msg3 MAC PDU overrides the MAC PDU used for the bundled transmissions. And a PDU mismatch occurs on the very next repetition of the bundle (in next slot). Here again, as soon as the two PDUs have different sizes, the UE is unable to use the configured grant of the bundle to transmit the PDU in the HARQ buffer, which is an error case. Similar to issue 2, it looks challenging for the gNB to react in time upon receiving Msg3 to prevent the error case to occur in the very next slot. Hence this is another bug that needs to be fixed.

It is also interesting to check how short a Random Access procedure can be compared to a transmission bundle.

With lowest possible values of the ra-ResponseWindow of {1, 2, 4, … } slots, RAR transmission in the same slot as the PDCCH with CRC scrambled by RA-RNTI, and smallest possible values of K2 + ∆ = 3 slots for Msg3 scheduling e.g. for SCS = 15 KHz ([5] TS38.214-f30 Section 6.1.2.1.1), it is clear that a complete Random Access procedure can start and terminate within a transmission bundle. This is illustrated in Figure 3, showing examples where the initial transmission of the bundle may occur before the UE receives the RAR, or even before the Random Access procedure was started.

Note that, assuming the above overriding by Msg3 is solved somehow, in the case where the Random Access procedure terminates before the end of the transmissions bundle, yet another error occurs when the HARQ buffer is flushed upon RA successful completion. And here again, although Msg4 (that does not require much processing) can be sent right away, it looks challenging for the gNB to cancel the bundle by e.g. requesting its retransmission right away (a new allocation must be computed). Therefore we further classify the issue 3 into issue 3-a (bundle PDU overridden by Msg3 PDU) and issue 3-b (HARQ buffer flushed before bundle end upon RA completion), also described in Figure 3 (bottom).

Figure 3: HARQ Process #0 for a transmission bundle overridden by Msg3

Observation 5: If a MAC PDU for a transmission bundle (from dynamic or configured grant) on HARQ process #0 is overridden by Msg3 MAC PDU or is flushed at RA completion, it will result in an error case in the UE of UL grant/MAC PDU size mismatch.

Observation 6: The above error cases cannot be anticipated by gNB.

Proposal 3: The issues of a transmission bundle (from dynamic or configured grant) on HARQ process #0 overridden by Msg3 MAC PDU or flushed at RA completion shall be fixed.

2.2 Solutions

2.2.1 Solutions already discussed

Given it got most support at last meeting, we only focus on comparing the solutions from option 2 (See Section 1).

- Option 2-a [6]: fix the problem in Section 5.4.1 UL Grant reception by adding a new condition for processing a configured grant that its HARQ process is not currently used for Msg3 transmission:

For each Serving Cell and each configured uplink grant, if configured and activated, the MAC entity shall: 1> if the PUSCH duration of the configured uplink grant does not overlap with the PUSCH duration of an uplink grant received on the PDCCH for this Serving Cell: 2> set the HARQ Process ID to the HARQ Process ID associated with this PUSCH duration; 2> if the configuredGrantTimer for the corresponding HARQ process is not running: 2> if the corresponding HARQ Process of the configured uplink grant is not a HARQ Process currently used for Msg3 transmission: 3> consider the NDI bit for the corresponding HARQ process to have been toggled; 3> deliver the configured uplink grant and the associated HARQ information to the HARQ entity

In the above, it is unclear what means “currently” in “HARQ Process currently used for Msg3 transmission”. It could be reduced to the period of transmission time, or the period between the (initial) transmission and the reception of Msg4 (thus also including potential re-transmissions), or the whole period from receiving the RAR up to Msg4 reception. We assume the latter interpretation (wide sense) in the following.

- Option 2-b [4]: fix the problem in Section 5.4.2.1 HARQ entity by adding a new condition for skipping an UL grant that it is not on HARQ process #0 while the ra-ContentionResolutionTimer is running:

For each uplink grant, the HARQ entity shall: 1> identify the HARQ process associated with this grant, and for each identified HARQ process: 2> if the received grant was not addressed to a Temporary C-RNTI on PDCCH, and the NDI provided in the associated HARQ information has been toggled compared to the value in the previous transmission of this TB of this HARQ process; or 2> if the uplink grant was received on PDCCH for the C-RNTI and the HARQ buffer of the identified process is empty; or 2> if the uplink grant was received in a Random Access Response; or 2> if the uplink grant is part of a bundle of the configured uplink grant, and may be used for initial transmission according to subclause 6.1.2.3 of TS 38.214 [7], and if no MAC PDU has been obtained for this bundle: 3> if there is a MAC PDU in the Msg3 buffer and the uplink grant was received in a Random Access Response: 4> obtain the MAC PDU to transmit from the Msg3 buffer. 3> else if the uplink grant is a configured uplink grant on HARQ process 0 and ra-ContentionResolutionTimer is running: 4> skip the UL grant; 3> else: 4> obtain the MAC PDU to transmit from the Multiplexing and assembly entity, if any

- Option 2-c [7]: At the end of Section 5.4.1 UL Grant Reception, add a generic statement as follows:

5.4.1 UL Grant reception Uplink grant is either received dynamically on the PDCCH, in a Random Access Response, or configured semi-persistently by RRC. The MAC entity shall have an uplink grant to transmit on the UL-SCH. To perform the requested transmissions, the MAC layer receives HARQ information from lower layers. […] The MAC entity shall not consider the configured uplink grant associated with a HARQ process to be used for transmission of the Msg3 while the contention-based Random Access procedure is ongoing.

Note this solution is unclear because “shall not consider” is too vague, e.g. it does not say what to do with a bundle and also if the HARQ buffer should be flushed or not (as is currently the case, for example, in the HARQ entity when skipping a configured grant). In the following, we interpret it as same as Option 2-a, i.e. it filters out the 1st instance of a bundle only.

We compare the above solutions by assessing how well they address the issues listed in Section 2.1 in the below Table.

Table 1: Comparison of the solutions wrt issues to solve

| Issues / Solutions | Issue 1: HARQ Process#0 for dynamic or configured grants overridden by Msg3 PDU | Issue 2: HARQ Process#0 for Msg3 PDU overridden by dynamic or configured grant | Issue 3a: HARQ Process#0 for a transmission bundle overridden by Msg3 | Issue 3b: HARQ Process#0 for a transmission bundle flushed at RA completion |
| --- | --- | --- | --- | --- |
| Option 2-a | The collision with a configured grant is addressed by not sending a CG between RAR and Msg3. The collision with a dynamic grant is not addressed. | The collision with a configured grant is addressed by not sending a CG after Msg3. The collision with a dynamic grant is not addressed. | Not addressed when the CG bundle starts before the RAR. Not addressed at all for a bundle from a dynamic grant. | Not addressed when the CG bundle starts before the RAR. Not addressed at all for a bundle from a dynamic grant. |
| Option 2-b | Not addressed | The collision with a configured grant is addressed by not sending a CG after Msg3. The collision with a dynamic grant is not addressed. | Not addressed for a CG bundle because the current “filter” is only in the “initial transmission” part of the HARQ entity. Not addressed at all for a bundle from a dynamic grant. | Not addressed for a CG bundle because the current “filter” is only in the “initial transmission” part of the HARQ entity. Not addressed at all for a bundle from a dynamic grant. |
| Option 2-c | The collision with a configured grant is addressed by not sending a CG between RAR and Msg3. The collision with a dynamic grant is not addressed. | The collision with a configured grant is addressed by not sending a CG after Msg3. The collision with a dynamic grant is not addressed. | Not addressed when the bundle starts before RA was initiated. Not addressed at all for a bundle from a dynamic grant. | Not addressed when the bundle starts before RA was initiated. Not addressed at all for a bundle from a dynamic grant. |

As can be observed, none of the current solutions are satisfactory for addressing all issues raised in Section 2.1. Hence an improved solution must be derived.

2.2.2 Improved solutions

- Improved solution 1:

Across options 2-a/b/c we observed in Section 2.2.1 that options 2-a and 2-c where unclear and could be given different interpretations. On the other hand Table 1 shows that option 2-b does not address issue 1 but options 2-a and 2-c do. However, as discussed in Section 2.1, issue #1 is not critical and does not need to be fixed. Hence we focus on enhancing option 2-b so that it properly fixes issues #2 and #3. This is done by:

- Extending the current “filter” to also filter out dynamic scheduling grants when Msg3 was transmitted:

3> if there is a MAC PDU in the Msg3 buffer and the uplink grant was received in a Random Access Response: 4> obtain the MAC PDU to transmit from the Msg3 buffer. 3> else if the uplink grant is an uplink grant on HARQ process 0 not received in a Random Access Response and ra-ContentionResolutionTimer is running: 4> skip the UL grant; 3> else: 4> obtain the MAC PDU to transmit from the Multiplexing and assembly entity, if any;

- Extending the current “filter” in the “initial transmission” part of the HARQ entity to the “retransmission” part also, and adding a new condition to address the case where the HARQ buffer was flushed upon RA successful termination:

2> else (i.e. retransmission): 3> if the uplink grant received on PDCCH was addressed to CS-RNTI and if the HARQ buffer of the identified process is empty; or 3> if the uplink grant is part of a bundle and if no MAC PDU has been obtained for this bundle; or 3> if the uplink grant is part of a bundle of the configured uplink grant, and the PUSCH of the uplink grant overlaps with a PUSCH of another uplink grant received on the PDCCH for this Serving Cell; or 3> if the uplink grant is an uplink grant on HARQ process 0 not addressed to a Temporary C-RNTI on PDCCH and ra-ContentionResolutionTimer is running; or 3> if the uplink grant is an uplink grant on HARQ process 0 not addressed to a Temporary C-RNTI on PDCCH and the HARQ buffer of HARQ process 0 is empty: 4> ignore the uplink grant. 3> else:

We refer to this solution as “Improved solution - Option 1” and the resulting CR is given in [8].

- Improved solution 2:

All the above solutions are based on the same principle to “block” the uplink grant colliding with Msg3 on HARQ process 0. From the above discussion it is clear that many issues are resulting from this collision and require specific fixes for each. And we may have missed further issues.

This is why another approach is to simply let both transmissions happen in parallel by avoiding Msg3 to override the HARQ buffer. This can be very simple if we just use the existing Msg3 buffer as HARQ buffer when HARQ process #0 is used for Msg3 transmission. The proposed model is shown in Figure 4.

Figure 4: Reusing Msg3 buffer as HARQ buffer for Msg3 in HARQ Process #0

Figure 5 further shows how it fixes all issues raised so far. As can be seen, fixing issue 3-b also requires to no longer flush the HARQ buffer at RA completion, which is anyway not needed anymore since Msg3 no longer uses the HARQ buffer.

This approach removes the burden of managing the collisions and makes scheduler life much easier. And the other information (UL grant and HARQ info) associated to each (now fully independent) transmission is anyways managed independently in NR by the HARQ entity which explicitly provides it to the HARQ process on each transmission. Hence there is no possible confusion on which UL grant and HARQ info is associated with Msg3 and other HP0 transmissions.

Figure 5: All Issues are solved by improved solution 2

This solution is captured with the below simple changes to MAC specification:

### 5.1.5 Contention Resolution Contention Resolution is based on either C-RNTI on PDCCH of the SpCell or UE Contention Resolution Identity on DL-SCH. […] 1> if the Contention Resolution is considered not successful: 2> flush the HARQ buffer used for transmission of the MAC PDU in the Msg3 buffer; 2> increment PREAMBLE_TRANSMISSION_COUNTER by 1; 2> if PREAMBLE_TRANSMISSION_COUNTER = preambleTransMax + 1: 3> indicate a Random Access problem to upper layers. 3> if this Random Access procedure was triggered for SI request: 4> consider the Random Access procedure unsuccessfully completed. […] ### 5.1.6 Completion of the Random Access procedure Upon completion of the Random Access procedure, the MAC entity shall: - discard explicitly signalled contention-free Random Access Resources except contention-free Random Access Resources for beam failure recovery request, if any; 1> flush the HARQ buffer used for transmission of the MAC PDU in the Msg3 buffer. […] 5.4.2.2 HARQ process Each HARQ process is associated with a HARQ buffer. HARQ process zero uses the Msg3 buffer as HARQ buffer when generating a transmission for Msg3. New transmissions are performed on the resource and with the MCS indicated on either PDCCH, Random Access Response, or RRC. Retransmissions are performed on the resource and, if provided, with the MCS indicated on PDCCH, or on the same resource and with the same MCS as was used for last made transmission attempt within a bundle. If the HARQ entity requests a new transmission for a TB, the HARQ process shall: 1> if the MAC PDU was obtained from the Msg3 buffer: 2> use the MAC PDU from the Msg3 buffer; 1> else: 2> store the MAC PDU in the associated HARQ buffer; 1> store the uplink grant received from the HARQ entity; 1> generate a transmission as described below. […] NOTE: When referring to the HARQ buffer of the identified HARQ process in the rest of this specification, the Msg3 buffer is not included.

We refer to this solution as “Improved solution - Option 2” and the resulting CR is given in [9].

Proposal 4: Adopt the solution avoiding Msg3 to override the HARQ buffer by reusing the Msg3 buffer as independent HARQ buffer for Msg3 in HARQ Process #0.

Proposal 5: if proposal #4 is not acceptable, adopt the improved solution 1 to fix the issues.

- Conclusion

In this document, we analyzed the issues raised in last meeting regarding the operation on HARQ process 0 during RA procedure. We also highlighted new issues not discussed so far. Observations and proposals are found below:

Observation 1: MAC PDU from multiplexing and assembly entity in HARQ process #0 may be overridden by MAC PDU from Msg3 during dynamic scheduling as well as configured grant scheduling.

Observation 2: Upon receiving Msg3 gNB identifies the UE and can prevent the error case (PDU size mismatch) at the UE.

Proposal 1: The issue that MAC PDU from multiplexing and assembly entity is overridden by Msg3 is not a severe issue and does not need to be fixed.

Observation 3: If Msg3 is overridden by MAC PDU of dynamic scheduling or configured grant, the error case in the UE of UL grant/MAC PDU size mismatch upon Msg3 retransmission cannot be anticipated by the gNB.

Observation 4: Msg3 loss cannot be recovered and leaves no other option but RA re-attempt.

Proposal 2: The issue of Msg3 overridden by MAC PDU of dynamic scheduling or configured grant shall be fixed.

Observation 5: If a MAC PDU for a transmission bundle (from dynamic or configured grant) on HARQ process #0 is overridden by Msg3 MAC PDU or is flushed at RA completion, it will result in an error case in the UE of UL grant/MAC PDU size mismatch.

Observation 6: The above error cases cannot be anticipated by gNB.

Proposal 3: The issues of a transmission bundle (from dynamic or configured grant) on HARQ process #0 overridden by Msg3 MAC PDU or flushed at RA completion shall be fixed.

Proposal 4: Adopt the solution avoiding Msg3 to override the HARQ buffer by reusing the Msg3 buffer as independent HARQ buffer for Msg3 in HARQ Process #0.

Proposal 5: if proposal #4 is not acceptable, adopt the improved solution 1 to fix the issues.

- Reference

- R2-181xxxx, chairman notes of meeting, RAN2# 103 meeting;

- R2-1809504, Handling configured Grant Timer during random access procedure, ASUSTek, RAN2#AH1807 meeting;

- R2-1813851, “Msg3 grant overlapping with another UL grant”, CATT

- R2-1811215, Correction on Obtaining Configured Grant MAC PDU, CATT, 3GPP TSG-RAN WG2 Meeting 103;

- TS38.214-f30, Physical layer procedures for data, Release 15.

- R2-1811879, Correction on Configured Scheduling during Random Access Procedure, Ericsson, 3GPP TSG-RAN WG2 Meeting 103;

- R2-1812817, Skipping CG in RA procedure, LG Electronics Inc. 3GPP TSG-RAN WG2 Meeting 103;

- R2-1813854, 38321 CR310, Correction to fix collisions on HP#0 during Random Access Procedure - Option 1

- R2-1813855, 38321 CR422, Correction to fix collisions on HP#0 during Random Access Procedure - Option 2