3GPP TSG-RAN WG2 Meeting #103bis	R2-1813892

Chengdu, China, 8 – 12 October 2018

Agenda item:	11.6

Source: 	Huawei, HiSilicon

Title: 	Considerations on the impact of high propagation delay in NTN

Document for:	Discussion and Decision

Introduction

In the RAN plenary #80 meeting, a new RAN3 SI on “solutions evaluation for NR to support Non Terrestrial Network” was approved [1]. The objectives of the SI for Layer 2 and above, and RAN architecture are reported as follows:

- Study the following aspects and identify related solutions if needed: Propagation delay: Identify timing requirements and solutions on layer 2 aspects, MAC, RLC, RRC, to support non-terrestrial network propagation delays considering FDD and TDD duplexing mode. This includes radio link management. [RAN2]

- Handover: Study and identify mobility requirements and necessary measurements that may be needed for handovers between some non-terrestrial space-borne vehicles (such as Non Geo stationary satellites) that move at much higher speed but over predictable paths [RAN2, RAN1]

- Architecture: Identify needs for the 5G’s Radio Access Network architecture to support non-terrestrial networks (e.g. handling of network identities) [RAN3]

  - Paging: procedure adaptations in case of moving satellite foot prints or cells

In this contribution, we discuss the impact of high propagation delay in Non Terrestrial Network (NTN).

Discussion

The following tables show altitude and delay of different type satellites referring to [2].

Table 1 Propagation delays for different NGEO satellites (altitude and payload types)

|  |  | LEO at 600 km |  | LEO at 1500 km |  | MEO at 10000 km |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Elevation angle | Path | Distance D (km) | Delay (ms) | Distance D (km) | Delay (ms) | Distance D (km) | Delay (ms) |
| UE: 10° | satellite - UE | 1932.24 | 6,440 | 3647.5 | 12,158 | 14018.16 | 46.727 |
| GW: 5° | satellite - gateway | 2329.01 | 7.763 | 4101.6 | 13.672 | 14539.4 | 48.464 |
| 90° | satellite - UE | 600 | 2 | 1500 | 5 | 10000 | 33.333 |
| Bent pipe satellite |  |  |  |  |  |  |  |
| One way delay | Gateway-satellite_UE | 4261.2 | 14.204 | 7749.2 | 25.83 | 28557.6 | 95.192 |
| Round Trip Delay | Twice | 8522.5 | 28.408 | 15498.4 | 51.661 | 57115.2 | 190.38 |
| Regenerative satellite |  |  |  |  |  |  |  |
| One way delay | Satellite -UE | 1932.24 | 6.44 | 3647.5 | 12.16 | 14018.16 | 46.73 |
| Round Trip Delay | Satellite-UE-Satellite | 3864.48 | 12.88 | 7295 | 24.32 | 28036.32 | 93.45 |

Table 2 Propagation delays for GEO satellite at 35786 km

|  | GEO at 35786 km |  |  |
| --- | --- | --- | --- |
| Elevation angle | Path | D (km) | Time (ms) |
| UE :10° | satellite - UE | 40586 | 135.286 |
| GW : 5° | satellite - gateway | 41126.6 | 137.088 |
| 90° | satellite - UE | 35786 | 119.286 |
| Bent Pipe satellite |  |  |  |
| One way delay | Gateway-satellite_UE | 81712.6 | 272.375 |
| Round trip Time | Twice | 163425.3 | 544.751 |
| Regenerative Satellite |  |  |  |
| One way delay | Satellite -UE | 40586 | 135.286 |
| Round Trip Time | Satellite-UE-Satellite | 81172 | 270.572 |

As shown in above tables, the round trip delay of MEO at 10000 Km is up to 93.45ms, and the round trip delay rises to 270.572ms for GSO satellite. This means extremely long RTT comparing to terrestrial networks and will have a large impact on HARQ process, handover and MAC/RLC procedures.

- Impact on HARQ

According to RAN1’s evaluation, the required number of HARQ processes of GEO and LEO with considering 1ms slot duration and 15kHz SCS are 600 and 50, respectively. Obviously, in NTN, the RTT normally exceeds the maximum conventional HARQ process. However, it seems not feasible for some UEs to simply extend the number of HARQ processes because of memory restrictions and limited processing capacity. To handle this, RAN1 considers two study principles.

-	Enhancing existing HARQ operation to extend the HARQ processing accommodating low to moderate NTN RTT delays.

-	Limiting HARQ capabilities and/or disabling HARQ for long RTT delays.

Hence, RAN2 should also follow the two principles to study HARQ.

Proposal 1: RAN2 should also follow the two principles to study HARQ.

- Impact on MAC/RLC procedures

During RACH procedure, UE receives RAR message at RAR window and the RAR window will start at one offset after UE transmitting preamble. In NTN, the RAR window start time should be later because of high RTT. RAN2 may need to study RAR window. Moreover, ARQ requires that the transmitted packets be buffered in anticipation of potential packet loss and released only after the successful receipt of an acknowledgement, or until a time-out mechanism reinitiating a retransmission. The long RTT requires larger transmission buffer and maximum SN value, and longer reassemble timer. Furthermore, long RTT potentially limits the number of retransmission allowed for each transmitted packet in both the forward and return links. Hence, RAN2 should study the ARQ buffer size, the maximum SN value, reassemble timer length, and retransmission mechanism for long RTT.

Proposal 2: RAN2 should study RAR window, the ARQ buffer size, the maximum SN value, reassemble timer length, and retransmission mechanism for long RTT.

- Impact on Handover

The handover procedure, including measurement configuration, measurement results report, handover decision, handover preparing, and RRC reconfiguration steps, is time-sensitive to avoid data transmission interruption. The high propagation delay causes high measurement configuration and report delay, and high RRC reconfiguration delay, both of which will result in high handover delay. High handover delay may bring many issues, e.g., handover failure. Hence, RAN2 should study possible handover issues due to high handover delay.

Proposal 3: RAN2 should study possible handover issues due to high handover delay, e.g., handover failure

Conclusion

In this contribution, we discussed the impact of high propagation delay in Non Terrestrial Network (NTN), and we get the following observation and proposals:

Proposal 1: RAN2 should also follow the two principles to study HARQ.

Proposal 2: RAN2 should study RAR window, the ARQ buffer size, the maximum SN value, reassemble timer length, and retransmission mechanism for long RTT.

Proposal 3: RAN2 should study possible handover issues due to high handover delay, e.g., handover failure.

References

[1]		RP-181370, Study on solutions evaluation for NR to support Non Terrestrial Network.

[2]		TR 38.811, Study on New Radio (NR) to support non terrestrial networks.