3GPP TSG-RAN WG2#103bis	R2-1814090

Chengdu, PR China, 8th – 12th October 2018

| CR-Form-v11.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.331 | CR | 0254 | rev | - | Current version: | 15.3.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | x | Radio Access Network | x | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Access barring check after handover |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Ericsson |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R2 |  |  |  |  |  |  |  |  |  |
| Work item code: | NR_newRAT-Core |  |  |  |  |  |  | Date: |  | 2018-09-27 |
| Category: | F |  |  |  |  |  |  | Release: |  | Rel-15 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) Rel-12 (Release 12) Rel-13 (Release 13) Rel-14 (Release 14) Rel-15 (Release 15) Rel-16 (Release 16) |  |
| Reason for change: |  | In RAN2#103 it was agreed that after a handover in RRC_CONNECTED the UE cannot perform barring check until it has valid UAC information (from SIB1 for NR or SIB25 for eLTE) from the target cell. This CR is also related to RIL comment I612. |  |  |  |  |  |  |  |  |
| Summary of change: |  | A note is added clarifying that barring check cannot be performed until the UAC information from SIB1 of the target cell has been acquired. Impact Analysis Impacted functionality: Access control Inter-operability: If the CR is implemented by the UE but not by the gNB, there is no inter-operability issue foreseen. If the CR is implemented by the gNB but not by the UE, there is no inter-operability issue foreseen. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | The access barring check procedure after handover is not fully specified which may lead to inconsistent UE behaviour. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 5.3.14 Unified Access Control |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  | TS/TR ... CR ... |  |  |  |
| affected: |  |  | X | Test specifications |  |  | TS/TR ... CR ... |  |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  | TS/TR ... CR ... |  |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |

START OF CHANGES

### 5.3.14	Unified Access Control

#### 5.3.14.1	General

The purpose of this procedure is to perform access barring check for an access attempt associated with a given Access Category and one or more Access Identities upon request from upper layers according to TS 24.501 [23] or the RRC layer.

NOTE: 	After a handover resulting in change of PCell in RRC_CONNECTED the UE does not perform access barring check until it has obtained valid UAC information (from SIB1) from the target cell. Access barring checks thus need to be postponed until SIB1 has been acquired.

END OF CHANGES