3GPP TSG-RAN WG2 #103bis	R2-1814112

Chengdu, China, 8th October – 12th October 2018

Agenda Item:	12.1.6

Source:	Ericsson

Title:	Support of quality report in Msg3 for MTC

Document for:	Discussion, Decision

# Introduction

In the work item description (WID) of Rel-16 MTC enhancements for LTE, the following objectives are listed as improvements of DL transmission efficiency and/or UE power consumption [1].

- Specify support for mobile-terminated (MT) early data transmission (EDT) [RAN2, RAN3]

- Specify quality report in MSG3 at least for EDT [RAN1, RAN2]

- Specify MPDCCH performance improvement by using CRS at least for connected mode [RAN1, RAN2, RAN4]

- Specify support for UE-group wake-up signal (WUS) [RAN1, RAN2, RAN4]

In this paper we discuss the support of Msg3 quality reporting in Msg3 at least for EDT.

# Discussion

Recently for NB-IoT the support for anchor carrier measurement reporting in Msg3 has been specified. 4 bits have been used as below.

-- ASN1START

CQI-NPDCCH-NB-r14 ::=	ENUMERATED {

noMeasurements, candidateRep-A, candidateRep-B, candidateRep-C,

candidateRep-D, candidateRep-E, candidateRep-F, candidateRep-G,

candidateRep-H, candidateRep-I, candidateRep-J, candidateRep-K,

candidateRep-L}

-- ASN1STOP

Further, a short CQI reporting is also supported in RRCConnectionReestablishmentRequest-NB as below.

-- ASN1START

CQI-NPDCCH-Short-NB-r14 ::=	ENUMERATED {

noMeasurements, candidateRep-1, candidateRep-2, candidateRep-3}

-- ASN1STOP

It is expected that similar measurement reporting as above would be required for MTC. However, it is up to RAN1 and RAN4 to decide.

From RAN2 perspective, the below messages, for example would be impacted for EDT and non-EDT case respectively [2].

RRCEarlyDataRequest message

-- ASN1START

RRCEarlyDataRequest-r15 ::=		SEQUENCE {

criticalExtensions					CHOICE {

rrcEarlyDataRequest-r15				RRCEarlyDataRequest-r15-IEs,

criticalExtensionsFuture			SEQUENCE {}

}

}

RRCEarlyDataRequest-r15-IEs ::=	SEQUENCE {

s-TMSI-r15						S-TMSI,

establishmentCause-r15			ENUMERATED {	mo-Data-r15, delayTolerantAccess-r15},

dedicatedInfoNAS-r15				DedicatedInfoNAS,

nonCriticalExtension				SEQUENCE {}			OPTIONAL

}

-- ASN1STOP

-- ASN1START

RRCConnectionRequest ::=			SEQUENCE {

criticalExtensions					CHOICE {

rrcConnectionRequest-r8				RRCConnectionRequest-r8-IEs,

criticalExtensionsFuture			SEQUENCE {}

}

}

RRCConnectionRequest-r8-IEs ::=		SEQUENCE {

ue-Identity							InitialUE-Identity,

establishmentCause					EstablishmentCause,

spare								BIT STRING (SIZE (1))

}

InitialUE-Identity ::=				CHOICE {

s-TMSI								S-TMSI,

randomValue							BIT STRING (SIZE (40))

}

EstablishmentCause ::=				ENUMERATED {

emergency, highPriorityAccess, mt-Access, mo-Signalling,

mo-Data, delayTolerantAccess-v1020, mo-VoiceCall-v1280, spare1}

-- ASN1STOP

It should be possible to extend the above message using non-critical extension so that UE can include the measurement results. However, the parameters (granularity and number of bits required) should be provided by RAN1/RAN4.

The EDT Msg3 may have more flexibility because the UL grant may be high as data also needs to be included.

However, legacy Msg3 for MTC also does not have big size constraint, it should be possible to have granular reporting.

From 36.213

Table 3: TBS table for Msg3 in CE mode A

| IMCS | NPRB |  |  |  |
| --- | --- | --- | --- | --- |
|  | 1 | 2 | 3 | 6 |
| 0 | 16 | 32 | 56 | 152 |
| 1 | 24 | 56 | 88 | 208 |
| 2 | 32 | 72 | 144 | 256 |
| 3 | 40 | 104 | 176 | 328 |
| 4 | 56 | 120 | 208 | 408 |
| 5 | 72 | 144 | 224 | 504 |
| 6 | 328 | 176 | 256 | 600 |
| 7 | 104 | 224 | 328 | 712 |

Table 4: TBS table for Msg3 in CE mode B

| ITBS | NPRB |  |
| --- | --- | --- |
|  | 1 | 2 |
| 0 | 56 | 152 |
| 1 | 88 | 208 |
| 2 | 144 | 256 |
| 3 | 176 | 328 |

There are enough bits in Msg3 for channel quality reporting in both the non-EDT    case and the EDT case.

Wait for RAN1 and RAN4 to progress on this to decide upon the exact number of bits and granularity needed to report the measurement quality and update the above messages accordingly.

# Conclusion

In section 2 we made the following observations:

Observation 1	It is expected that similar measurement reporting as above would be required for MTC. However, it is up to RAN1 and RAN4 to decide.

Observation 2	There are enough bits in Msg3 for channel quality reporting in both the non-EDT case and the EDT case.

Based on the discussion in section 2 we propose the following:

Proposal 1	Wait for RAN1 and RAN4 to progress on this to decide upon the exact number of bits and granularity needed to report the measurement quality and update the above messages accordingly.

# References

RP-181450, “New WID on Rel-16 MTC enhancements for LTE”, source Ericsson, 3GPP TSG RAN Meeting #80, La Jolla, USA, June 11-14, 2018.

3gpp 36.331, RRC v15.3.0