3GPP TSG-RAN WG2#103bis     	                                                    R2-1814171

Chengdu, China, 8th – 12th October 2018

Source:             ZTE

Title:                 Discussion on LTE and NR Uu based resource allocation and configuration for sidelink

Agenda item:    11.4.3

Document for:  Discussion

Introduction

In RAN#80, the new SI named “Study on NR V2X” was agreed to study the enhancement to support advanced V2X services captured in TR 22.886 [1]. The following objective for Uu-based sidelink resource allocation/configuration are identified [2]:

3: Uu-based sidelink resource allocation/configuration (LTE V2X Mode3-like and Mode4-like) [RAN1, RAN2]: - Identify necessary enhancements of LTE Uu and NR Uu to control NR sidelink from the cellular network - Identify necessary enhancements of NR Uu to control LTE sidelink from the cellular network

And the following agreements for NR sidelink resource allocation mechanism were achieved at RAN1#94 meeting.

At least two sidelink resource allocation modes are defined for NR-V2X sidelink communication -Mode 1: Base station schedules sidelink resource(s) to be used by UE for sidelink transmission(s) -Mode 2: UE determines (i.e. base station does not schedule) sidelink transmission resource(s) within sidelink resources configured by base station/network or pre-configured sidelink resources

In this contribution, we will discuss the potential requirements and enhancements of LTE Uu to control NR sidelink and NR Uu to control LTE sidelink. And our views on the potential enhancements are provided.

Discussion

Network coverage and resource allocation cases

During the initial stage of NR network deployment, the NR coverage might be limited. It is very likely that the LTE network has broader coverage than NR network coverage. But in the future, we cannot exclude the scenario there is only NR cellular network. So there are several network coverage scenarios as below:

- Scenario 1: V2X UE is only in coverage of LTE network

- Scenario 2: V2X UE is in coverage of both LTE network and NR network

- Scenario 3: V2X UE is only in coverage of NR network

Suppose a UE supporting V2X application is only in coverage of LTE cell, UE might still prefer to transmit the V2X messages of advanced V2X services via NR V2X, which is expected to fulfil lower latency and higher reliability requirements.  Similarly, if the UE is only in coverage of NR cell, the V2X message of LTE related V2X applications still need to be sent via LTE V2X. In order to support above scenarios and requirements, two cases of cross-RAT sidelink resource allocation shall be considered as below. We will discuss them one by one.

Case 1: The LTE network controls the resource allocation of the NR sidelink;

Case 2: The NR network controls the resource allocation of the LTE sidelink

Uu based sidelink resource allocation

The LTE network controls the resource allocation of the NR sidelink

It is agreed that at least two sidelink resource allocation modes are defined for NR V2X sidelink communication at RAN1#94 meeting. For NR sidelink resource allocation mode 1, base station schedules sidelink resource(s) to be used by UE for sidelink transmission(s). For NR sidelink resource allocation mode 2, UE determines (i.e. base station does not schedule) sidelink transmission resource(s) within sidelink resources configured by base station/network or pre-configured sidelink resources. In our opinion, at least NR sidelink resource allocation mode 2 could be supported by LTE network. To be specific, the NR sidelink resource pools configuration can be transmitted via Uu signalling for only LTE coverage scenario. It means new signalling should be designed to configure NR sidelink resource pools and other parameters via LTE Uu, e.g., a new SIB for NR sidelink should be defined on LTE Uu. Moreover, if the LTE and NR sidelink share the same frequency resource, the LTE can coordinate resource usage between LTE sidelink and NR sidelink.

Proposal 1: The LTE network should provide the NR sidelink resource pool for NR V2X capable UE.

Proposal 2: New broadcast system information and dedicated signalling should be designed to support NR sidelink resource configuration via LTE Uu.

Compared to NR, the SR/BSR procedure of LTE is time consuming since LTE RATs have lower SR time domain density than NR RATs. However, since one of the requirements of advanced V2X services is to support low latency e.g., <=3ms, it is a challenging to support the requirement of low latency services via LTE Uu-based resource allocation. On the other hand, since multiple SCSs/numerologies should be supported in NR V2X while only one SCS value is supported in LTE (15KHz), it may also need a lot of modifications and enhancements to the LTE Uu to support NR sidelink resource allocation mode 1. Therefore, the benefit of the LTE Uu based dynamic allocate NR sidelink resource and the impacts on LTE specification should be evaluated. And it is suggested to deprioritize the LTE based NR sidelink resource scheduling.

Observation 1: It is a challenging to support the requirement of advanced V2X services via LTE Uu-based resource allocation.

Proposal 3: The LTE network scheduling NR sidelink resource should be deprioritized.

The NR network controls the LTE sidelink

Generally speaking, the mechanism of LTE sidelink resource allocation and resource pool configuration shall be the baseline of the NR sidelink. In order to support the NR Uu based LTE sidelink resource allocation mode 1, a new NR DCI format with the same information as LTE DCI 5A to schedule LTE sidelink mode 3 transmission need to be designed. For sidelink resource allocation mode 2, NR Uu RRC signaling should support the LTE sidelink resource pools configuration. Furthermore, if the LTE and NR sidelink share the same frequency resource, the gNB can also coordinate resource pools between LTE sidelink and NR sidelink.

Proposal 4: The mechanism of LTE sidelink resource allocation and resource pool configuration shall be the baseline of the NR sidelink.

Proposal 5: To support the NR Uu based LTE sidelink resource allocation, the NR network should provide the LTE sidelink resource pool configuration for the UEs.

For the NR Uu based mode 1 LTE sidelink resource allocation, the resource request mechanism shall be studied. Based on the requirements listed in TR22.886, if the V2X UE supports both LTE and NR, the V2X UE should choose the best RAT to support the given application of interest. That means, the 3GPP network can associate a V2X application to one or more 3GPP RATs. Therefore, for the UE supporting different V2X application, it may support transmitting V2X messages in NR and LTE RAT simultaneously. When the UE requests to the sidelink resource, it is necessary for the gNB to know which RAT sidelink resource are requested.

Proposal 6: The UE may support transmitting V2X messages in NR and LTE RAT simultaneously. So it is necessary for the gNB to know which RAT sidelink resources are requested.

Sidelink resource allocation for dual-connectivity scenario

When a V2X UE is in coverage of both LTE and NR networks, the V2X UE may be dual-connected to an eNB and a gNB. In this scenarios, there are several cases for V2X sidelink resource allocation.

- Case 1: The LTE network control its LTE V2X resource allocation, and the NR network control its NR V2X sidelink resource allocation separately. That means the basic V2X service is served by the LTE cell group and the advanced V2X services is served by the NR cell group.

- Case 2:  Only the MCG allocate the sidelink resource, including LTE resource and NR resource. It is suitable for NE-DC scenario. For EN-DC, as analyzed in the section 2.1, the LTE Uu based dynamic NR sidelink resource allocation is difficult to realize.

- Case 3:  For both EN-DC and NE-DC, only the gNB allocate the V2X resource, including LTE V2X resource and NR V2X resource. Based on the analysis in section 2.2, gNB should provide both the LTE sidelink resource pool and NR sidelink resource pool for the UEs, and scheduling LTE and NR sidelink resource is supported.

Proposal 7: It is suggested to consider sidelink resource allocation mechanism for dual-connectivity scenario.

Conclusion

In this paper, the the potential enhancements of LTE Uu to control NR sidelinkand and NR Uu to control LTE sidelink were discussed and the following observation and proposals are given:

Observation 1: It is a challenging to support the requirement of advanced V2X services via LTE Uu-based resource allocation.

Proposal 1: The LTE network should provide the NR sidelink resource pool for NR V2X capable UE.

Proposal 2: New broadcast system information and dedicated signalling should be designed to support NR sidelink resource configuration via LTE Uu.

Proposal 3: The LTE network scheduling NR sidelink resource should be deprioritized.

Proposal 4: The mechanism of LTE sidelink resource allocation and resource pool configuration shall be the baseline of the NR sidelink.

Proposal 5: To support the NR Uu based LTE sidelink resource allocation, the NR network should provide the LTE sidelink resource pool configuration for the UEs.

Proposal 6: The UE may support transmitting V2X messages in NR and LTE RAT simultaneously. So it is necessary for the gNB to know which RAT sidelink resources are requested.

Proposal 7: It is suggested to consider sidelink resource allocation mechanism for dual-connectivity scenario.

Reference

- 22.886-g00, Study on enhancement of 3GPP Support for 5G V2X Services, 3GPP.

- RP-181480, New SID: Study on NR V2X. 3GPP TSG RAN #80 Meeting, June 11th -14th, 2018.