Multi-user packet PUSCH mode design

1	Introduction

In RAN1#93 meeting, some agreements were reached on UL HARQ-ACK feedback for eMTC. But there is still no good solution on how to support HARQ-ACK feedbacks for multiple UEs in a single common DCI[1].

In this contribution, the left issues on UL HARQ-ACK feedback are discussed.

2	Discussion

Considering the signaling overhead and resource consumption reduction, group-based HARQ-ACK feedback to multiple UEs in a DCI should be supported by MPDCCH. But how to support HARQ-ACK feedbacks for multiple UEs in a single common DCI still failing to produce an effective solution at previous meeting.

In some contributions, the new UE specific search space was introduced to provide HARQ-ACK feedbacks to multiple UEs. But the drawbacks in using this scheme are[2]:

- The eNB would need to group UEs in the same coverage level so that their feedback can be transmitted in the same DCI since it is not efficient to multiplex UEs in good coverage with those in poor coverage in a single DCI.

- It is unclear how this can be used for early termination of PUSCH repetition.  It is challenging for the scheduler to ensure that all PUSCH end at the same time including those with early PUSCH termination.

- Multiplexing of multiple UEs would lead to a larger than required DCI size leading to higher repetitions for each individual UE.

So multiplexing UL HARQ-ACK feedbacks for multiple UEs in a single common DCI is not efficient and may increase scheduler complexity.

In an MTC user-intensive scenario, if the channel conditions are poor, the success rate of each MTC user uploading data is not high. In the case of an upload failure, repeated retransmissions are caused, which results in a great waste of PUSCH channel resources and MPDCCH channel resources, and not conducive to the energy saving of MTC users. Considering the large number of UEs in the MTC scenario, the service scenarios are different. The scene of MTC users should be classified first, then the PUSCH method and the HARQ-ACK feedbacks method should be suitable for specifically considered.

For example, let MTC users choose a more appropriate way to upload data according to channel quality and intensity. More dispersed MTC users upload their own data to the base station directly through the PUSCH. The MTC users with high degree of aggregation are grouped, each group has one relay, accepts the data of other MTC users in the group, and then it transmits the data to the base station, so that the channel resources of the MPDCCH such as HARQ-ACK feedbacks can be saved.

Proposal 1: A relay demodulation device may be selected for the MTC device to transmit information to the base station.

3	Conclusion

In this contribution, we provide the analysis of possible UL HARQ feedback schemes based on MPDCCH. The following proposals are made.

Proposal 1: A relay demodulation device may be selected for the MTC device to transmit information to the base station.

4	Reference

- R1-1807465,”Summary of Uplink HARQ-ACK Feedback for MTC”	ZTE

- R1-1807245, “Remaining issues in explicit uplink HARQ-ACK feedback”, Sony