3GPP TSG-RAN WG2 Meeting 103bis	R2-1814276

Chengdu, China, 8th – 12th October, 2018

Agenda item:		11.4.2.4

Source:	Hyundai Motors

Title: 	Discussion on NR zone design for sidelink resource allocation considering vehicle speed

Document for:	Discussion and Decision

1. Introduction

The new study item on NR V2X [1] was approved in RAN#80 meeting and one of the objectives for NR sidelink design are described as follows.

1: Sidelink design [RAN1, RAN2]: - Identify technical solutions for a NR sidelink design to meet the requirements of advanced V2X services, including - Study the support of sidelink unicast, sidelink groupcast and sidelink broadcast - Study NR sidelink physical layer structures and procedure(s) - Study sidelink synchronization mechanism - Study sidelink resource allocation mechanism (also including objective 3) - Study sidelink L2/L3 protocols NOTE: Only the performance of advanced V2X use cases will be evaluated in the design of NR sidelink.

In this contribution, we discuss the aspect of NR zone design for flexible sidelink resource allocation considering varying vehicle speed.

2. Discussion

In [1], it is described that NR V2X is needed to support the advanced use cases explained in [2]. For the advanced use cases listed in [2], the expected requirements to meet the needed latency, reliability and throughput are made more stringent together with the vehicle speed up to 250km/h. Contrary to NR-V2X, LTE-V2X was made considering the use cases with less stringent requirements. One of the features introduced for sidelink resource allocation in LTE V2X was “Zone”. In LTE V2X, the resource pool was divided based on the geographical regions and the vehicle UEs in an area can only utilize the resource pool that have been assigned to the area. As described in [3], the geographical zones can be configured by eNB or pre-configured. That is, the vehicle UEs can identify the zones by using either the information provided by the eNB when the vehicle UEs are in coverage or the information pre-configured when the vehicle UEs are in out of coverage.

We think the zone concept for sidelink resource allocation in LTE V2X can be inherited for NR V2X with additional enhancement, for example, how to handle the vehicle UEs with varying speed up to 250km/h. Typically, the vehicle speed can change very often in urban/highway scenarios. We can assume that the mode 3 vehicle UEs are in very high speed when they obtain the sidelink resources assigned to the current location from gNB. However, the sidelink resources associated with the zone should be updated shortly to the sidelink resources associated with the new zone because the sidelink resource allocated in old zone cannot be used long enough for the vehicle UEs with high speed. With only periodical geo-location reporting, we think the gNB cannot trace the precise location of high speed vehicle UEs. The result is that the vehicle UEs cannot obtain the up-to-date sidelink resources associated with the changed location and use the SL resources obtained from old zone since the gNB cannot properly allocate the sidelink resources until it receives the geo-location reporting in the next period. Since the NR coverage would be much smaller than the LTE coverage, the frequent zone update would be inevitable for the NR vehicle UEs with high speed. Thus, the zone based resource allocation in LTE V2X may not work well in NR.

Proposal. RAN2 is asked to discuss and investigate a solution on NR zone design considering varying vehicle speed for flexible resource allocation.

3.  Conclusion

In this contribution, the following is proposed.

Proposal. RAN2 is asked to discuss and investigate a solution on NR zone design considering varying vehicle speed for flexible resource allocation.

4.  References

- RP-181429, “Study on NR V2X”, Vodafone,

- 3GPP TR 22.886, https://3gpp.org

- 3GPP TS 36.300, https://3gpp.org