3GPP TSG-RAN WG2 Meeting #103bis                                                   	R2-1814341

Chengdu, P.R. China, October 8th – 12th 2018

Agenda Item:	12.1.8

Source: 	Ericsson

Title: 	CE mode improvements for LTE-MTC non-BL UEs

Document for: 	Discussion and Decision

# 1	Introduction

At the RAN plenary #80, a new Work Item (WI) on “Rel-16 MTC enhancements” was approved [1]. The WID states that one objective is to specify CE mode A and B improvements for non-BL UEs from among the techniques listed below:

The objective is to specify the following set of improvements for machine-type communications for BL/CE UEs. Extreme coverage for non-BL UEs: - Specify CE mode A and B improvements for non-BL UEs from among the following list [RAN1, RAN2, RAN4] - Enhancements to idle mode mobility - UE demodulation performance requirements for 2 RX antennas and full duplex FDD - Dual layer DL reception - Feedback based on CSI-RS - ETWS/CMAS in connected mode

This contribution provides an analysis on each of the potential enhancements listed in the WID’s objective.

# 2	Candidates for achieving CE mode A and B improvements for non-BL UEs

Below we provide a brief description for each of the potential enhancements as listed in the WID for achieving CE mode A and B improvements for non-BL UEs and consider which WG should lead the discussion for each of them.

## 2.1	Enhancements to idle mode mobility

Support of UEs in Enhanced Coverage is described in 36.300 clause 23.7b [2]. Below are provided few extracts from that clause regarding the behaviour of UE capable of enhanced coverage (whilst not necessarily being BL UE):

A UE capable of enhanced coverage acquires, if needed, and uses legacy system information when in normal coverage if it is not a BL UE. A UE capable of enhanced coverage acquires, if needed, and uses system information specific for UEs in enhanced coverage.

A UE in enhanced coverage camps on a suitable cell where S criterion for UEs in enhanced coverage is fullfilled. The UE shall re-select to inter-frequency cells in which it is able to operate in normal coverage over cells in which it has to be in enhanced coverage.

RAN2 should consider enhancements to the behavior of CE capable non-BL UE regarding the transitions between normal coverage and CE mode operations to improve the performance. For example:

-	Even if the S criterion is passed for normal coverage, should it be possible for the UE to switch to CE mode operation under some circumstances;

-	Whether the UE should monitor only PDCCH/MPDCCH or both for paging;

-	Is the UE required to have only one set of valid SI, or is the UE required to read both, normal and -BR versions of SI.

Enhancements to idle mode mobility of non-BL UEs in CE are RAN2-related, and thus should be discussed lead by RAN2.

## 2.2	UE demodulation performance requirements for 2RX antennas and full duplex FDD

“UE demodulation performance requirements for 2 RX antennas and full duplex FDD” has also been listed as a potential candidate for improving CE Mode A and B for non-BL UEs. The Technical Specification 36.101, which is under RAN4 responsibility, contains already several requirements touching upon this description from Rel-13 (e.g., clause 8.11.1.1.2, “Closed-loop spatial multiplexing performance”, which assumes 2Rx for CE Mode A and clause 8.11.1.1.3, “Transmit diversity performance”, which assumes 2Rx for CE Mode B) [4]. Indeed, RAN4 has already covered performance requirements related to “CEMode A/B + 2Rx/4Rx + FDD/HD-FDD/TDD,” as can be seen from clause 8.1.2.8A of [4].

RAN4 has already standardized performance requirements related to “CEMode A/B + 2Rx/4Rx + FDD/HD-FDD/TDD”. Hence, any further “UE demodulation performance requirement” concerning this candidate may only be triggered if a new feature touching upon “2 RX antennas and full duplex FDD” would be standardized.

No RAN2 impact identified for “UE demodulation performance requirements for 2RX antennas and full duplex FDD”.

## 2.3	Dual layer DL reception

Aiming at increasing the data rate, multiple data layers are transmitted simultaneously in the same frequency range via multiple transmit antennas to multiple receive antennas. In a dual-layer transmission scheme, two data streams are transmitted over Nt ≥ 2 antennas, while the receiver detects the transmitted streams using Nr ≥ 2 receive antennas.

In our view dual layer DL reception is in principle a RAN1-related feature and therefore it should be it should be discussed and lead in RAN1.

The feasibility of Dual layer DL reception should be lead and studied in RAN1.

No direct RAN2 impact identified on “Dual layer DL reception”.

## 2.4	Feedback based on CSI-RS

In Rel-10, up to 8 layers were made available for spatial multiplexing, which created a need for 8-layer channel estimation since the Cell-specific RS (CRS) was originally designed for up to 4-layer spatial multiplexing. This was the reason the Channel State Information Reference Signal (CSI-RS) was introduced, which is used to provide channel estimation for up to 8 layers (antenna ports). One important aspect to mention is that due to the sparse placement of CSI-RS (as to minimize the interference on UEs that perceive CSI-RS as interference), the CSI-RS-based CSI reports will be reported over longer time intervals than CRS-based CSI reports.

CSI reporting based on CSI-RS is RAN1 related, and therefore it should be discussed and lead in RAN1.

Feedback based on CSI-RS should be lead and discussed in RAN1.

No direct RAN2 impact identified on "Feedback based on CSI-RS".

## 2.5	ETWS/CMAS in connected mode

The last of the listed candidates for improving non-BL UEs in CE Mode A and B is “ETWS/CMAS in connected mode”. ETWS stands for Earthquake and Tsunami Warning System, which is a public warning system aimed at broadcasting emergency information to users within a specific area in situations such as an Earthquake or a Tsunami. CMAS stands for Commercial Mobile Alert System, which can also be used e.g. for notifications and warnings in emergency situations. ETWS and CMAS mechanisms are quite similar and specified in RAN2, thus RAN2 should lead the discussion in ETWS/CMAS improvements.

ETWS/CMAS improvements in connected mode should be lead and discussed in RAN2.

ETWS/CMAS indication is a rare occurrence signalled only in emergency situations. Despite being rare, when signalled, it is critical that the ETWS/CMAS capable UEs acquire the indication as soon as possible. Missing the indication or even having too long delay to receive the indication may cause the emergency situation to become even more dangerous for people relying on these indications. This could be prevented by receiving the indication more quickly.

Current mechanism for ETWS/CMAS capable UEs in CE in RRC_CONNECTED to receive the ETWS/CMAS indication is that the network releases the UE to idle mode and pages the UE with the indication set. However, this is not preferred solution due to the delay caused by the procedures and the increased paging load caused by a large number of ETWS/CMAS capable UEs in a certain emergency area to be paged simultaneously, which, when taking into account also the coverage enhanced operation, may cause that part of those UEs may miss the paging altogether.

Current solution of releasing the UEs to idle mode for ETWS/CMAS notification reception for UEs in CE in RRC_CONNECTED is inefficient.

Therefore, RAN2 should come up with better solution to inform ETWS/CMAS UEs in CE when in RRC_CONNECTED about activation of ETWS/CMAS. Some options for such solutions include requiring the UE to monitor SI change/paging/direct indication in RRC_CONNECTED or defining new message/signalling for ETWS/CMAS which would be received by the UEs in RRC_CONNECTED.

RAN2 to specify a new solution for ETWS/CMAS capable UEs using enhanced coverage functionality when in RRC_CONNECTED to receive ETWS/CMAS notification.

# 3	Conclusion

In the previous sections we made the following observations and proposals:

Proposal 1	Enhancements to idle mode mobility of non-BL UEs in CE are RAN2-related, and thus should be discussed lead by RAN2.

Observation 1	RAN4 has already standardized performance requirements related to “CEMode A/B + 2Rx/4Rx + FDD/HD-FDD/TDD”. Hence, any further “UE demodulation performance requirement” concerning this candidate may only be triggered if a new feature touching upon “2 RX antennas and full duplex FDD” would be standardized.

Observation 2	No RAN2 impact identified for “UE demodulation performance requirements for 2RX antennas and full duplex FDD”.

Observation 3	The feasibility of Dual layer DL reception should be lead and studied in RAN1.

Observation 4	No direct RAN2 impact identified on “Dual layer DL reception”.

Observation 5	Feedback based on CSI-RS should be lead and discussed in RAN1.

Observation 6	No direct RAN2 impact identified on "Feedback based on CSI-RS".

Observation 7	ETWS/CMAS improvements in connected mode should be lead and discussed in RAN2.

Observation 8	Current solution of releasing the UEs to idle mode for ETWS/CMAS notification reception for UEs in CE in RRC_CONNECTED is inefficient.

Proposal 2	RAN2 to specify a new solution for ETWS/CMAS capable UEs using enhanced coverage functionality when in RRC_CONNECTED to receive ETWS/CMAS notification.

# 4	References

- RP-181878, “Revised WID: Additional MTC enhancements for LTE”, Ericsson, RAN #81, September 10–13, 2018.

- TS 36.000, Evolved Universal Terrestrial Radio Access (E-UTRA) and Evolved Universal Terrestrial Radio Access Network (E-UTRAN); Overall description; Stage 2.

- TS 36.304, Evolved Universal Terrestrial Radio Access (E-UTRA); User Equipment (UE) procedures in idle mode.

- TS 36.101, Evolved Universal Terrestrial Radio Access (E-UTRA); User Equipment (UE) radio transmission and reception.

- TS 36.331, Evolved Universal Terrestrial Radio Access (E-UTRA); Radio Resource Control (RRC); Protocol specification.