3GPP TSG-RAN WG2 Meeting #103bis	R2-1814389

Chengdu, China, 08 - 12 October 2018	R2-18xxxxx

Agenda item:	10.4.4.2

Source:	Nokia, Nokia Shanghai Bell

Title:	Clarification to TS 36.306 on MIMO capabilities in EN-DC

WID/SID:	NR_newRAT-Core - Release 15

Document for:	Discussion and Decision

# 1	Introduction

In this contribution an issue with the usage of the DL MIMO capabilities for LTE part of the EN-DC capabilities is discussed.

# 2	Background

In the latest version of TS 36.306, 3 versions of the field supportedMIMO-CapabilityDL exist:

#### 4.3.4.7	supportedMIMO-CapabilityDL-r10

This field defines the maximum number of spatial multiplexing layers in the downlink direction for a certain band and bandwidth class in a supportedBandCombination supported by the UE. For bandwidth classes that include multiple component carriers (i.e. bandwidth classes B, C, D and so on), the field defines the maximum number of spatial multiplexing layers supported by the UE on all component carriers in the corresponding bandwidth class.

The support for more layers in supportedMIMO-CapabilityDL than given by the "maximum number of supported layers for spatial multiplexing in DL" derived from the ue-Category (without suffix) in the UE-EUTRA-Capability IE is only applicable to transmission mode 9 and transmission mode 10.

#### 4.3.4.40	supportedMIMO-CapabilityDL-r12

This field defines the maximum number of spatial multiplexing layers in the downlink direction supported by the UE on a single component carrier for bandwidth classes that include multiple component carriers (i.e. bandwidth classes B, C, D and so on).

The support for more layers in supportedMIMO-CapabilityDL-12 than given by the "maximum number of supported layers for spatial multiplexing in DL" derived from the ue-Category or ue-CategoryDL in the UE-EUTRA-Capability IE is only applicable to transmission mode 9 and transmission mode 10.

#### 4.3.5.30	supportedMIMO-CapabilityDL-r15

If present, this field defines the number of downlink MIMO layers the UE supports when the UE is configured with sTTI. Only two layers or four layers for MIMO support using this field are applicable with sTTI.

However, in TS 36.331, the usage is as follows: In 36.331, the following IE marked in CYAN is referring to the TM9 MIMO capability for component carrier in MR-DC Band combination: The IE is set to supportedMIMO-CapabilityDL-r10 in the following ASN.1.

FeatureSetDL-PerCC-r15::= SEQUENCE {

fourLayerTM3-TM4-r15 ENUMERATED {supported} OPTIONAL,

supportedMIMO-CapabilityDL-r15 MIMO-CapabilityDL-r10 OPTIONAL,

supportedCSI-Proc-r15 ENUMERATED {n1, n3, n4} OPTIONAL

}

Proposal 1: Define supportedMIMO-CapabilityDL-r15 for EN-DC based on MIMO-CapabilityDL-r10.

Proposal 2: Consider redefining the existing supportedMIMO-CapabilityDL-r15 IE into supportedMIMO-CapabilityDL-sTTI-r15 to specifically target it for sTTI linked MIMO requirement.

# 3	Conclusion

In this contribution we have discussed the potential confusion created by the TS 36.306 description for supportedMIMO-CapabilityDL-r15 IE. Based on the above discussion, we propose:

Proposal 1: Define supportedMIMO-CapabilityDL-r15 for EN-DC based on MIMO-CapabilityDL-r10.

Proposal 2: Consider redefining the existing supportedMIMO-CapabilityDL-r15 IE into supportedMIMO-CapabilityDL-sTTI-r15 to specifically target it for sTTI linked MIMO requirement.

# References

- 3GPP TS 21.801 Drafting Rules

- Rx-18xxxxx Title - Company