3GPP TSG-RAN WG2#103bis	Tdoc R2-1814402

Chengdu, P.R. China, 8th – 12th October 2018

Agenda Item:	10.4.1.6.1

Source:	Ericsson

Title:	Considerations on value tags for PWS related SIBs

Document for:	Discussion, Decision

# 1	Introduction

It has been agreed that there are no SIB specific value tags for the PWS related SIBs (SIB6, SIB7 and SIB8) since they cannot be used for these SIBs as for the other NR SIBs. At the RAN2#103 meeting a proposal to use value tags for the PWS SIBs in order for the network to indicate what PWS warning messages that are being transmitted (in [1]) was however discussed. There were no such agreements but the discussion was postponed to the next meeting.

This contribution discusses some different aspects on the proposal to use value tags for PWS related SIBs.

# 2	Discussion

At the RAN2 NR AH#1807 meeting it was agreed that the PWS SIBs shall not have any value tags. The PWS SIBs differ from other SIBs in that they may be transmitted as segmented messages (SIB7 and SIB8), meaning that the UE needs to receive all the segments before being able to successfully decode the warning message. Different from other SIBs, the PWS SIBs themselves also contain information about what specific warning message they contain. The SIB specific value tags that are defined in NR are thus not relevant as such for the PWS SIBs.

The contribution in [1], where an optimization is proposed to introduce value tags for PWS related SIBs, in order to decrease the related number of SIB acquisitions, was discussed at the RAN2#103 meeting. It was there proposed to include information in SIB1 about what warning message (even including segment information) that is provided in the corresponding PWS SIBs (SIB6, SIB7 and SIB8).

It should however be considered that PWS warning message transmissions are typically rare and the potential benefit with introducing additional mechanisms for UE battery saving during PWS warning transmissions would be very limited (if any at all). Since the UE would need to check SIB1 to determine whether it needs to reacquire any PWS SIB in the proposed solution in [1] it is not clear what UE battery saving that would actually be achieved with such mechanism. It should also be considered that the size of SIB1 then would be increased.

PWS warning messages can be addressed for transmission to an area consisting of one or several cells, dependent on the type of warning. An CMAS warning message can even be addressed to an area smaller than one cell, using the geofencing information that has been added in Rel-15. There is thus no relation between the System Information Area and the warning areas towards which a PWS warning is transmitted. This means that using a value tag for information about transmitted warning messages, as proposed in [1], would only be applicable within a cell. A UE could thus not make any use of such stored information in case it makes a cell reselection since it has no value in the new cell.

It is not clear what UE battery saving would be achieved by adding information about transmitted warning messages in SIB1 during PWS warning transmissions.

There is no relation between System Information Areas and areas where warning messages are transmitted. Received value tag(s) for PWS SIBs would thus be of no use if the UE performs cell reselection.

The PWS SIBs contain warning messages that may be crucial for a user to receive. It is therefore important to have a mechanism that ensures that each UE receives all warning messages that are being transmitted as soon as possible. Several different warning messages may need to be transmitted at the same time and new updated warning messages may need to be transmitted after some time to provide further information. Each warning message (for ETWS secondary notifications and CMAS notifications) may then also need to be segmented in order to provide the whole warning message. The SIB specific value tags consist of 5 bits (32 values). If value tags would be introduced in SIB1 to indicate what PWS warning messages that are being transmitted, such value tags would need to cover also the different combinations of warning messages that may be transmitted, to make sure that each UE can determine whether it has already received all the warning messages. With a value tag size of 5 bits, only 5 different PWS messages would then be supported within 3 hours according to the defined value tags for NR, even if assuming that the same value tag would be used for all segments of a warning message.

Since value tags for PWS SIBs would need to cover all different combinations of warning messages, only 5 different warning messages would be supported within 3 hours.

The issue on provision of value tags for SIBs that are provided through dedicated signalling was discussed at the RAN2 NR-AH 1807 meeting. It was then concluded that this is not needed in Rel-15 since the PWS SIBs are the only ones part of Other SI that are provided through dedicated signalling in this release and they do not have any value tags. If value tags would be introduced for PWS SIBs this would however not be true anymore and RAN2 would then need to define how to provide a UE with value tags of SIBs that are transmitted through dedicated signalling already in Rel-15.

With value tags for PWS SIBs RAN2 would need to define how the UE is provided with the value tags of SIBs that are received through dedicated signalling already in Rel-15.

Given that the benefit on UE power consumption through adding value tags in SIB1 for PWS SIBs is questionable and that there are several uncertain aspects regarding the concept, it is proposed that RAN2 confirms that no value tags are used for the PWS SIBs.

RAN2 to confirm that no value tags are used for PWS SIBs.

# Conclusion

In the previous sections we made the following observations:

Observation 1	It is not clear what UE battery saving would be achieved by adding information about transmitted warning messages in SIB1 during PWS warning transmissions.

Observation 2	There is no relation between System Information Areas and areas where warning messages are transmitted. Received value tag(s) for PWS SIBs would thus be of no use if the UE performs cell reselection.

Observation 3	Since value tags for PWS SIBs would need to cover all different combinations of warning messages, only 5 different warning messages would be supported within 3 hours.

Observation 4	With value tags for PWS SIBs RAN2 would need to define how the UE is provided with the value tags of SIBs that are received through dedicated signalling already in Rel-15.

Based on the discussion in the previous sections we propose the following:

Proposal 1	RAN2 to confirm that no value tags are used for PWS SIBs.

# References

R2-1811492, Value tags for PWS related SIBs, ZTE Corporation, Sanechips, RAN2#103, Gothenburg, Sweden, 20th-24th August 2018