3GPP TSG-RAN WG2 Meeting #103bis	R2-1814610

Chengdu, China, 8th - 12th October 2018

Agenda Item:	9.14.2

Source: 	MediaTek Inc.

Title:  	NCC and Key Handling for UP-EDT

Document for:	Discussion and decision

Introduction

In previous meeting, we had some discussion on the nextHopChainingCount for user plane early data transmission (UP-EDT). The following conclusions were captured in Chairman’s note [1].

- Regarding the following proposal “Consider renaming nextHopChainingCount to nextHopChainingCountNext in RRCConnectionRelease message.” => RAN2 does not intend to rename nextHopChainingCount unless need is identified. [CB#402] To progress the discussion regarding the need to store both the “old” and “new” NCC [Ericsson] => The discussion is postponed to be discussed in the next meeting.

In addition, we agreed to introduce a new section in TS 36.331 to describe the UE behaviour upon abortion of UP-EDT, covering the cases of (1) fallback indication from lower layer, (2) receiving RRCConnectionReject in response to an RRCConnectionResumeRequest for EDT, (3) expiry of T300, and (4) cell reselection after EDT has been initiated.

#### 5.3.3.9a Abortion of UP-EDT The UE shall: 1> delete the KeNB, KRRCint, KRRCenc and KUPenc keys derived in accordance with 5.3.3.3a; 1> re-establish RLC entities for all SRBs and DRBs; 1> suspend all SRB(s) and DRB(s) except SRB0; 1> configure lower layers to suspend integrity protection and ciphering;

In this paper, we further discuss the RAN2 details on remaining issues.

Discussion

NCC and key handling for successful UP-EDT in UL

To support UP-EDT, NCC is carried by RRC Connection Release (with suspend), and UE derives new keys based on this NCC. Then for MSG3 security, “old” integrity key (used in the last RRC connection as in legacy) is used to generate shortResumeMAC-I, while “new” key (based on NCC received in RRCConnectionRelease message with suspend) is used to cipher data.

After successful UP-EDT in the UL (i.e., data in MSG3), UE may receive different response from eNB. The connection may simply be suspended, or be asked to fallback to connection resumption or establishment. We now discuss the NCC and Keys stored in UE for these cases as follows.

UE receives RRCConnectionRelease message (with suspend)

This means successful UP-EDT in UL and network simply wants to suspend the connection afterwards. The UE temporally stores both “old” and “new” security context (NCC, NH, and keys) after key derivation based on the NCC receive in previous RRCConnectionRelease message, and the context will be replaced with “new” ones upon reception of RRCConnectionRelease message after EDT. The procedure is shown in Figure 1.

Figure 1.	RRCConnectionRelease after UP-EDT

UE receives RRCConnectionResume message

This means successful UP-EDT in UL and network wants to resume the connection afterwards, and is considered as fallback to RRC connection resumption. The NCC in RRCConnectionResume message (in response to RRCConnectionResumeRequest for EDT) is to be ignored. Similar to the previous case, the UE temporally stores both “old” and “new” security context after key derivation based on the NCC receive in previous RRCConnectionResume message, and the context will be replaced with “new” ones upon reception of RRCConnectionResume message after EDT. The procedure is shown in Figure 2.

Figure 2.	RRCConnectionResume after UP-EDT

UE receives RRCConnectionSetup

If the RRCConnectionSetup is received in response to an RRCConnectionResumeRequest (whether for EDT or not), UE discard the (if stored) the NCC value. If the RRCConnectionResumeRequest is for EDT, this is considered as fallback to RRC connection establishment. The procedure is illustrated in Figure 3.

Figure 3.	RRCConnectionSetup after UP-EDT

We observe that for successful UP-EDT, the UE needs to store two sets of NCCs and corresponding keys during the procedure. At some stage the “new” NCC will be used to replace NCC, and this can be handled properly by the UE. It is unnecessary to mention explicitly in the specification the need to store both the “old” and “new” NCC.

Observation 1:	For successful UP-EDT, the UE needs to store two sets of NCCs and corresponding keys during the procedure. At some stage the “new” NCC will be used to replace NCC, and this can be handled properly by the UE.

Proposal 1:	It is unnecessary to mention explicitly in the specification the need to store both the “old” and “new” NCC.

NCC and key handling when UP-EDT is aborted

As mentioned before, UP-EDT may be aborted in several cases. We now discuss related NCC and key handling issues.

- Receiving fallback indication from lower layer: According to Section 5.3.3.3c of TS 36.331, UE shall perform the actions upon abortion of UP-EDT as specified in 5.3.3.9a, and then initiate transmission of the RRCConnectionResumeRequest message in accordance with 5.3.3.3a. Since the UE is to send RRCConnectionResumeRequest message without data, the “new” NCC is not to be used.

- Receiving RRCConnectionReject: According to Section 5.3.3.3c of TS 36.331, UE shall perform the actions upon abortion of UP-EDT as specified in 5.3.3.9a. UE is not prohibited from sending another EDT MSG3 with data cipher using new key derived from “new” NCC.

- Expiry of T300: Similar to above, according to Section 5.3.3.6 of TS 36.331, UE is only asked to perform the actions upon abortion of UP-EDT.

- Cell reselection after EDT has been initiated: UE performs the actions upon abortion of UP-EDT.

The case of Receiving RRCConnectionReject is shown in Figure 4.

Figure 4.	RRCConnectionReject after UP-EDT attempt

Among the four cases, except for receiving fallback indication from lower layer, it is possible for UE to initiate another EDT attempt using the ‘new’ NCC received in last RRCConnectionRelease message. However, if the EDT resume request is rejected, or UE receives no response when T300 expires, it may not be a good idea to try UP-EDT again. The UE would better perform a normal resumption request or connection setup request. If cell reselection happens, the reselected eNB may not be able to verify the keys and the EDT may eventually fail. Therefore, we suggest that the UE also discard the ‘new’ NCC received RRCConnectionRelease message upon abortion of UP-EDT.

Proposal 2:	UE discards the “new” NCC received in RRCConnectionRelease message upon abortion of UP-EDT.

The corresponding text proposal is shown below.

#### 5.3.3.9a Abortion of UP-EDT The UE shall: 1> discard the nextHopChainingCount received in RRCConnectionRelease message; 1> delete the KeNB, KRRCint, KRRCenc and KUPenc keys derived in accordance with 5.3.3.3a; 1> re-establish RLC entities for all SRBs and DRBs; 1> suspend all SRB(s) and DRB(s) except SRB0; 1> configure lower layers to suspend integrity protection and ciphering;

Conclusion

We have the following observations:

Observation 1:	For successful UP-EDT, the UE needs to store two sets of NCCs and corresponding keys during the procedure. At some stage the “new” NCC will be used to replace NCC, and this can be handled properly by the UE.

It is proposed to discuss and decide on the following proposals:

Proposal 1:	It is unnecessary to mention explicitly in the specification the need to store both the “old” and “new” NCC.

Proposal 2:	UE discards the “new” NCC received in RRCConnectionRelease message upon abortion of UP-EDT.

Reference

- RAN2 #103 Chairman’s note

- R2-1813091, Introduction of EDT for eMTC and NB-IoT enhancements, TS36.331 CR