3GPP TSG-RAN2 Meeting #103bis 	R2-1814666

Chengdu, China, 2018-10-08 to 2018-10-12

| CR-Form-v11.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.331 | CR | CRNum | rev | - | Current version: | 15.3.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | X | Radio Access Network | X | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Secondary node resume in NE-DC and NR-DC |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Ericsson |  |  |  |  |  |  |  |  |  |
| Source to TSG: | n.a. (draft CR to be merged into rapporteur's CR when agreed) |  |  |  |  |  |  |  |  |  |
| Work item code: | NR_newRAT-Core |  |  |  |  |  |  | Date: |  | 2018-09-27 |
| Category: | B |  |  |  |  |  |  | Release: |  | Rel-15 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) Rel-12 (Release 12) Rel-13 (Release 13) Rel-14 (Release 14) Rel-15 (Release 15) Rel-16 (Release 16) |  |
| Reason for change: |  | Currently, secondary node resume is supported in LTE but not in NR. To support this, the sk-Counter, the secondary RadioBearerConfig and the secondaryCellGroup need to be included in the RRCResume message. Since NE-DC will require the LTE lower layer for the SN, while NR-DC will require NR lower layers, the RRCResume message need to be able to include either one of them (i.e. secondaryCellGroupConfig or eutra-SecondaryCellGroupConfig). In case of NR-DC, the RRC Resume emssage should also inlcude the measurement configurations for the SCG (e.g. measConfig2), whereas for NE-DC the SCG ocnfigurations are included in the RRCConnectionReconfiguration message included in the eutra-SecondaryCellGroupConfig. Since it should be possible to configure, re-configure (delta) or release the SCG configurations (in secondaryCellGroup or eutra-SecondaryCellGroup) a mrdc-Release flag is introduced. If the flag is included, any stored SCG configurations (including measConfig and SRB3) are released. Then new SCG configurations can be applied. If the release flag is not included, delta configurations are applicable. Alternatively, the fullConfig flag can be set which releases both the SCG and RB config. |  |  |  |  |  |  |  |  |
| Summary of change: |  | RRCResume message and procedures: Added sk-counter, secondaryCellGroup, mrdc-Release, eutra-SecondaryCellGroup, measConfig2, and radioBearerConfig2. The procedures to release the MR-DC configurations are captured in CR R2-1814668. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Configuration of SN will not be possible during RRC Resume |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 5.3.13.4 Reception of the RRCResume by the UE 6.2.2 - RRCResume |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  | TS/TR ... CR ... |  |  |  |
| affected: |  |  | X | Test specifications |  |  | TS/TR ... CR ... |  |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  | TS/TR ... CR ... |  |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |

START OF FIRST CHANGE

#### 5.3.13.4	Reception of the RRCResume by the UE

The UE shall:

1>	stop timer T319;

1>	if the RRCResume includes the fullConfig:

2>	perform the full configuration procedure as specified in 5.3.5.11;

1>	else:

2>	restore the PDCP state and reset COUNT value for SRB2 and all DRBs;

2>	restore the cellGroupConfig from the stored UE AS context;

2>	indicate to lower layers that stored UE AS context is used;

1>	discard the fullI-RNTI, shortI-RNTI and the stored UE AS context, except ran-NotificationAreaInfo;

1>	if the RRCResume includes the masterCellGroup:

2>	perform the cell group configuration for the received masterCellGroup according to 5.3.5.5;

1>	if the RRCResume includes the radioBearerConfig:

2>	perform the radio bearer configuration according to 5.3.5.6;

1>if the RRCResume message includes the sk-Counter:

2>	perform security key update procedure as specified in 5.3.5.7;

1>	if the RRCResume message includes the radioBearerConfig2:

2>	perform the radio bearer configuration according to 5.3.5.6;

1>	if the RRCResume message includes the mrdc-Release:

2>	release the MR-DC configurations as specified in section 5.3.5.10a;

Editor’s note: The procedures for MR-DC release are FFS.

1>	if the RRCResume message includes the secondaryCellGroup:

2>	perform the cell group configuration for the SCG according to 5.3.5.5;

1>	if the RRCResume message includes the measConfig2:

2>	perform the measurement configuration procedure as specified in 5.5.2;

1>	if the RRCResume message includes the eutra-secondaryCellGroup:

2>	perform the cell group configuration for the SCG according to TS 36.331 [10], section 5.3.5.4;

1>	resume SRB2 and all DRBs;

1>	if stored, discard the cell reselection priority information provided by the cellReselectionPriorities or inherited from another RAT;

1> stop timer T320, if running;

1>	if the RRCResume message includes the measConfig:

2>	perform the measurement configuration procedure as specified in 5.5.2;

1>	resume measurements if suspended;

Editor’s Note: FFS Whether there is a need to define UE actions related to access control timers (equivalent to T302, T303, T305, T306, T308 in LTE). For example, informing upper layers if a given timer is not running.

1>	enter RRC_CONNECTED;

1>	indicate to upper layers that the suspended RRC connection has been resumed;

1>	stop the cell re-selection procedure;

1>	consider the current cell to be the PCell;

1>	set the content of the of RRCResumeComplete message as follows:

2>  if the upper layer provides NAS PDU, set the dedicatedNAS-Message to include the information received from upper layers;

2> if the upper layer provides a PLMN, set the selectedPLMN-Identity to PLMN selected by upper layers (TS 24.501 [23]) from the PLMN(s) included in the plmn-IdentityList in SIB1;

2>	if the masterCellGroup contains the reportUplinkTxDirectCurrent:

3> include the uplinkTxDirectCurrentList;

1>	submit the RRCResumeComplete message to lower layers for transmission;

1>	the procedure ends.

END OF FIRST CHANGE

START OF SECOND CHANGE

### 6.2.2	Message definitions

#### –	RRCResume

The RRCResume message is used to resume the suspended RRC connection.

Signalling radio bearer: SRB1

RLC-SAP: AM

Logical channel: DCCH

Direction: Network to UE

RRCResume message

-- ASN1START

-- TAG-RRCRESUME-START

RRCResume ::=                       SEQUENCE {

rrc-TransactionIdentifier           RRC-TransactionIdentifier,

criticalExtensions                  CHOICE {

rrcResume                           RRCResume-IEs,

criticalExtensionsFuture            SEQUENCE {}

}

}

RRCResume-IEs ::=                   SEQUENCE {

radioBearerConfig                   RadioBearerConfig                                                       OPTIONAL, -- Need M

masterCellGroup                     OCTET STRING (CONTAINING CellGroupConfig)                               OPTIONAL, -- Need M

measConfig                          MeasConfig                                                              OPTIONAL, -- Need M

fullConfig                          ENUMERATED {true}                                                       OPTIONAL, -- Need N

lateNonCriticalExtension            OCTET STRING                                                            OPTIONAL,

nonCriticalExtension                RRCResume-v15xx-IEs                                                              OPTIONAL

}

RRCResume-v15xx-IEs ::=             SEQUENCE {

radioBearerConfig2-r15x             OCTET STRING (CONTAINING RadioBearerConfig)                                                       OPTIONAL, -- Need M

sk-Counter-r15x                     INTEGER (0..65535)                                               OPTIONAL, -- Need N

mrdc-Release-r15x                   ENUMERATED {true}                                                OPTIONAL, -- Need N

secondaryCellGroup-r15x             OCTET STRING (CONTAINING CellGroupConfig)                               OPTIONAL, -- Need M

measConfig2-r15x                    OCTET STRING (CONTAINING MeasConfig)                               OPTIONAL, -- Need M

eutra-SecondaryCellGroup-r15x       OCTET STRING (CONTAINING RRCConnectionReconfiguration)                               OPTIONAL, -- Need M

nonCriticalExtension                SEQUENCE{}                                                              OPTIONAL

}

-- TAG-RRCRESUME-STOP

-- ASN1STOP

RRCResume-IEs field descriptions

masterCellGroup Configuration of the master cell group (NR Standalone or MR-DC):

measConfig Configuration of measurements.

measConfig2 Configuration of a secondary set of measurements (NR-DC).

mrdc-Release A one-shot field indicating whether the UE releases the stored SCG related configuration (e.g. secondaryCellGroup, eutra-SecondaryCellGroup, SRB3 and SCG measConfig). New SCG configurations can then be configured.

radioBearerConfig, radioBearerConfig2 Configuration of Radio Bearers (DRBs, SRBs) including SDAP/PDCP.

secondaryCellGroup Configuration of NR secondary cell group.

eutra-SecondaryCellGroup Includes the LTE RRCConnectionReconfiguration message as specified in TS 36.331 [10]. In this version of the specification, the LTE RRC message only includes fields radioResourceConfigDedicated (with no PDCP configuration present) and/or mobilityControlInfo and/ or measConfig. If eutra-secondaryCellGroup is configured, the network always includes this field upon MN handover to initiate an LTE SCG reconfiguration with sync and key change.

sk-Counter A one-shot counter used upon refresh of S-KgNB or S-KeNB based on the newly derived KgNB during RRC Resume. The field is mandatory present in case of resumption or configuration of MR-DC during RRC resume. Otherwise, the field is absent.

END OF SECOND CHANGE