3GPP TSG-RAN2 Meeting #103bis 	R2-1814668

Chengdu, China, 2018-10-08 to 2018-10-12

| CR-Form-v11.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.331 | CR | CRNum | rev | - | Current version: | 15.3.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | X | Radio Access Network | X | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Release of secondary cell group in NR-DC and NE-DC (38.331) |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Ericsson |  |  |  |  |  |  |  |  |  |
| Source to TSG: | n.a. (draft CR to be merged into rapporteur's CR when agreed) |  |  |  |  |  |  |  |  |  |
| Work item code: | NR_newRAT-Core |  |  |  |  |  |  | Date: |  | 2018-09-27 |
| Category: | B |  |  |  |  |  |  | Release: |  | Rel-15 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) Rel-12 (Release 12) Rel-13 (Release 13) Rel-14 (Release 14) Rel-15 (Release 15) Rel-16 (Release 16) |  |
| Reason for change: |  | When the network triggers the UE to release the SCG in MR-DC, the SRB3, SCG measConfig and SCG configurations shall be released. If the UE is in NE-DC, the SCG will be E-UTRA and the release will be configured in 36.331. |  |  |  |  |  |  |  |  |
| Summary of change: |  | - Secondary cell group release extended to be able to trigger from NR - MR-DC release, Added procedure to release the MR-DC configurations RRCResume message and procedures: Description of how to trigger the MR-DC release are described in CR R2-1814565 for RRC Reconfiguration and CR R2-1814666 for RRC Resume. The procedures in 36.331 to release the SCG in NE-DC changes are described in CR R2-1814667. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Release of SCG will not be supported in MR-DC |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 5.3.5.4 Secondary cell group release 5.3.5.10a MR-DC release |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  | TS/TR ... CR ... |  |  |  |
| affected: |  |  | X | Test specifications |  |  | TS/TR ... CR ... |  |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  | TS/TR ... CR ... |  |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |

START OF FIRST CHANGE

#### 5.3.5.4	Secondary cell group release

The UE shall:

1>	as a result of SCG release triggered by E-UTRA or NR:

2> reset SCG MAC, if configured;

2>	for each RLC bearer that is part of the SCG configuration:

3>	perform RLC bearer release procedure as specified in 5.3.5.5.3;

2>	release the SCG configuration;

2>	stop timer T310 for the corresponding SpCell, if running;

2>	stop timer T304 for the corresponding SpCell, if running.

NOTE:	Release of cell group means only release of the lower layer configuration of the cell group but the RadioBearerConfig may not be released.

END OF FIRST CHANGE

START OF SECOND CHANGE

#### 5.3.5.10	EN-DC release

The UE shall:

1>	as a result of EN-DC release triggered by E-UTRA:

2> release SRB3 (configured according to radioBearerConfig), if present;

2>	release measConfig;

2> release the SCG configuration as specified in section 5.3.5.4.

#### 5.3.5.10a	MR-DC release

The UE shall:

1> release SRB3 (configured according to radioBearerConfig), if present;

1>	if the UE is configured with NR SCG:

2> release the SCG configuration as specified in section 5.3.5.4;

2> release measConfig2;

1>	else (the UE is configured with E-UTRA SCG):

2> release the SCG configuration as specified in TS 36.331 [10], section 5.3.10.x to release the E-UTRA SCG;

Editor’s note: It is FFS how the E-UTRA SCG is released in 36.331.

END OF SECOND CHANGE